The NGS Data Sheet

##### See file  dsdata.txt for more information about the datasheet.

DATABASE = Sybase ,PROGRAM = datasheet, VERSION = 7.42

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0090 ***********************************************************************

JY0090  DESIGNATION -  727

JY0090  PID         -  JY0090

JY0090  STATE/COUNTY-  OH/HOCKING

JY0090  USGS QUAD   -  LOGAN (1992)

JY0090

JY0090                         *CURRENT SURVEY CONTROL

JY0090  ___________________________________________________________________

JY0090* NAD 83(1986)-  39 32 39.     (N)    277 34 08      (E)     SCALED

JY0090* NAVD 88     -       221.609  (meters)     727.06   (feet)  ADJUSTED

JY0090  ___________________________________________________________________

JY0090  GEOID HEIGHT-         -34.13  (meters)                     GEOID03

JY0090  DYNAMIC HT  -         221.478 (meters)     726.63  (feet)  COMP

JY0090  MODELED GRAV-     980,030.5   (mgal)                       NAVD 88

JY0090

JY0090  VERT ORDER  -  SECOND    CLASS 0

JY0090

JY0090.The horizontal coordinates were scaled from a topographic map and have

JY0090.an estimated accuracy of +/- 6 seconds.

JY0090

JY0090.The orthometric height was determined by differential leveling

JY0090.and adjusted by the National Geodetic Survey in June 1991..

JY0090

JY0090.The geoid height was determined by GEOID03.

JY0090

JY0090.The dynamic height is computed by dividing the NAVD 88

JY0090.geopotential number by the normal gravity value computed on the

JY0090.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0090.degrees latitude (g = 980.6199 gals.).

JY0090

JY0090.The modeled gravity was interpolated from observed gravity values.

JY0090

JY0090;                    North         East    Units  Estimated Accuracy

JY0090;SPC OH S     -   171,430.      605,920.      MT  (+/- 180 meters Scaled)

JY0090

JY0090                          SUPERSEDED SURVEY CONTROL

JY0090

JY0090  NGVD 29 (??/??/92)  221.732  (m)          727.47   (f) ADJ UNCH    2 0

JY0090

JY0090.Superseded values are not recommended for survey control.

JY0090.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0090.See file dsdata.txt to determine how the superseded data were derived.

JY0090

JY0090_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD770781(NAD 83)

JY0090_MARKER: DD = SURVEY DISK

JY0090_SETTING: 30 = SET IN A LIGHT STRUCTURE

JY0090_SP_SET: CULVERT

JY0090_STAMPING: 727.46

JY0090_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY0090

JY0090  HISTORY     - Date     Condition        Report By

JY0090  HISTORY     - UNK      MONUMENTED       USGS

JY0090  HISTORY     - 1936     GOOD             NGS

JY0090  HISTORY     - 19881220 MARK NOT FOUND   USPSQD

JY0090  HISTORY     - 20051227 GOOD             GEOCAC

JY0090

JY0090                          STATION DESCRIPTION

JY0090

JY0090'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0090'1.4 MI NW FROM LOGAN.

JY0090'1.4 MILES NORTHWEST ALONG OLD STATE HIGHWAY 31 FROM THE POST

JY0090'OFFICE AT LOGAN, HOCKING COUNTY, 112 YARDS WEST OF THE NORTHWEST

JY0090'CORNER OF A SHELL SERVICE STATION, AT A SMALL CONCRETE CULVERT,

JY0090'IN THE TOP OF THE SOUTH HEADWALL, ONE FOOT EAST OF THE WEST END,

JY0090'39 FEET SOUTH OF THE CENTERLINE OF THE HIGHWAY, 15.5 FEET SOUTH

JY0090'OF THE GUARDRAIL FENCE, AND ABOUT 6 FEET LOWER THAN THE HIGHWAY.

JY0090'A UNITED STATES GEOLOGICAL SURVEY STANDARD DISK, STAMPED 727.46.

JY0090

JY0090                          STATION RECOVERY (1988)

JY0090

JY0090'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0090'MARK NOT FOUND.

JY0090

JY0090                          STATION RECOVERY (2005)

JY0090

JY0090'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0090'ABOUT 0.1 MILE WEST OF THE INTERSECTION OF SR 664 AND CHIEFTAIN DRIVE,

JY0090'AT A SMALL CONCRETE CULVERT, IN THE TOP OF THE SOUTH HEADWALL, ONE

JY0090'FOOT EAST OF THE WEST END, 39 FEET SOUTH OF THE CENTERLINE OF

JY0090'CHIEFTAIN DRIVE, AND ABOUT 6 FEET LOWER THAN THE HIGHWAY. A UNITED

JY0090'STATES GEOLOGICAL SURVEY STANDARD DISK, STAMPED 727.46.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0091 ***********************************************************************

JY0091  DESIGNATION -  734

JY0091  PID         -  JY0091

JY0091  STATE/COUNTY-  OH/HOCKING

JY0091  USGS QUAD   -  LOGAN (1992)

JY0091

JY0091                         *CURRENT SURVEY CONTROL

JY0091  ___________________________________________________________________

JY0091* NAD 83(1986)-  39 32 55.     (N)    277 33 36      (E)     SCALED

JY0091* NAVD 88     -       223.706  (meters)     733.94   (feet)  ADJUSTED

JY0091  ___________________________________________________________________

JY0091  GEOID HEIGHT-         -34.13  (meters)                     GEOID03

JY0091  DYNAMIC HT  -         223.574 (meters)     733.51  (feet)  COMP

JY0091  MODELED GRAV-     980,029.7   (mgal)                       NAVD 88

JY0091

JY0091  VERT ORDER  -  SECOND    CLASS 0

JY0091

JY0091.The horizontal coordinates were scaled from a topographic map and have

JY0091.an estimated accuracy of +/- 6 seconds.

JY0091

JY0091.The orthometric height was determined by differential leveling

JY0091.and adjusted by the National Geodetic Survey in June 1991..

JY0091

JY0091.The geoid height was determined by GEOID03.

JY0091

JY0091.The dynamic height is computed by dividing the NAVD 88

JY0091.geopotential number by the normal gravity value computed on the

JY0091.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0091.degrees latitude (g = 980.6199 gals.).

JY0091

JY0091.The modeled gravity was interpolated from observed gravity values.

JY0091

JY0091;                    North         East    Units  Estimated Accuracy

JY0091;SPC OH S     -   171,920.      605,160.      MT  (+/- 180 meters Scaled)

JY0091

JY0091                          SUPERSEDED SURVEY CONTROL

JY0091

JY0091  NGVD 29 (??/??/92)  223.829  (m)          734.35   (f) ADJ UNCH    2 0

JY0091

JY0091.Superseded values are not recommended for survey control.

JY0091.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0091.See file dsdata.txt to determine how the superseded data were derived.

JY0091

JY0091_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD762786(NAD 83)

JY0091_MARKER: DD = SURVEY DISK

JY0091_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0091_SP_SET: BRIDGE

JY0091_STAMPING: 734.18

JY0091_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0091_SATELLITE: THE SITE LOCATION WAS REPORTED AS NOT SUITABLE FOR

JY0091+SATELLITE: SATELLITE OBSERVATIONS - June 17, 2005

JY0091

JY0091  HISTORY     - Date     Condition        Report By

JY0091  HISTORY     - UNK      MONUMENTED       USGS

JY0091  HISTORY     - 1936     GOOD             NGS

JY0091  HISTORY     - 1987     GOOD             OHDT

JY0091  HISTORY     - 19881220 GOOD             USPSQD

JY0091  HISTORY     - 20050617 GOOD             GEOCAC

JY0091

JY0091                          STATION DESCRIPTION

JY0091

JY0091'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0091'2 MI NW FROM LOGAN.

JY0091'2.0 MILES NORTHWEST ALONG OLD STATE HIGHWAY 31 FROM THE COUNTY

JY0091'COURTHOUSE AT LOGAN, HOCKING COUNTY, 300 YARDS WEST OF THE HOCKING

JY0091'VALLEY SILVER FOX CO. PROPERTY, 60 YARDS WEST OF THE RIVERSIDE

JY0091'TOURIST CAP, 30 YARDS NORTH OF THE NORTH BANK OF HOCKING RIVER, AT

JY0091'A BRIDGE OVER HARPERS RUN, IN THE TOP OF THE SOUTH WING WALL, 26

JY0091'FEET SOUTHWEST OF THE CENTERLINE OF THE HIGHWAY, AND LEVEL WITH THE

JY0091'HIGHWAY.  A UNITED STATES GEOLOGICAL SURVEY STANDARD DISK,

JY0091'STAMPED 734.18.

JY0091

JY0091                          STATION RECOVERY (1987)

JY0091

JY0091'RECOVERY NOTE BY OHIO DEPARTMENT OF TRANSPORTATION 1987 (UNK)

JY0091'RECOVERED IN GOOD CONDITION.

JY0091

JY0091                          STATION RECOVERY (1988)

JY0091

JY0091'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0091'RECOVERED IN GOOD CONDITION.

JY0091

JY0091                          STATION RECOVERY (2005)

JY0091

JY0091'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0091'2 MI NW FROM LOGAN. 2.0 MILES NORTHWEST ALONG CR 33A (CHEIFTAIN DR)

JY0091'FROM THE COUNTY COURTHOUSE AT LOGAN, HOCKING COUNTY, 30 YARDS NORTH OF

JY0091'THE NORTH BANK OF HOCKING RIVER, AT A BRIDGE OVER HARPERS RUN, IN THE

JY0091'TOP OF THE SOUTH WING WALL, 26 FEET SOUTHWEST OF THE CENTERLINE OF THE

JY0091'HIGHWAY, AND LEVEL WITH THE HIGHWAY.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0104 ***********************************************************************

JY0104  DESIGNATION -  737

JY0104  PID         -  JY0104

JY0104  STATE/COUNTY-  OH/HOCKING

JY0104  USGS QUAD   -  LOGAN (1992)

JY0104

JY0104                         *CURRENT SURVEY CONTROL

JY0104  ___________________________________________________________________

JY0104* NAD 83(1986)-  39 32 56.     (N)    277 36 18      (E)     SCALED

JY0104* NAVD 88     -       224.522  (meters)     736.62   (feet)  POSTED

JY0104  ___________________________________________________________________

JY0104  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0104  DYNAMIC HT  -         224.390 (meters)     736.19  (feet)  COMP

JY0104  MODELED GRAV-     980,034.2   (mgal)                       NAVD 88

JY0104

JY0104  VERT ORDER  -  * POSTED, Code F , SEE BELOW

JY0104

JY0104.The horizontal coordinates were scaled from a topographic map and have

JY0104.an estimated accuracy of +/- 6 seconds.

JY0104

JY0104.The orthometric height was determined by differential leveling

JY0104.and adjusted by the National Geodetic Survey in 1992..

JY0104.* This is a POSTED BENCH MARK height.  Code F indicates a distribution

JY0104.rate greater than 8.0 mm/km.

JY0104

JY0104.The geoid height was determined by GEOID03.

JY0104

JY0104.The dynamic height is computed by dividing the NAVD 88

JY0104.geopotential number by the normal gravity value computed on the

JY0104.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0104.degrees latitude (g = 980.6199 gals.).

JY0104

JY0104.The modeled gravity was interpolated from observed gravity values.

JY0104

JY0104;                    North         East    Units  Estimated Accuracy

JY0104;SPC OH S     -   171,950.      609,030.      MT  (+/- 180 meters Scaled)

JY0104

JY0104                          SUPERSEDED SURVEY CONTROL

JY0104

JY0104  NGVD 29 (??/??/92)  224.677  (m)          737.13   (f) ADJ UNCH    2 0

JY0104

JY0104.Superseded values are not recommended for survey control.

JY0104.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0104.See file dsdata.txt to determine how the superseded data were derived.

JY0104

JY0104_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD801786(NAD 83)

JY0104_MARKER: DD = SURVEY DISK

JY0104_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0104_SP_SET: BRIDGE

JY0104_STAMPING: 737

JY0104_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0104

JY0104  HISTORY     - Date     Condition        Report By

JY0104  HISTORY     - UNK      MONUMENTED       USGS

JY0104  HISTORY     - 1934     GOOD             NGS

JY0104  HISTORY     - 19881227 POOR             USPSQD

JY0104  HISTORY     - 20051227 MARK NOT FOUND   GEOCAC

JY0104

JY0104                          STATION DESCRIPTION

JY0104

JY0104'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0104'1.2 MI NE FROM LOGAN.

JY0104'1.2 MILES NORTHEAST ALONG STATE HIGHWAY 75 FROM THE POST OFFICE

JY0104'AT LOGAN, HOCKING COUNTY, 0.2 MILE SOUTH OF THE C.H. COHAGEN

JY0104'SERVICE STATION, AT THE EAST SIDE OF A CONCRETE BRIDGE, IN THE

JY0104'TOP OF THE SOUTH WING WALL, 21 FEET EAST OF THE CENTERLINE OF THE

JY0104'HIGHWAY, AND LEVEL WITH THE ROADWAY.  A UNITED STATES GEOLOGICAL

JY0104'SURVEY STANDARD DISK, STAMPED 737.

JY0104

JY0104                          STATION RECOVERY (1988)

JY0104

JY0104'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0104'MARK RECOVERED IN POOR CONDITION.

JY0104

JY0104                          STATION RECOVERY (2005)

JY0104

JY0104'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0104'THE BRIDGE HAS BEEN REPLACED AND THE DISK DESCRIBED IN 1934 IS

JY0104'PRESUMED DESTROYED.

JY0104'

JY0104'A NGS DISK STAMPED '737 RESET 1988' IS MOUNTED ON THE TOP OF THE SOUTH

JY0104'ABUTMENT EAST OF THE GUARDRAIL AND LEVEL WITH THE PAVEMENT.

JY0104'

JY0104'

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0102 ***********************************************************************

JY0102  DESIGNATION -  741

JY0102  PID         -  JY0102

JY0102  STATE/COUNTY-  OH/HOCKING

JY0102  USGS QUAD   -  LOGAN (1992)

JY0102

JY0102                         *CURRENT SURVEY CONTROL

JY0102  ___________________________________________________________________

JY0102* NAD 83(1986)-  39 32 24.     (N)    277 35 32      (E)     SCALED

JY0102* NAVD 88     -       225.660  (meters)     740.35   (feet)  POSTED

JY0102  ___________________________________________________________________

JY0102  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0102  DYNAMIC HT  -         225.530 (meters)     739.93  (feet)  COMP

JY0102  MODELED GRAV-     980,033.5   (mgal)                       NAVD 88

JY0102

JY0102  VERT ORDER  -  * POSTED, Code F , SEE BELOW

JY0102

JY0102.The horizontal coordinates were scaled from a topographic map and have

JY0102.an estimated accuracy of +/- 6 seconds.

JY0102

JY0102.The orthometric height was determined by differential leveling

JY0102.and adjusted by the National Geodetic Survey in 1992..

JY0102.* This is a POSTED BENCH MARK height.  Code F indicates a distribution

JY0102.rate greater than 8.0 mm/km.

JY0102

JY0102.The geoid height was determined by GEOID03.

JY0102

JY0102.The dynamic height is computed by dividing the NAVD 88

JY0102.geopotential number by the normal gravity value computed on the

JY0102.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0102.degrees latitude (g = 980.6199 gals.).

JY0102

JY0102.The modeled gravity was interpolated from observed gravity values.

JY0102

JY0102;                    North         East    Units  Estimated Accuracy

JY0102;SPC OH S     -   170,970.      607,930.      MT  (+/- 180 meters Scaled)

JY0102

JY0102                          SUPERSEDED SURVEY CONTROL

JY0102

JY0102  NGVD 29 (??/??/92)  225.784  (m)          740.76   (f) ADJ UNCH    2 0

JY0102

JY0102.Superseded values are not recommended for survey control.

JY0102.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0102.See file dsdata.txt to determine how the superseded data were derived.

JY0102

JY0102_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD790776(NAD 83)

JY0102_MARKER: DD = SURVEY DISK

JY0102_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0102_SP_SET: FOUNDATION

JY0102_STAMPING: 740.64

JY0102_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0102_SATELLITE: THE SITE LOCATION WAS REPORTED AS NOT SUITABLE FOR

JY0102+SATELLITE: SATELLITE OBSERVATIONS - December 27, 2005

JY0102

JY0102  HISTORY     - Date     Condition        Report By

JY0102  HISTORY     - UNK      MONUMENTED       USGS

JY0102  HISTORY     - 1934     GOOD             NGS

JY0102  HISTORY     - 20051227 GOOD             GEOCAC

JY0102

JY0102                          STATION DESCRIPTION

JY0102

JY0102'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0102'AT LOGAN.

JY0102'AT LOGAN, HOCKING COUNTY, 87 YARDS SOUTHWEST OF THE CENTER OF THE

JY0102'CROSSING OF STATE HIGHWAY 75 AND 31, 65 YARDS EAST OF THE

JY0102'NORTHEAST CORNER OF THE COUNTY COURTHOUSE, AT THE LOGAN REPUBLICAN

JY0102'PRINTING SHOP, IN THE TOP OF THE NORTH FOUNDATION, 12 FEET EAST

JY0102'OF THE CENTER OF THE NORTH DOOR, 5 FEET WEST OF THE CENTER OF THE

JY0102'DOOR TO A STAIRWAY LEADING TO THE SECOND FLOOR, 42 FEET SOUTH OF

JY0102'THE CENTERLINE OF MAIN STREET, AND LEVEL WITH THE PAVEMENT.  A

JY0102'UNITED STATES GEOLOGICAL SURVEY STANDARD DISK, STAMPED 740.64.

JY0102

JY0102                          STATION RECOVERY (2005)

JY0102

JY0102'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0102'THE BUILDING NOW HOUSES THE HOCKING COUNTY MUNICIPAL COURT AND IS AT

JY0102'51 E MAIN STREET.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0105 ***********************************************************************

JY0105  DESIGNATION -  746

JY0105  PID         -  JY0105

JY0105  STATE/COUNTY-  OH/HOCKING

JY0105  USGS QUAD   -  LOGAN (1992)

JY0105

JY0105                         *CURRENT SURVEY CONTROL

JY0105  ___________________________________________________________________

JY0105* NAD 83(1986)-  39 33 42.     (N)    277 36 43      (E)     SCALED

JY0105* NAVD 88     -       227.228  (meters)     745.50   (feet)  POSTED

JY0105  ___________________________________________________________________

JY0105  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0105  DYNAMIC HT  -         227.090 (meters)     745.04  (feet)  COMP

JY0105  MODELED GRAV-     980,034.7   (mgal)                       NAVD 88

JY0105

JY0105  VERT ORDER  -  * POSTED, Code F , SEE BELOW

JY0105

JY0105.The horizontal coordinates were scaled from a topographic map and have

JY0105.an estimated accuracy of +/- 6 seconds.

JY0105

JY0105.The orthometric height was determined by differential leveling

JY0105.and adjusted by the National Geodetic Survey in 1992..

JY0105.* This is a POSTED BENCH MARK height.  Code F indicates a distribution

JY0105.rate greater than 8.0 mm/km.

JY0105

JY0105.The geoid height was determined by GEOID03.

JY0105

JY0105.The dynamic height is computed by dividing the NAVD 88

JY0105.geopotential number by the normal gravity value computed on the

JY0105.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0105.degrees latitude (g = 980.6199 gals.).

JY0105

JY0105.The modeled gravity was interpolated from observed gravity values.

JY0105

JY0105;                    North         East    Units  Estimated Accuracy

JY0105;SPC OH S     -   173,370.      609,620.      MT  (+/- 180 meters Scaled)

JY0105

JY0105                          SUPERSEDED SURVEY CONTROL

JY0105

JY0105  NGVD 29 (??/??/92)  227.405  (m)          746.08   (f) ADJ UNCH    2 0

JY0105

JY0105.Superseded values are not recommended for survey control.

JY0105.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0105.See file dsdata.txt to determine how the superseded data were derived.

JY0105

JY0105_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD807800(NAD 83)

JY0105_MARKER: DD = SURVEY DISK

JY0105_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0105_SP_SET: ABUTMENT

JY0105_STAMPING: 745.92

JY0105_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0105

JY0105  HISTORY     - Date     Condition        Report By

JY0105  HISTORY     - UNK      MONUMENTED       USGS

JY0105  HISTORY     - 1934     GOOD             NGS

JY0105  HISTORY     - 19881227 MARK NOT FOUND   USPSQD

JY0105  HISTORY     - 20051227 MARK NOT FOUND   GEOCAC

JY0105

JY0105                          STATION DESCRIPTION

JY0105

JY0105'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0105'2.2 MI NE FROM LOGAN.

JY0105'2.2 MILES NORTHEAST ALONG STATE HIGHWAY 75 FROM THE POST OFFICE

JY0105'AT LOGAN, HOCKING COUNTY, AT BRIDGE HO-75+148, IN THE TOP OF THE

JY0105'SOUTHWEST CORNER OF THE ABUTMENT, 6 INCHES EAST OF THE WEST END,

JY0105'27 FEET WEST OF THE CENTERLINE OF THE HIGHWAY, AND ABOUT ONE FOOT

JY0105'LOWER THAN THE ROADWAY.  A UNITED STATES GEOLOGICAL SURVEY STANDARD

JY0105'DISK, STAMPED 745.92.

JY0105

JY0105                          STATION RECOVERY (1988)

JY0105

JY0105'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0105'MARK NOT FOUND.

JY0105

JY0105                          STATION RECOVERY (2005)

JY0105

JY0105'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0105'THE BRIDGE HAS BEEN REPLACED BY A ROUND CONCRETE CULVERT.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0094 ***********************************************************************

JY0094  DESIGNATION -  749

JY0094  PID         -  JY0094

JY0094  STATE/COUNTY-  OH/HOCKING

JY0094  USGS QUAD   -  LOGAN (1992)

JY0094

JY0094                         *CURRENT SURVEY CONTROL

JY0094  ___________________________________________________________________

JY0094* NAD 83(1986)-  39 33 54.     (N)    277 31 21      (E)     SCALED

JY0094* NAVD 88     -       228.051  (meters)     748.20   (feet)  ADJUSTED

JY0094  ___________________________________________________________________

JY0094  GEOID HEIGHT-         -34.15  (meters)                     GEOID03

JY0094  DYNAMIC HT  -         227.916 (meters)     747.75  (feet)  COMP

JY0094  MODELED GRAV-     980,026.6   (mgal)                       NAVD 88

JY0094

JY0094  VERT ORDER  -  SECOND    CLASS 0

JY0094

JY0094.The horizontal coordinates were scaled from a topographic map and have

JY0094.an estimated accuracy of +/- 6 seconds.

JY0094

JY0094.The orthometric height was determined by differential leveling

JY0094.and adjusted by the National Geodetic Survey in June 1991..

JY0094

JY0094.The geoid height was determined by GEOID03.

JY0094

JY0094.The dynamic height is computed by dividing the NAVD 88

JY0094.geopotential number by the normal gravity value computed on the

JY0094.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0094.degrees latitude (g = 980.6199 gals.).

JY0094

JY0094.The modeled gravity was interpolated from observed gravity values.

JY0094

JY0094;                    North         East    Units  Estimated Accuracy

JY0094;SPC OH S     -   173,740.      601,930.      MT  (+/- 180 meters Scaled)

JY0094

JY0094                          SUPERSEDED SURVEY CONTROL

JY0094

JY0094  NGVD 29 (??/??/92)  228.175  (m)          748.60   (f) ADJ UNCH    2 0

JY0094

JY0094.Superseded values are not recommended for survey control.

JY0094.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0094.See file dsdata.txt to determine how the superseded data were derived.

JY0094

JY0094_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD730805(NAD 83)

JY0094_MARKER: DD = SURVEY DISK

JY0094_SETTING: 30 = SET IN A LIGHT STRUCTURE

JY0094_SP_SET: SIDEWALK

JY0094_STAMPING: 749

JY0094_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY0094

JY0094  HISTORY     - Date     Condition        Report By

JY0094  HISTORY     - UNK      MONUMENTED       USGS

JY0094  HISTORY     - 1936     GOOD             NGS

JY0094  HISTORY     - 20050617 POOR             GEOCAC

JY0094

JY0094                          STATION DESCRIPTION

JY0094

JY0094'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0094'AT ENTERPRISE.

JY0094'AT ENTERPRISE, HOCKING COUNTY, ON STATE HIGHWAY 180, 34 YARDS

JY0094'NORTH OF THE NORTH CORNER OF THE CHESAPEAKE AND OHIO RAILWAY

JY0094'STATION, 8 FEET SOUTH OF THE SOUTH CORNER OF THE POST OFFICE AND

JY0094'STORE, 22 YARDS NORTH OF THE CENTERLINE OF THE TRACK, 15 FEET EAST

JY0094'OF THE CENTERLINE OF THE HIGHWAY, 2 FEET NORTH OF A POLE, AND IN

JY0094'THE TOP OF THE SIDEWALK.  A UNITED STATES GEOLOGICAL SURVEY

JY0094'STANDARD DISK, STAMPED 749.

JY0094

JY0094                          STATION RECOVERY (2005)

JY0094

JY0094'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0094'RAILWAY STATION, POST OFFICE/STORE AND SIDEWALK HAVE BEEN REMOVED.

JY0094'

JY0094'AT ENTERPRISE, 22 YARDS NORTH OF THE CENTERLINE OF THE INDIANA AND

JY0094'OHIO RAILWAY TRACK, 15 FEET EAST OF THE CENTERLINE OF ENTERPRISE-ILES

JY0094'RD, AND IN THE TOP OF A 3 BY 4 FOOT CONCRETE SIDEWALK SLAB. THE

JY0094'STAMPING IS ILLEGIBLE.

JY0094'

JY0094'HANDHELD GPS COORDINATES N39 33.908 W82 28.644

JY0094'

JY0094'

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1907 ***********************************************************************

JY1907  DESIGNATION -  753 RESET

JY1907  PID         -  JY1907

JY1907  STATE/COUNTY-  OH/HOCKING

JY1907  USGS QUAD   -  GORE (1992)

JY1907

JY1907                         *CURRENT SURVEY CONTROL

JY1907  ___________________________________________________________________

JY1907* NAD 83(1986)-  39 35 32.     (N)    277 39 37      (E)     SCALED

JY1907* NAVD 88     -       229.20   (+/-2cm)     752.0    (feet)  VERTCON

JY1907  ___________________________________________________________________

JY1907  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY1907

JY1907  VERT ORDER  -  THIRD (See Below)

JY1907

JY1907.The horizontal coordinates were scaled from a topographic map and have

JY1907.an estimated accuracy of +/- 6 seconds.

JY1907

JY1907.The NAVD 88 height was computed by applying the VERTCON shift value to

JY1907.the NGVD 29 height (displayed under SUPERSEDED SURVEY CONTROL.)

JY1907.The vertical order pertains to the NGVD 29 superseded value.

JY1907

JY1907.The geoid height was determined by GEOID03.

JY1907

JY1907;                    North         East    Units  Estimated Accuracy

JY1907;SPC OH S     -   176,770.      613,770.      MT  (+/- 180 meters Scaled)

JY1907

JY1907                          SUPERSEDED SURVEY CONTROL

JY1907

JY1907  NGVD 29 (10/13/05)  229.44   (m)          752.8    (f) RESET       3

JY1907

JY1907.Superseded values are not recommended for survey control.

JY1907.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1907.See file dsdata.txt to determine how the superseded data were derived.

JY1907

JY1907_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD849833(NAD 83)

JY1907_MARKER: DV = VERTICAL CONTROL DISK

JY1907_SETTING: 37 = SET IN A MASSIVE RETAINING WALL

JY1907_SP_SET: BRIDGE WINGWALL

JY1907_STAMPING: 753 RESET 1992

JY1907_MARK LOGO: NGS

JY1907_MAGNETIC: N = NO MAGNETIC MATERIAL

JY1907_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY1907

JY1907  HISTORY     - Date     Condition        Report By

JY1907  HISTORY     - 1992     MONUMENTED       OHDT

JY1907

JY1907                          STATION DESCRIPTION

JY1907

JY1907'DESCRIBED BY OHIO DEPARTMENT OF TRANSPORTATION 1992

JY1907'3.2 MILES (5.1 KM) NORTHWEST OF GORE, ALONG STATE ROUTE 93, 228 FEET

JY1907'(69.5 M) SOUTHEAST OF CENTERLINE OF STATE ROUTE 668 NORTH TO

JY1907'MAXVILLE, 120 FEET (36.6 M) NORTHWEST OF DRIVE TO RESIDENCE, 20 FEET

JY1907'(6.1 M) NORTH OF CENTERLINE IN NORTHEAST CONCRETE WINGWALL OF BRIDGE

JY1907'NO HOC-93-18.41.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0108 ***********************************************************************

JY0108  DESIGNATION -  760

JY0108  PID         -  JY0108

JY0108  STATE/COUNTY-  OH/HOCKING

JY0108  USGS QUAD   -  GORE (1992)

JY0108

JY0108                         *CURRENT SURVEY CONTROL

JY0108  ___________________________________________________________________

JY0108* NAD 83(1986)-  39 35 24.     (N)    277 39 04      (E)     SCALED

JY0108* NAVD 88     -       231.296  (meters)     758.84   (feet)  POSTED

JY0108  ___________________________________________________________________

JY0108  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY0108  DYNAMIC HT  -         231.160 (meters)     758.40  (feet)  COMP

JY0108  MODELED GRAV-     980,035.7   (mgal)                       NAVD 88

JY0108

JY0108  VERT ORDER  -  * POSTED, Code F , SEE BELOW

JY0108

JY0108.The horizontal coordinates were scaled from a topographic map and have

JY0108.an estimated accuracy of +/- 6 seconds.

JY0108

JY0108.The orthometric height was determined by differential leveling

JY0108.and adjusted by the National Geodetic Survey in 1992..

JY0108.* This is a POSTED BENCH MARK height.  Code F indicates a distribution

JY0108.rate greater than 8.0 mm/km.

JY0108

JY0108.The geoid height was determined by GEOID03.

JY0108

JY0108.The dynamic height is computed by dividing the NAVD 88

JY0108.geopotential number by the normal gravity value computed on the

JY0108.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0108.degrees latitude (g = 980.6199 gals.).

JY0108

JY0108.The modeled gravity was interpolated from observed gravity values.

JY0108

JY0108;                    North         East    Units  Estimated Accuracy

JY0108;SPC OH S     -   176,520.      612,980.      MT  (+/- 180 meters Scaled)

JY0108

JY0108                          SUPERSEDED SURVEY CONTROL

JY0108

JY0108  NGVD 29 (??/??/92)  231.546  (m)          759.66   (f) ADJ UNCH    2 0

JY0108

JY0108.Superseded values are not recommended for survey control.

JY0108.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0108.See file dsdata.txt to determine how the superseded data were derived.

JY0108

JY0108_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD841831(NAD 83)

JY0108_MARKER: DD = SURVEY DISK

JY0108_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0108_SP_SET: ABUTMENT

JY0108_STAMPING: 760

JY0108_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0108

JY0108  HISTORY     - Date     Condition        Report By

JY0108  HISTORY     - UNK      MONUMENTED       USGS

JY0108  HISTORY     - 1934     GOOD             NGS

JY0108

JY0108                          STATION DESCRIPTION

JY0108

JY0108'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0108'5.2 MI NE FROM LOGAN.

JY0108'5.2 MILES NORTHEAST ALONG STATE HIGHWAY 75 FROM THE POST OFFICE

JY0108'AT LOGAN, HOCKING COUNTY, 105 FEET WEST OF THE CENTER OF THE

JY0108'JUNCTION OF STATE HIGHWAY 312, 90 FEET NORTHEAST OF THE CENTER OF

JY0108'A GRAVEL ROAD CROSSING, AT AN ABANDONED BRIDGE, IN THE TOP OF THE

JY0108'SOUTH ABUTMENT, 30 FEET EAST OF THE SOUTHEAST POST OF A CONCRETE

JY0108'BRIDGE, 60 FEET EAST OF THE CENTERLINE OF THE HIGHWAY, 12 FEET

JY0108'WEST OF A 24-INCH ELM TREE, AND ABOUT 2 FEET LOWER THAN THE ROADWAY.

JY0108'A UNITED STATES GEOLOGICAL SURVEY STANDARD DISK, STAMPED 760.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0219 ***********************************************************************

JY0219  DESIGNATION -  776

JY0219  PID         -  JY0219

JY0219  STATE/COUNTY-  OH/HOCKING

JY0219  USGS QUAD   -  UNION FURNACE (1995)

JY0219

JY0219                         *CURRENT SURVEY CONTROL

JY0219  ___________________________________________________________________

JY0219* NAD 83(1986)-  39 23 52.     (N)    277 38 11      (E)     SCALED

JY0219* NAVD 88     -       236.534  (meters)     776.03   (feet)  ADJUSTED

JY0219  ___________________________________________________________________

JY0219  GEOID HEIGHT-         -34.07  (meters)                     GEOID03

JY0219  DYNAMIC HT  -         236.390 (meters)     775.56  (feet)  COMP

JY0219  MODELED GRAV-     980,015.5   (mgal)                       NAVD 88

JY0219

JY0219  VERT ORDER  -  SECOND    CLASS 0

JY0219

JY0219.The horizontal coordinates were scaled from a topographic map and have

JY0219.an estimated accuracy of +/- 6 seconds.

JY0219

JY0219.The orthometric height was determined by differential leveling

JY0219.and adjusted by the National Geodetic Survey in June 1991..

JY0219

JY0219.The geoid height was determined by GEOID03.

JY0219

JY0219.The dynamic height is computed by dividing the NAVD 88

JY0219.geopotential number by the normal gravity value computed on the

JY0219.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0219.degrees latitude (g = 980.6199 gals.).

JY0219

JY0219.The modeled gravity was interpolated from observed gravity values.

JY0219

JY0219;                    North         East    Units  Estimated Accuracy

JY0219;SPC OH S     -   155,180.      611,750.      MT  (+/- 180 meters Scaled)

JY0219

JY0219                          SUPERSEDED SURVEY CONTROL

JY0219

JY0219  NGVD 29 (??/??/92)  236.676  (m)          776.49   (f) ADJ UNCH    2 0

JY0219

JY0219.Superseded values are not recommended for survey control.

JY0219.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0219.See file dsdata.txt to determine how the superseded data were derived.

JY0219

JY0219_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD825618(NAD 83)

JY0219_MARKER: DD = SURVEY DISK

JY0219_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0219_SP_SET: BRIDGE

JY0219_STAMPING: 776 ADJ

JY0219_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0219

JY0219  HISTORY     - Date     Condition        Report By

JY0219  HISTORY     - UNK      MONUMENTED       USGS

JY0219  HISTORY     - 1943     GOOD             NGS

JY0219  HISTORY     - 1976     GOOD             NGS

JY0219

JY0219                          STATION DESCRIPTION

JY0219

JY0219'DESCRIBED BY NATIONAL GEODETIC SURVEY 1943

JY0219'AT STARR.

JY0219'AT STARR, HOCKING COUNTY, ON THE CHESAPEAKE AND OHIO RAILWAY,

JY0219'110 YARDS SOUTH OF THE SOUTHWEST CORNER OF THE STATION, 0.3

JY0219'MILE SOUTH OF MILEPOST 61, AT THE WEST SIDE OF THE NORTHWEST

JY0219'WING WALL OF BRIDGE 61.3, IN THE TOP OF THE FIFTH COURSE

JY0219'OF MASONRY, 9 FEET WEST OF THE WEST RAIL, AND ABOUT 3 FEET

JY0219'LOWER THAN THE TRACK.  A UNITED STATES GEOLOGICAL SURVEY STANDARD

JY0219'DISK, STAMPED 776 ADJ.

JY0219

JY0219                          STATION RECOVERY (1976)

JY0219

JY0219'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1976

JY0219'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0543 ***********************************************************************

JY0543  DESIGNATION -  782

JY0543  PID         -  JY0543

JY0543  STATE/COUNTY-  OH/HOCKING

JY0543  USGS QUAD   -  LAURELVILLE (1992)

JY0543

JY0543                         *CURRENT SURVEY CONTROL

JY0543  ___________________________________________________________________

JY0543* NAD 83(1986)-  39 29 50.     (N)    277 17 51      (E)     SCALED

JY0543* NAVD 88     -       238.188  (meters)     781.46   (feet)  ADJUSTED

JY0543  ___________________________________________________________________

JY0543  GEOID HEIGHT-         -34.05  (meters)                     GEOID03

JY0543  DYNAMIC HT  -         238.045 (meters)     780.99  (feet)  COMP

JY0543  MODELED GRAV-     980,020.8   (mgal)                       NAVD 88

JY0543

JY0543  VERT ORDER  -  SECOND    CLASS 0

JY0543

JY0543.The horizontal coordinates were scaled from a topographic map and have

JY0543.an estimated accuracy of +/- 6 seconds.

JY0543

JY0543.The orthometric height was determined by differential leveling

JY0543.and adjusted by the National Geodetic Survey in June 1991..

JY0543

JY0543.The geoid height was determined by GEOID03.

JY0543

JY0543.The dynamic height is computed by dividing the NAVD 88

JY0543.geopotential number by the normal gravity value computed on the

JY0543.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0543.degrees latitude (g = 980.6199 gals.).

JY0543

JY0543.The modeled gravity was interpolated from observed gravity values.

JY0543

JY0543;                    North         East    Units  Estimated Accuracy

JY0543;SPC OH S     -   166,230.      582,580.      MT  (+/- 180 meters Scaled)

JY0543

JY0543                          SUPERSEDED SURVEY CONTROL

JY0543

JY0543  NGVD 29 (??/??/92)  238.328  (m)          781.91   (f) ADJ UNCH    2 0

JY0543

JY0543.Superseded values are not recommended for survey control.

JY0543.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0543.See file dsdata.txt to determine how the superseded data were derived.

JY0543

JY0543_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD536733(NAD 83)

JY0543_MARKER: DD = SURVEY DISK

JY0543_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0543_SP_SET: BRIDGE

JY0543_STAMPING: 782

JY0543_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0543

JY0543  HISTORY     - Date     Condition        Report By

JY0543  HISTORY     - UNK      MONUMENTED       USGS

JY0543  HISTORY     - 1936     GOOD             NGS

JY0543

JY0543                          STATION DESCRIPTION

JY0543

JY0543'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0543'1.9 MI W FROM SOUTH PERRY.

JY0543'1.9 MILES WEST ALONG STATE HIGHWAY 180 FROM THE POST OFFICE AT

JY0543'SOUTH PERRY, HOCKING COUNTY, 157 YARDS WEST OF THE INTERSECTION

JY0543'OF A GRAVEL ROAD, 146 YARDS WEST OF THE NORTHWEST CORNER OF A

JY0543'HOUSE, AT A CONCRETE BRIDGE, IN THE TOP OF THE WEST END OF THE

JY0543'WEST WING WALL, 7 YARDS SOUTH OF THE CENTERLINE OF THE HIGHWAY,

JY0543'5 YARDS NORTH OF A FENCE, AND LEVEL WITH THE HIGHWAY.  A UNITED

JY0543'STATES GEOLOGICAL SURVEY STANDARD DISK, STAMPED 782.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0542 ***********************************************************************

JY0542  DESIGNATION -  804

JY0542  PID         -  JY0542

JY0542  STATE/COUNTY-  OH/HOCKING

JY0542  USGS QUAD   -  LAURELVILLE (1992)

JY0542

JY0542                         *CURRENT SURVEY CONTROL

JY0542  ___________________________________________________________________

JY0542* NAD 83(1986)-  39 29 49.     (N)    277 19 47      (E)     SCALED

JY0542* NAVD 88     -       244.954  (meters)     803.65   (feet)  ADJUSTED

JY0542  ___________________________________________________________________

JY0542  GEOID HEIGHT-         -34.06  (meters)                     GEOID03

JY0542  DYNAMIC HT  -         244.807 (meters)     803.17  (feet)  COMP

JY0542  MODELED GRAV-     980,020.7   (mgal)                       NAVD 88

JY0542

JY0542  VERT ORDER  -  SECOND    CLASS 0

JY0542

JY0542.The horizontal coordinates were scaled from a topographic map and have

JY0542.an estimated accuracy of +/- 6 seconds.

JY0542

JY0542.The orthometric height was determined by differential leveling

JY0542.and adjusted by the National Geodetic Survey in June 1991..

JY0542

JY0542.The geoid height was determined by GEOID03.

JY0542

JY0542.The dynamic height is computed by dividing the NAVD 88

JY0542.geopotential number by the normal gravity value computed on the

JY0542.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0542.degrees latitude (g = 980.6199 gals.).

JY0542

JY0542.The modeled gravity was interpolated from observed gravity values.

JY0542

JY0542;                    North         East    Units  Estimated Accuracy

JY0542;SPC OH S     -   166,200.      585,350.      MT  (+/- 180 meters Scaled)

JY0542

JY0542                          SUPERSEDED SURVEY CONTROL

JY0542

JY0542  NGVD 29 (??/??/92)  245.092  (m)          804.11   (f) ADJ UNCH    2 0

JY0542

JY0542.Superseded values are not recommended for survey control.

JY0542.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0542.See file dsdata.txt to determine how the superseded data were derived.

JY0542

JY0542_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD563732(NAD 83)

JY0542_MARKER: Z = SEE DESCRIPTION

JY0542_SETTING: 0 = UNSPECIFIED SETTING

JY0542_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY0542

JY0542  HISTORY     - Date     Condition        Report By

JY0542  HISTORY     - UNK      MONUMENTED       USGS

JY0542  HISTORY     - 1936     GOOD             NGS

JY0542

JY0542                          STATION DESCRIPTION

JY0542

JY0542'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0542'AT SOUTH PERRY.

JY0542'AT SOUTH PERRY, HOCKING COUNTY, ON STATE HIGHWAY 180, 28 YARDS

JY0542'NORTHWEST OF THE CENTER OF THE JUNCTION OF A STREET LEADING

JY0542'NORTH, 18 YARDS SOUTHWEST OF THE SOUTHWEST CORNER OF A SINCLAIR

JY0542'SERVICE STATION, 11 YARDS NORTH OF THE CENTERLINE OF THE

JY0542'HIGHWAY, 4 YARDS SOUTH OF A CONCRETE CURB, ONE FOOT NORTHEAST

JY0542'OF THE SOUTHWEST CORNER OF A FENCE, AND ABOUT ONE FOOT ABOVE

JY0542'THE LEVEL OF THE HIGHWAY.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1398 ***********************************************************************

JY1398  DESIGNATION -  BARTON RESET

JY1398  PID         -  JY1398

JY1398  STATE/COUNTY-  OH/HOCKING

JY1398  USGS QUAD   -  NEW PLYMOUTH (1992)

JY1398

JY1398                         *CURRENT SURVEY CONTROL

JY1398  ___________________________________________________________________

JY1398* NAD 83(1995)-  39 26 22.31479(N)    277 30 13.68585(E)     ADJUSTED

JY1398* NAVD 88     -       342.     (meters)    1122.     (feet)  SCALED

JY1398  ___________________________________________________________________

JY1398  LAPLACE CORR-          -0.44  (seconds)                    DEFLEC99

JY1398  GEOID HEIGHT-         -34.07  (meters)                     GEOID03

JY1398

JY1398  HORZ ORDER  -  FIRST

JY1398

JY1398.The horizontal coordinates were established by classical geodetic methods

JY1398.and adjusted by the National Geodetic Survey in April 1998..

JY1398

JY1398.The orthometric height was scaled from a topographic map.

JY1398

JY1398.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1398

JY1398.The geoid height was determined by GEOID03.

JY1398

JY1398;                    North         East     Units Scale Factor Converg.

JY1398;SPC OH S     -   159,807.239   600,327.253   MT  0.99993637   +0 00 08.7

JY1398;SPC OH S     -   524,300.92  1,969,573.66   sFT  0.99993637   +0 00 08.7

JY1398;UTM  17      - 4,366,621.854   371,244.087   MT  0.99980411   -0 57 02.2

JY1398

JY1398!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1398!SPC OH S     -   0.99995171  x   0.99993637  =   0.99988809

JY1398!UTM  17      -   0.99995171  x   0.99980411  =   0.99975583

JY1398

JY1398|---------------------------------------------------------------------|

JY1398| PID    Reference Object                     Distance      Geod. Az  |

JY1398|                                                           dddmmss.s |

JY1398| CE6200 BARTON RM 4                          50.826 METERS 23325     |

JY1398| CE6201 BARTON RM 5                          37.119 METERS 28210     |

JY1398| CE6199 BARTON AZ MK                                       2884635.8 |

JY1398| JY1399 ROCK HOUSE LOOKOUT TOWER            APPROX.11.8 KM 3015233.1 |

JY1398|---------------------------------------------------------------------|

JY1398

JY1398                          SUPERSEDED SURVEY CONTROL

JY1398

JY1398  NAD 83(1986)-  39 26 22.31815(N)    277 30 13.68059(E) AD(       ) 1

JY1398  NAD 27      -  39 26 22.07700(N)    277 30 13.24400(E) AD(       ) 1

JY1398

JY1398.Superseded values are not recommended for survey control.

JY1398.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1398.See file dsdata.txt to determine how the superseded data were derived.

JY1398

JY1398_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7124466622(NAD 83)

JY1398_MARKER: V = STONE MONUMENT

JY1398_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY1398

JY1398  HISTORY     - Date     Condition        Report By

JY1398  HISTORY     - 1945     MONUMENTED       CGS

JY1398  HISTORY     - 1945     SEE DESCRIPTION  CGS

JY1398

JY1398                          STATION DESCRIPTION

JY1398

JY1398'DESCRIBED BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1398'STATION WAS RECOVERED AS DESCRIBED IN 1885.  THE GRANITE POST WAS

JY1398'REMOVED AND A STANDARD STATION DISK WAS REPLACED HOLDING THE

JY1398'ORIGINAL POSITION.  THE ORIGINAL STONE REFERENCE MARKS 1, 2, AND 3,

JY1398'WERE NOT RECOVERED.  TWO REFERENCE MARKS 4 AND 5 AND AN AZIMUTH MARK

JY1398'WERE ESTABLISHED.  THE COMPLETE NEW DESCRIPTION FOLLOWS-

JY1398'

JY1398'STATION IS LOCATED ABOUT 9.5 MILES SE OF THE CITY OF LOGAN ON LAND

JY1398'OWNED BY W.E. CLINE.  THE MARK IS SITUATED ON THE HIGHEST GROUND

JY1398'OF A SMALL BARE KNOLL ABOUT 0.15 MILE S OF THE FARM DWELLING.  THE

JY1398'SURFACE MARK IS APPROXIMATELY 10 INCHES BELOW THE SURFACE OF

JY1398'THE GROUND.  THE DISK IS STAMPED BARTON 1885 1945.

JY1398'

JY1398'SURFACE, UNDERGROUND, REFERENCE, AND AZIMUTH MARKS ARE STANDARD

JY1398'BRONZE DISKS SET IN CONCRETE.

JY1398'

JY1398'REFERENCE MARK 4 IS 166.75 FEET WSW OF THE STATION ON THE SW SLOPE OF

JY1398'THE HILL AT THE EDGE OF CULTIVATION.  THE MARK PROJECTS 4 INCHES

JY1398'AND THE DISK IS STAMPED BARTON NO 4 1945.

JY1398'

JY1398'REFERENCE MARK 5 IS 121.78 FEET WNW OF THE STATION ON THE W SLOPE OF

JY1398'THE HILL AT THE EDGE OF CULTIVATION.  THE MARK PROJECTS 4 INCHES

JY1398'AND THE DISK IS STAMPED BARTON NO 5 1945.  A STANDARD WITNESS POST

JY1398'WAS PLACED AT THIS MARK.

JY1398'

JY1398'AZIMUTH MARK IS APPROXIMATELY 0.25 MILE WNW OF THE STATION IN THE S

JY1398'RIGHT-OF-WAY FENCELINE.  IT IS ON A ROAD BANK 50 FEET W OF A FARM

JY1398'LANE, 16 FEET S OF THE APPROXIMATE CENTER OF THE DIRT ROAD, AND 2

JY1398'FEET SE OF A STANDARD WITNESS POST.  THE MARK PROJECTS 4 INCHES AND

JY1398'THE DISK IS STAMPED BARTON 1945.

JY1398'

JY1398'TO REACH THE STATION FROM THE JUNCTION OF U.S. HIGHWAY 33 AND STATE

JY1398'HIGHWAY 75 IN LOGAN, GO S ON STATE HIGHWAY 75 FOR 6.3 MILES TO A

JY1398'GULF SERVICE STATION IN THE VILLAGE OF EWING.  CONTINUE ON HIGHWAY 75

JY1398'FOR 0.6 MILE TO A DIRT T-ROAD RIGHT.  TURN RIGHT AND FOLLOW THE

JY1398'DIRT ROAD FOR 0.5 MILE, KEEP RIGHT AT FORKS FOR 0.5 MILE, THEN LEFT

JY1398'FORK FOR 1.35 MILES TO A CROSSROAD.  TURN LEFT AND GO 0.55 MILE,

JY1398'THEN TAKE THE RIGHT FORK FOR 1.8 MILES TO A CROSSROAD.  TURN RIGHT

JY1398'FOR 0.5 MILE TO A FARM LANE LEFT.* TURN LEFT TO THE FARM HOUSE AND

JY1398'GO S ALONG TOP OF RIDGE FOR 0.15 MILE TO THE HIGHEST POINT AND

JY1398'STATION.

JY1398'

JY1398'*TO REACH THE AZIMUTH MARK FROM THIS POINT, CONTINUE W ON THE DIRT

JY1398'ROAD FOR 0.25 MILE TO THE MARK ON THE LEFT AS DESCRIBED.

JY1398'

JY1398'HEIGHT OF LIGHT ABOVE STATION MARK - 22 METERS.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1427 ***********************************************************************

JY1427  DESIGNATION -  BLACK

JY1427  PID         -  JY1427

JY1427  STATE/COUNTY-  OH/HOCKING

JY1427  USGS QUAD   -  CLEARPORT (1994)

JY1427

JY1427                         *CURRENT SURVEY CONTROL

JY1427  ___________________________________________________________________

JY1427* NAD 83(1995)-  39 32 28.64704(N)    277 21 01.44429(E)     ADJUSTED

JY1427* NAVD 88     -       372.     (meters)    1220.     (feet)  SCALED

JY1427  ___________________________________________________________________

JY1427  LAPLACE CORR-          -0.92  (seconds)                    DEFLEC99

JY1427  GEOID HEIGHT-         -34.07  (meters)                     GEOID03

JY1427

JY1427  HORZ ORDER  -  FIRST

JY1427

JY1427.The horizontal coordinates were established by classical geodetic methods

JY1427.and adjusted by the National Geodetic Survey in April 1998..

JY1427

JY1427.The orthometric height was scaled from a topographic map.

JY1427

JY1427.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1427

JY1427.The geoid height was determined by GEOID03.

JY1427

JY1427;                    North         East     Units Scale Factor Converg.

JY1427;SPC OH S     -   171,114.954   587,140.894   MT  0.99993965   -0 05 41.7

JY1427;SPC OH S     -   561,399.64  1,926,311.42   sFT  0.99993965   -0 05 41.7

JY1427;UTM  17      - 4,378,146.301   358,249.325   MT  0.99984739   -1 03 01.3

JY1427

JY1427!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1427!SPC OH S     -   0.99994701  x   0.99993965  =   0.99988666

JY1427!UTM  17      -   0.99994701  x   0.99984739  =   0.99979441

JY1427

JY1427:                Primary Azimuth Mark                     Grid Az

JY1427:SPC OH S     -  BLACK AZ MK                              054 05 54.4

JY1427:UTM  17      -  BLACK AZ MK                              055 03 14.0

JY1427

JY1427|---------------------------------------------------------------------|

JY1427| PID    Reference Object                     Distance      Geod. Az  |

JY1427|                                                           dddmmss.s |

JY1427| CE5770 BLACK AZ MK                                        0540012.7 |

JY1427| JY1399 ROCK HOUSE LOOKOUT TOWER            APPROX. 6.0 KM 1475127.0 |

JY1427| CE5771 BLACK RM 1                           49.338 METERS 23325     |

JY1427| CE5772 BLACK RM 2                           48.872 METERS 31451     |

JY1427| JY1429 SCOUTS                              APPROX.14.2 KM 3571024.7 |

JY1427|---------------------------------------------------------------------|

JY1427

JY1427                          SUPERSEDED SURVEY CONTROL

JY1427

JY1427  NAD 83(1986)-  39 32 28.65193(N)    277 21 01.44040(E) AD(       ) 1

JY1427  NAD 27      -  39 32 28.42800(N)    277 21 01.03300(E) AD(       ) 1

JY1427

JY1427.Superseded values are not recommended for survey control.

JY1427.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1427.See file dsdata.txt to determine how the superseded data were derived.

JY1427

JY1427_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD5824978146(NAD 83)

JY1427_MARKER: DS = TRIANGULATION STATION DISK

JY1427_SETTING: 0 = UNSPECIFIED SETTING

JY1427

JY1427  HISTORY     - Date     Condition        Report By

JY1427  HISTORY     - 1945     MONUMENTED       CGS

JY1427

JY1427                          STATION DESCRIPTION

JY1427

JY1427'DESCRIBED BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1427'STATION IS LOCATED ABOUT 11 MILES BY ROAD NE OF LAURELVILLE, ON

JY1427'PROPERTY OF MRS. HIMBEGAR WHO LIVES ABOUT 200 YARDS NE OF THE

JY1427'STATION.  THE MARK IS SITUATED ON THE HIGHEST POINT OF PASTURE LAND

JY1427'NEAR THE W EDGE OF THE FIELD.  IT IS 179.5 FEET N OF A

JY1427'RIGHT-OF-WAY FENCELINE AND 123 FEET E OF A N-S FENCELINE.

JY1427'

JY1427'STATION MARK IS A BRONZE STATION DISK SET IN TOP OF A CONCRETE POST.

JY1427'THE MARK IS 12 INCHES BELOW THE SURFACE OF THE GROUND AND THE

JY1427'DISK IS STAMPED BLACK 1945.  UNDERGROUND MARK IS A BRONZE STATION

JY1427'DISK SET IN CONCRETE.

JY1427'

JY1427'REFERENCE MARK 1 IS 161.87 FEET SW OF THE STATION AT THE W EDGE OF

JY1427'THE FIELD.  IT IS ABOUT 50 FEET N OF A FENCE CORNER AND 1.5 FEET N

JY1427'OF A STANDARD WITNESS POST.  IT IS A BRONZE REFERENCE DISK SET IN TOP

JY1427'OF A CONCRETE POST.  THE MARK PROJECTS 4 INCHES AND THE DISK IS

JY1427'STAMPED BLACK NO. 1 1945.

JY1427'

JY1427'REFERENCE MARK 2 IS 160.34 FEET NW OF THE STATION IN THE FENCELINE.

JY1427'IT IS 2 FEET S OF A BOARD GATE AND 1 FOOT E OF THE FENCELINE.  IT IS

JY1427'A BRONZE REFERENCE DISK SET IN TOP OF A CONCRETE POST.  THE MARK

JY1427'PROJECTS 4 INCHES AND THE DISK IS STAMPED BLACK NO. 2 1945.

JY1427'

JY1427'AZIMUTH MARK IS APPROXIMATELY 0.15 MILE NE OF THE STATION AND ABOUT

JY1427'100 FEET S OF THE HIMBEGAR HOUSE.  IT IS SITUATED IN THE E

JY1427'RIGHT-OF-WAY FENCELINE, 16 FEET E OF THE APPROXIMATE CENTER OF THE

JY1427'ROAD, AND 2 FEET N OF A STANDARD WITNESS POST.  IT IS A BRONZE

JY1427'AZIMUTH DISK SET IN TOP OF A CONCRETE POST.  THE MARK PROJECTS 4

JY1427'INCHES AND THE DISK IS STAMPED BLACK 1945.

JY1427'

JY1427'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAYS 56 AND 180

JY1427'IN LAURELVILLE, GO NE ON STATE HIGHWAY 180 FOR 8.1 MILES TO A SIDE

JY1427'ROAD LEADING SHARP LEFT AT THE MOUND CROSSING GENERAL STORE.  TURN

JY1427'LEFT AND FOLLOW A GRAVELED ROAD NW FOR 3.05 MILES, THEN TAKE THE

JY1427'RIGHT FORK FOR 0.1 MILE TO THE AZIMUTH MARK AND MRS. HIMBEGARS HOUSE

JY1427'ON THE RIGHT.  TURN SHARP LEFT THROUGH A BOARD GATE AND BARN LOT

JY1427'AND ENTER A PASTURE FIELD THROUGH ANOTHER GATE.  FOLLOW THE TOP OF

JY1427'THE RIDGE FOR ABOUT 0.1 MILE TO THE HIGHEST GROUND AND STATION

JY1427'NEAR THE W EDGE OF THE FIELD AS DESCRIBED.

JY1427'

JY1427'HEIGHT OF LIGHT ABOVE STATION MARK - 26 METERS.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1366 ***********************************************************************

JY1366  DESIGNATION -  BROOKS RESET

JY1366  PID         -  JY1366

JY1366  STATE/COUNTY-  OH/HOCKING

JY1366  USGS QUAD   -  NELSONVILLE (1995)

JY1366

JY1366                         *CURRENT SURVEY CONTROL

JY1366  ___________________________________________________________________

JY1366* NAD 83(1995)-  39 29 01.85149(N)    277 46 04.76938(E)     ADJUSTED

JY1366* NAVD 88     -       323.     (meters)    1060.     (feet)  SCALED

JY1366  ___________________________________________________________________

JY1366  LAPLACE CORR-          -0.27  (seconds)                    DEFLEC99

JY1366  GEOID HEIGHT-         -34.07  (meters)                     GEOID03

JY1366

JY1366  HORZ ORDER  -  FIRST

JY1366

JY1366.The horizontal coordinates were established by classical geodetic methods

JY1366.and adjusted by the National Geodetic Survey in April 1998..

JY1366

JY1366.The orthometric height was scaled from a topographic map.

JY1366

JY1366.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1366

JY1366.The geoid height was determined by GEOID03.

JY1366

JY1366;                    North         East     Units Scale Factor Converg.

JY1366;SPC OH S     -   164,761.258   623,054.722   MT  0.99993741   +0 10 12.2

JY1366;SPC OH S     -   540,554.23  2,044,138.70   sFT  0.99993741   +0 10 12.2

JY1366;UTM  17      - 4,371,196.302   394,047.029   MT  0.99973821   -0 47 00.5

JY1366

JY1366!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1366!SPC OH S     -   0.99995470  x   0.99993741  =   0.99989212

JY1366!UTM  17      -   0.99995470  x   0.99973821  =   0.99969292

JY1366

JY1366:                Primary Azimuth Mark                     Grid Az

JY1366:SPC OH S     -  BROOKS AZ MK                             156 09 49.7

JY1366:UTM  17      -  BROOKS AZ MK                             157 07 02.4

JY1366

JY1366|---------------------------------------------------------------------|

JY1366| PID    Reference Object                     Distance      Geod. Az  |

JY1366|                                                           dddmmss.s |

JY1366| CE5817 BROOKS AZ MK                                       1562001.9 |

JY1366| CE5818 BROOKS RM 4                          11.824 METERS 17440     |

JY1366| CE5819 BROOKS RM 5                          11.320 METERS 33030     |

JY1366|---------------------------------------------------------------------|

JY1366

JY1366                          SUPERSEDED SURVEY CONTROL

JY1366

JY1366  NAD 83(1986)-  39 29 01.85345(N)    277 46 04.76590(E) AD(       ) 1

JY1366  NAD 27      -  39 29 01.60300(N)    277 46 04.30300(E) AD(       ) 1

JY1366

JY1366.Superseded values are not recommended for survey control.

JY1366.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1366.See file dsdata.txt to determine how the superseded data were derived.

JY1366

JY1366_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD9404771196(NAD 83)

JY1366_MARKER: Z = SEE DESCRIPTION

JY1366_SETTING: 0 = UNSPECIFIED SETTING

JY1366

JY1366  HISTORY     - Date     Condition        Report By

JY1366  HISTORY     - 1944     MONUMENTED       CGS

JY1366  HISTORY     - 1944     SEE DESCRIPTION  CGS

JY1366  HISTORY     - 1945     GOOD             CGS

JY1366

JY1366                          STATION DESCRIPTION

JY1366

JY1366'DESCRIBED BY COAST AND GEODETIC SURVEY 1944 (FN)

JY1366'STATION WAS RECOVERED AND FOUND TO BE IN BAD CONDITION.  THE STATION

JY1366'AND REFERENCE MARKS WERE BROKEN AND FOUND LYING ON THE GROUND.  THE

JY1366'UNDERGROUND MARK WAS RECOVERED AND FOUND TO BE IN VERY GOOD

JY1366'CONDITION.  A NEW STATION MARK, TWO NEW REFERENCE MARKS, AND AN

JY1366'AZIMUTH MARK WERE ESTABLISHED BY THIS PARTY.

JY1366'

JY1366'STATION IS LOCATED ABOUT 1-1/2 MILES N OF NELSONVILLE, ON THE HIGHEST

JY1366'POINT OF A TIMBERED HILL OWNED BY THE FEDERAL GOVERNMENT.  IT IS

JY1366'11 FEET W OF A TRIANGULAR BLAZE ON A 15-INCH OAK TREE, 32 FEET S OF A

JY1366'TRIANGULAR BLAZE ON A 15-INCH OAK TREE, 11 FEET E OF A TRIANGULAR

JY1366'BLAZE ON A 24-INCH OAK TREE, AND 2 FEET E OF A WITNESS POST.  THE

JY1366'DISK IS STAMPED BROOKS 1881 1944.

JY1366'

JY1366'SURFACE, UNDERGROUND, REFERENCE, AND AZIMUTH MARKS ARE BRONZE DISKS

JY1366'SET IN CONCRETE.

JY1366'

JY1366'REFERENCE MARK 4 IS 29 FEET SE OF TRIANGULAR BLAZE ON 15-INCH OAK

JY1366'TREE.  IT IS STAMPED BROOKS NO 4 1944.

JY1366'

JY1366'REFERENCE MARK 5 IS 29 FEET N OF TRIANGULAR BLAZE ON 15-INCH OAK TREE

JY1366'AND 33 FEET N OF TRIANGULAR BLAZE ON 15-INCH OAK TREE.  IT IS

JY1366'STAMPED BROOKS NO 5 1944.

JY1366'

JY1366'AZIMUTH MARK IS 16 FEET N OF THE CENTER LINE OF ROAD, NEAR THE W END

JY1366'OF A ROADSIDE PARKWAY, 11 FEET E OF 10-INCH TWIN OAK TREE, AND 3

JY1366'FEET N OF A WITNESS POST.  STAMPED BROOKS 1944.

JY1366'

JY1366'TO REACH THE STATION FROM THE TRAFFIC LIGHT AT THE N END OF SQUARE IN

JY1366'NELSONVILLE, GO N ON FORT STREET (PASSING SCHOOL ON E SIDE) FOR

JY1366'0.25 MILE TO A SMALL STORE ON THE LEFT AND A BRICK ROAD LEADING UP

JY1366'HILL ON THE RIGHT.  TURN RIGHT AND GO 0.5 MILE TO A FORK AT THE TOP

JY1366'OF RIDGE, TAKE LEFT FORK AND FOLLOW MAIN ROAD FOR 0.80 MILE TO A

JY1366'ROADSIDE PARKWAY ON THE RIGHT AND THE AZIMUTH MARK.  CONTINUE

JY1366'STRAIGHT AHEAD FOR 0.15 MILE TO A TRACK ROAD RIGHT AT THE BASE OF THE

JY1366'HILL, TURN RIGHT ON TRACK ROAD AND GO 0.25 MILE TO A FORK, TAKE

JY1366'RIGHT FORK AND GO 0.2 MILE TO A FORK AND A TRIANGULAR BLAZE ON A

JY1366'SMALL OAK TREE, TAKE RIGHT FORK AND GO UPGRADE FOR 0.1 MILE TO THE

JY1366'TOP OF THE RIDGE.  TURN SHARP RIGHT THROUGH BRUSH TO THREE OAK TREES

JY1366'WITH TRIANGULAR BLAZES AND THE STATION.

JY1366'

JY1366'HEIGHT OF LIGHT ABOVE STATION MARK - 26 METERS.

JY1366

JY1366                          STATION RECOVERY (1945)

JY1366

JY1366'RECOVERY NOTE BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1366'STATION WAS RECOVERED AS DESCRIBED BY FRED NATELLA IN 1944.  AT THIS

JY1366'TIME THE STATION WAS RE-MARKED, REFERENCE MARKS AND AN AZIMUTH MARK

JY1366'WERE ESTABLISHED.  THE 1944 DESCRIPTION IS COMPLETE AND ADEQUATE, AND

JY1366'SINCE THE STATION WAS OCCUPIED BY THIS PARTY LATE LAST FALL, A

JY1366'COMPLETE DESCRIPTION IS UNNECESSARY AT THIS TIME.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1424 ***********************************************************************

JY1424  DESIGNATION -  BROWN

JY1424  PID         -  JY1424

JY1424  STATE/COUNTY-  OH/HOCKING

JY1424  USGS QUAD   -  ROCKBRIDGE (1992)

JY1424

JY1424                         *CURRENT SURVEY CONTROL

JY1424  ___________________________________________________________________

JY1424* NAD 83(1995)-  39 33 53.86928(N)    277 23 49.97132(E)     ADJUSTED

JY1424* NAVD 88     -       364.     (meters)    1194.     (feet)  SCALED

JY1424  ___________________________________________________________________

JY1424  LAPLACE CORR-          -1.33  (seconds)                    DEFLEC99

JY1424  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY1424

JY1424  HORZ ORDER  -  SECOND

JY1424

JY1424.The horizontal coordinates were established by classical geodetic methods

JY1424.and adjusted by the National Geodetic Survey in April 1998..

JY1424

JY1424.The orthometric height was scaled from a topographic map.

JY1424

JY1424.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1424

JY1424.The geoid height was determined by GEOID03.

JY1424

JY1424;                    North         East     Units Scale Factor Converg.

JY1424;SPC OH S     -   173,737.475   591,167.807   MT  0.99994086   -0 03 54.8

JY1424;SPC OH S     -   570,003.70  1,939,523.05   sFT  0.99994086   -0 03 54.8

JY1424;UTM  17      - 4,380,701.039   362,319.011   MT  0.99983339   -1 01 15.8

JY1424

JY1424!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1424!SPC OH S     -   0.99994827  x   0.99994086  =   0.99988913

JY1424!UTM  17      -   0.99994827  x   0.99983339  =   0.99978167

JY1424

JY1424                          SUPERSEDED SURVEY CONTROL

JY1424

JY1424  NAD 83(1986)-  39 33 53.87333(N)    277 23 49.96716(E) AD(       ) 2

JY1424  NAD 27      -  39 33 53.64600(N)    277 23 49.54800(E) AD(       ) 2

JY1424

JY1424.Superseded values are not recommended for survey control.

JY1424.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1424.See file dsdata.txt to determine how the superseded data were derived.

JY1424

JY1424_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD6231980701(NAD 83)

JY1424_MARKER: V = STONE MONUMENT

JY1424_SETTING: 0 = UNSPECIFIED SETTING

JY1424_MARK LOGO: USGS

JY1424_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY1424

JY1424  HISTORY     - Date     Condition        Report By

JY1424  HISTORY     - 1881     MONUMENTED       USGS

JY1424  HISTORY     - 1945     GOOD             CGS

JY1424

JY1424                          STATION DESCRIPTION

JY1424

JY1424'DESCRIBED BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1424'STATION IS LOCATED ON A HIGH PROMINENT HILL ABOUT 13 MILES (BY ROAD)

JY1424'WNW OF LOGAN AND ABOUT 12 MILES (AIR LINE) S OF LANCASTER ON LAND

JY1424'OWNED BY MR. WILLIS BROWN.  IT IS 15.2 FEET E OF THE E FENCE OF

JY1424'BROWNS PRIVATE CEMETERY, 41.5 FEET SE OF THE NE CORNER OF THE

JY1424'CEMETERY, AND 60 FEET NE OF THE SE CORNER OF THE FENCE.  THE MARK IS

JY1424'A 6- BY 6-INCH MARBLE POST, 30 INCHES LONG WITH A CROSS ON THE TOP

JY1424'AND USGS ENGRAVED ON THE TOP.  THE POST PROJECTS ABOUT 8 INCHES.

JY1424'

JY1424'A TRAVERSE CONNECTION WAS MADE TO BROWN 2, DISTANCE 118.31 FEET.

JY1424'

JY1424'TO REACH THE STATION FROM THE JUNCTION OF U.S. HIGHWAY 33 AND STATE

JY1424'HIGHWAY 796 IN ROCKBRIDGE, GO W ON U.S. HIGHWAY 33 FOR 0.5 MILE TO

JY1424'A GRAVELED ROAD LEFT, TURN LEFT AND GO UP A STEEP GRADE AND FOLLOW

JY1424'THE MAIN TRAVELED ROAD FOR 2.1 MILES TO A FORK, KEEP RIGHT AND GO

JY1424'3.1 MILES TO A CROSSROAD AND A HOUSE ON THE NW CORNER, TURN RIGHT AND

JY1424'GO 0.1 MILE TO A DIM ROAD RIGHT LEADING TO THE CEMETERY.  TURN RIGHT

JY1424'FOR 0.05 MILE TO THE STATION AS DESCRIBED.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1425 ***********************************************************************

JY1425  DESIGNATION -  BROWN 2

JY1425  PID         -  JY1425

JY1425  STATE/COUNTY-  OH/HOCKING

JY1425  USGS QUAD   -  ROCKBRIDGE (1992)

JY1425

JY1425                         *CURRENT SURVEY CONTROL

JY1425  ___________________________________________________________________

JY1425* NAD 83(1995)-  39 33 53.40941(N)    277 23 48.58240(E)     ADJUSTED

JY1425* NAVD 88     -       363.34   (+/-2cm)    1192.1    (feet)  VERTCON

JY1425  ___________________________________________________________________

JY1425  X           -     633,903.480 (meters)                     COMP

JY1425  Y           -  -4,882,902.633 (meters)                     COMP

JY1425  Z           -   4,041,065.084 (meters)                     COMP

JY1425  LAPLACE CORR-          -1.32  (seconds)                    DEFLEC99

JY1425  ELLIP HEIGHT-         329.15  (meters)          (10/07/05) GPS OBS

JY1425  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY1425

JY1425  HORZ ORDER  -  FIRST

JY1425  VERT ORDER  -  THIRD ? (See Below)

JY1425  ELLP ORDER  -  FOURTH    CLASS I

JY1425

JY1425.The horizontal coordinates were established by classical geodetic methods

JY1425.and adjusted by the National Geodetic Survey in April 1998..

JY1425

JY1425.The NAVD 88 height was computed by applying the VERTCON shift value to

JY1425.the NGVD 29 height (displayed under SUPERSEDED SURVEY CONTROL.)

JY1425.The vertical order pertains to the NGVD 29 superseded value.

JY1425

JY1425.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JY1425

JY1425.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1425

JY1425.The ellipsoidal height was determined by GPS observations

JY1425.and is referenced to NAD 83.

JY1425

JY1425.The geoid height was determined by GEOID03.

JY1425

JY1425;                    North         East     Units Scale Factor Converg.

JY1425;SPC OH S     -   173,723.331   591,134.639   MT  0.99994085   -0 03 55.7

JY1425;SPC OH S     -   569,957.30  1,939,414.23   sFT  0.99994085   -0 03 55.7

JY1425;UTM  17      - 4,380,687.451   362,285.615   MT  0.99983350   -1 01 16.7

JY1425

JY1425!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1425!SPC OH S     -   0.99994836  x   0.99994085  =   0.99988922

JY1425!UTM  17      -   0.99994836  x   0.99983350  =   0.99978187

JY1425

JY1425:                Primary Azimuth Mark                     Grid Az

JY1425:SPC OH S     -  SUGAR GROVE LOOKOUT TOWER                061 38 33.8

JY1425:UTM  17      -  SUGAR GROVE LOOKOUT TOWER                062 35 54.8

JY1425

JY1425|---------------------------------------------------------------------|

JY1425| PID    Reference Object                     Distance      Geod. Az  |

JY1425|                                                           dddmmss.s |

JY1425| JY1426 SUGAR GROVE LOOKOUT TOWER           APPROX. 9.3 KM 0613438.1 |

JY1425| JY1424 BROWN                                36.060 METERS 06650     |

JY1425| CE5823 BROWN 2 AZ MK                                      0922426.6 |

JY1425| CE5825 BROWN 2 RM 2                         23.145 METERS 10020     |

JY1425| JY1399 ROCK HOUSE LOOKOUT TOWER            APPROX. 7.7 KM 1855522.8 |

JY1425| JY1431 BOYS INDUSTRIAL SCHOOL SPIPE        APPROX. 8.9 KM 3463017.6 |

JY1425| JY1418 BOYS INDUSTRIAL SCHOOL STACK        APPROX. 8.9 KM 3491152.9 |

JY1425| CE5824 BROWN 2 RM 1                         17.636 METERS 35741     |

JY1425|---------------------------------------------------------------------|

JY1425

JY1425                          SUPERSEDED SURVEY CONTROL

JY1425

JY1425  ELLIP H (04/01/98)  329.19   (m)                       GP(       ) 4 1

JY1425  NAD 83(1986)-  39 33 53.41347(N)    277 23 48.57827(E) AD(       ) 1

JY1425  NAD 27      -  39 33 53.18600(N)    277 23 48.15900(E) AD(       ) 1

JY1425  NGVD 29 (07/19/86)  363.48   (m)         1192.5    (f) LEVELING    3

JY1425

JY1425.Superseded values are not recommended for survey control.

JY1425.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1425.See file dsdata.txt to determine how the superseded data were derived.

JY1425

JY1425_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD6228680687(NAD 83)

JY1425_MARKER: DS = TRIANGULATION STATION DISK

JY1425_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY1425_SP_SET: CONCRETE POST

JY1425_STAMPING: BROWN 2 1945

JY1425_MARK LOGO: CGS

JY1425_MAGNETIC: N = NO MAGNETIC MATERIAL

JY1425_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY1425+STABILITY: SURFACE MOTION

JY1425_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JY1425+SATELLITE: SATELLITE OBSERVATIONS - August 27, 1991

JY1425

JY1425  HISTORY     - Date     Condition        Report By

JY1425  HISTORY     - 1945     MONUMENTED       CGS

JY1425  HISTORY     - 1945     GOOD             CGS

JY1425  HISTORY     - 19910827 GOOD             WOOLPT

JY1425

JY1425                          STATION DESCRIPTION

JY1425

JY1425'DESCRIBED BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1425'STATION IS LOCATED ON A HIGH PROMINENT HILL ABOUT 13 MILES (BY ROAD)

JY1425'WNW OF LOGAN AND ABOUT 12 MILES (AIR LINE) S OF LANCASTER.  IT IS

JY1425'ON LAND OWNED BY WILLIS BROWN.  IT IS IN THE W FENCE LINE OF BROWNS

JY1425'PRIVATE OWNED CEMETERY AND ABOUT 1 MILE N OF BROWNS CORNERS.  IT IS

JY1425'18.6 FEET N OF THE SW CORNER OF THE CEMETERY, 81.8 FEET S OF THE NW

JY1425'CORNER OF THE CEMETERY, AND 3 FEET N OF A STANDARD WITNESS POST.

JY1425'

JY1425'STATION MARK IS A BRONZE STATION DISK SET IN TOP OF A CONCRETE POST.

JY1425'THE DISK IS STAMPED BROWN 2 1945 AND IS FLUSH WITH THE GROUND.

JY1425'UNDERGROUND MARK IS A BRONZE STATION DISK SET IN CONCRETE.

JY1425'

JY1425'REFERENCE MARK 1 IS 57.86 FEET N OF THE STATION IN THE W FENCE LINE

JY1425'OF THE CEMETERY.  IT IS 24 FEET SOUTH OF THE WEST CORNER OF THE

JY1425'CEMETERY.  IT IS A BRONZE REFERENCE DISK SET

JY1425'IN TOP OF CONCRETE POST, IS

JY1425'STAMPED BROWN 2 NO. 1 1945, AND IS FLUSH WITH THE GROUND.

JY1425'

JY1425'REFERENCE MARK 2 IS 75.93 FEET SE OF THE STATION IN THE S FENCE LINE

JY1425'OF THE CEMETERY.  IT IS 24.5 FEET W OF THE SE CORNER OF THE

JY1425'CEMETERY.  IT IS A BRONZE REFERENCE DISK SET IN TOP OF A CONCRETE

JY1425'POST, IS STAMPED BROWN 2 NO. 2 1945, AND IS FLUSH WITH THE

JY1425'GROUND.

JY1425'

JY1425'AZIMUTH MARK IS ABOUT 0.3 MILE E OF THE STATION IN THE E FENCE LINE

JY1425'OF THE ROAD.  IT IS 33 FEET E OF THE APPROXIMATE CENTER OF THE

JY1425'ROAD AND 3 FEET S OF A STANDARD WITNESS POST.  IT IS A BRONZE AZIMUTH

JY1425'DISK SET IN TOP OF A CONCRETE POST.  IT IS STAMPED BROWN 2 1945 AND

JY1425'IS FLUSH WITH THE GROUND.

JY1425'

JY1425'STATION BROWN 1881 IS 118.31 FEET NE OF THE STATION.  IT IS 15.2 FEET

JY1425'E OF THE E FENCE OF THE CEMETERY, 60.0 FEET NE OF THE SE CORNER

JY1425'OF THE FENCE, AND 41.5 FEET SE OF THE NE CORNER OF THE CEMETERY.  THE

JY1425'MARK IS A 6- BY 6-INCH MARBLE POST 30 INCHES LONG WITH A CROSS ON

JY1425'THE TOP AND USGS ENGRAVED ON THE TOP.  THE MARK PROJECTS ABOUT 8

JY1425'INCHES.

JY1425'

JY1425'TO REACH THE STATION FROM THE JUNCTION OF U.S. HIGHWAY 33 AND STATE

JY1425'HIGHWAY 796 IN ROCKBRIDGE, GO W ON U.S. HIGHWAY 33 FOR 0.5 MILE TO

JY1425'A GRAVEL ROAD LEFT, TURN LEFT AND GO UP STEEP GRADE AND FOLLOW MAIN

JY1425'TRAVELED ROAD FOR 2.10 MILES TO A FORK, KEEP RIGHT FORK AND GO 2.7

JY1425'MILES TO THE AZIMUTH MARK ON THE LEFT, CONTINUE STRAIGHT AHEAD FOR

JY1425'0.4 MILES TO A CROSS ROADS AND A HOUSE ON THE NW CORNER, TURN RIGHT

JY1425'AND GO 0.1 MILE TO THE DRIVE LEADING TO THE BROWN PRIVATE CEMETERY,

JY1425'TURN RIGHT AND GO ABOUT 0.05 MILE TO THE ENTRANCE TO THE CEMETERY

JY1425'AND STATION.

JY1425'

JY1425'HEIGHT OF LIGHT ABOVE STATION MARK - 22 METERS.

JY1425

JY1425                          STATION RECOVERY (1945)

JY1425

JY1425'RECOVERY NOTE BY COAST AND GEODETIC SURVEY 1945

JY1425'RECOVERED IN GOOD CONDITION.

JY1425

JY1425                          STATION RECOVERY (1991)

JY1425

JY1425'RECOVERY NOTE BY WOOLPERT CONSULTANTS 1991

JY1425'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

DH6218 ***********************************************************************

DH6218  DESIGNATION -  CLAYTON

DH6218  PID         -  DH6218

DH6218  STATE/COUNTY-  OH/HOCKING

DH6218  USGS QUAD   -  UNION FURNACE (1995)

DH6218

DH6218                         *CURRENT SURVEY CONTROL

DH6218  ___________________________________________________________________

DH6218* NAD 83(1995)-  39 27 23.89071(N)    277 42 47.73171(E)     ADJUSTED

DH6218* NAVD 88     -       211.4    (meters)     694.     (feet)  GPS OBS

DH6218  ___________________________________________________________________

DH6218  X           -     661,870.844 (meters)                     COMP

DH6218  Y           -  -4,886,785.225 (meters)                     COMP

DH6218  Z           -   4,031,700.096 (meters)                     COMP

DH6218  LAPLACE CORR-          -0.19  (seconds)                    DEFLEC99

DH6218  ELLIP HEIGHT-         177.30  (meters)          (11/14/05) GPS OBS

DH6218  GEOID HEIGHT-         -34.08  (meters)                     GEOID03

DH6218

DH6218  HORZ ORDER  -  FIRST

DH6218  ELLP ORDER  -  FOURTH    CLASS II

DH6218

DH6218.The horizontal coordinates were established by GPS observations

DH6218.and adjusted by the CANTER SURVEYING GPS in November 2005..

DH6218

DH6218.The orthometric height was determined by GPS observations and a

DH6218.high-resolution geoid model.

DH6218

DH6218.The X, Y, and Z were computed from the position and the ellipsoidal ht.

DH6218

DH6218.The Laplace correction was computed from DEFLEC99 derived deflections.

DH6218

DH6218.The ellipsoidal height was determined by GPS observations

DH6218.and is referenced to NAD 83.

DH6218

DH6218.The geoid height was determined by GEOID03.

DH6218

DH6218;                    North         East     Units Scale Factor Converg.

DH6218;SPC OH S     -   161,727.786   618,353.333   MT  0.99993670   +0 08 07.1

DH6218;SPC OH S     -   530,601.91  2,028,714.23   sFT  0.99993670   +0 08 07.1

DH6218;UTM  17      - 4,368,242.039   389,296.735   MT  0.99975089   -0 49 04.1

DH6218

DH6218!             -  Elev Factor  x  Scale Factor =   Combined Factor

DH6218!SPC OH S     -   0.99997218  x   0.99993670  =   0.99990889

DH6218!UTM  17      -   0.99997218  x   0.99975089  =   0.99972308

DH6218

DH6218                          SUPERSEDED SURVEY CONTROL

DH6218

DH6218.No superseded survey control is available for this station.

DH6218

DH6218_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD8929768242(NAD 83)

DH6218_MARKER: DD = SURVEY DISK

DH6218_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

DH6218_STAMPING: CLAYTON

DH6218_PROJECTION: PROJECTING 0 CENTIMETERS

DH6218_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

DH6218+STABILITY: SURFACE MOTION

DH6218_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

DH6218+SATELLITE: SATELLITE OBSERVATIONS - November 11, 2004

DH6218

DH6218  HISTORY     - Date     Condition        Report By

DH6218  HISTORY     - UNK      MONUMENTED

DH6218  HISTORY     - 20041111 GOOD             LOCSUR

DH6218

DH6218                          STATION DESCRIPTION

DH6218

DH6218'DESCRIBED BY LOCAL SURVEYOR (INDIVIDUAL OR FIRM) 2004 (DN)

DH6218'DESCRIBED BY CANTER SURVEYING (2004) THE MARK IS LOCATED IN HOCKING

DH6218'COUNTY, STARR TOWNSHIP ALONG U.S. 33. TO REACH THE MARK FROM THE

DH6218'INTERSECTION OF U.S. 33 AND STATE ROUTE 278 IN THE TOWN OF

DH6218'NELSONVILLE, PRECEED NORTH WEST ALONG U.S. 33 FOR APPROXIMATELY 4.1 KM

DH6218'(2.5 MI) TO A SET  OF CANAL LOCKS AND THE STATION ON THE LEFT. THE

DH6218'MARK IS LOCATED 6.01 M (20 FT) SOUTH OF THE EDGE OF PAVEMENT OF U.S.

DH6218'33, 22.25 M (73 FT)  SOUTH WEST OF A GUARD RAIL, AND 1.67 M ( 5.5 FT)

DH6218'WEST OF A SANDSTONE ABUTMENT. THE STATION IS A TABLET STAMPED CLAYTON

DH6218'SET ATOP A CONCRETE POST .15 M (.5 FT) ABOVE THE  GROUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1331 ***********************************************************************

JY1331  DESIGNATION -  COOK RESET

JY1331  PID         -  JY1331

JY1331  STATE/COUNTY-  OH/HOCKING

JY1331  USGS QUAD   -  GORE (1992)

JY1331

JY1331                         *CURRENT SURVEY CONTROL

JY1331  ___________________________________________________________________

JY1331* NAD 83(1995)-  39 32 29.11214(N)    277 39 18.33616(E)     ADJUSTED

JY1331* NAVD 88     -       325.     (meters)    1066.     (feet)  SCALED

JY1331  ___________________________________________________________________

JY1331  LAPLACE CORR-           0.75  (seconds)                    DEFLEC99

JY1331  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY1331

JY1331  HORZ ORDER  -  FIRST

JY1331

JY1331.The horizontal coordinates were established by classical geodetic methods

JY1331.and adjusted by the National Geodetic Survey in April 1998..

JY1331

JY1331.The orthometric height was scaled from a topographic map.

JY1331

JY1331.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1331

JY1331.The geoid height was determined by GEOID03.

JY1331

JY1331;                    North         East     Units Scale Factor Converg.

JY1331;SPC OH S     -   171,130.094   613,331.379   MT  0.99993965   +0 05 54.3

JY1331;SPC OH S     -   561,449.32  2,012,238.03   sFT  0.99993965   +0 05 54.3

JY1331;UTM  17      - 4,377,724.939   384,432.885   MT  0.99976443   -0 51 22.7

JY1331

JY1331!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1331!SPC OH S     -   0.99995439  x   0.99993965  =   0.99989404

JY1331!UTM  17      -   0.99995439  x   0.99976443  =   0.99971883

JY1331

JY1331|---------------------------------------------------------------------|

JY1331| PID    Reference Object                     Distance      Geod. Az  |

JY1331|                                                           dddmmss.s |

JY1331| CE5904 COOK RM 4                            22.475 METERS 00630     |

JY1331| CE5905 COOK RM 5                            19.753 METERS 09109     |

JY1331| JY1366 BROOKS RESET                        APPROX.11.6 KM 1231923.9 |

JY1331| CE5903 COOK AZ MK                                         2350819.5 |

JY1331|---------------------------------------------------------------------|

JY1331

JY1331                          SUPERSEDED SURVEY CONTROL

JY1331

JY1331  NAD 83(1986)-  39 32 29.11496(N)    277 39 18.33233(E) AD(       ) 1

JY1331  NAD 27      -  39 32 28.87700(N)    277 39 17.88200(E) AD(       ) 1

JY1331

JY1331.Superseded values are not recommended for survey control.

JY1331.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1331.See file dsdata.txt to determine how the superseded data were derived.

JY1331

JY1331_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD8443377725(NAD 83)

JY1331_MARKER: V = STONE MONUMENT

JY1331_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY1331

JY1331  HISTORY     - Date     Condition        Report By

JY1331  HISTORY     - 1945     MONUMENTED       CGS

JY1331  HISTORY     - 1945     SEE DESCRIPTION  CGS

JY1331  HISTORY     - 1970     GOOD             NGS

JY1331

JY1331                          STATION DESCRIPTION

JY1331

JY1331'DESCRIBED BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1331'STATION WAS RECOVERED AS DESCRIBED IN 1881.  THE GRANITE POST WAS

JY1331'RECOVERED AND A STANDARD STATION DISK WAS REPLACED HOLDING THE

JY1331'ORIGINAL POSITION.  THE ORIGINAL STONE REFERENCE MARKS 1, 2, AND 3,

JY1331'WERE NOT RECOVERED.  TWO REFERENCE MARKS 4 AND 5 AND AN AZIMUTH

JY1331'MARK WERE ESTABLISHED BY THIS PARTY.  A COMPLETE DESCRIPTION FOLLOWS-

JY1331'

JY1331'STATION IS LOCATED ABOUT 3.5 MILES E OF THE CITY OF LOGAN ON LAND

JY1331'OWNED BY COOK HEIRS.  THE MARK IS LOCATED ON A HIGH SCRUB APPLE

JY1331'TREE HILL ABOUT 0.15 MILE NW OF AN OLD HOUSE.  IT IS 4.5 FEET S OF A

JY1331'STANDARD WITNESS POST.  THE DISK IS STAMPED COOK 1881 1945 AND

JY1331'PROJECTS ABOUT 4 INCHES.

JY1331'

JY1331'SURFACE, UNDERGROUND, REFERENCE, AND AZIMUTH MARKS ARE STANDARD DISKS

JY1331'SET IN CONCRETE.

JY1331'

JY1331'REFERENCE MARK 4 IS 73.80 FEET N OF THE STATION ON THE N SLOPE OF THE

JY1331'HILL.  THE DISK IS STAMPED COOK NO 1 1945 AND PROJECTS ABOUT 4

JY1331'INCHES.

JY1331'

JY1331'REFERENCE MARK 5 IS 65.14 FEET E OF THE STATION ON THE E SLOPE OF THE

JY1331'HILL.  THE DISK IS STAMPED COOK NO 5 1945 AND PROJECTS ABOUT 4

JY1331'INCHES.

JY1331'

JY1331'AZIMUTH MARK IS ABOUT 0.5 MILE SW OF THE STATION ON THE N SIDE OF A

JY1331'DIRT ROAD.  IT IS 2 FEET E OF A STANDARD WITNESS POST, 12 FEET N

JY1331'OF THE APPROXIMATE CENTER OF THE ROAD, AND 1 FOOT S OF THE FENCE.

JY1331'THE DISK IS STAMPED COOK 1945 AND PROJECTS ABOUT 6 INCHES.

JY1331'

JY1331'TO REACH THE STATION FROM THE JUNCTION OF U.S.HIGHWAY 33 AND STATE

JY1331'HIGHWAY 75 IN THE CITY OF LOGAN, GO SE ON U.S.  HIGHWAY 33 FOR

JY1331'2.4 MILES TO STATE HIGHWAY 328 RIGHT, CONTINUE ON HIGHWAY 33 FOR 0.35

JY1331'MILE TO A T-ROAD LEFT, TURN LEFT 0.1 MILE TO A FORK.  TAKE LEFT

JY1331'FORK AND GO 0.9 MILE TO ANOTHER FORK, TURN RIGHT AND GO 0.2 MILE TO

JY1331'THE AZIMUTH MARK ON THE LEFT, CONTINUE STRAIGHT AHEAD FOR 0.8

JY1331'MILE TO A DIRT T-ROAD LEFT, TURN LEFT AND GO 0.4 MILE TO AN OLD HOUSE

JY1331'ON THE RIGHT AND END OF TRUCK TRAVEL.  CONTINUE STRAIGHT AHEAD

JY1331'ON A DIM TRACK ROAD FOR 0.2 MILE TO THE TOP OF THE GRADE, TURN

JY1331'LEFT AND FOLLOW TRACK ROAD, PASSING ANOTHER OLD HOUSE ON THE RIGHT,

JY1331'AND GO ABOUT 0.3 MILE TO THE HIGHEST POINT AND STATION.

JY1331'

JY1331'HEIGHT OF LIGHT ABOVE STATION MARK - 26 METERS.

JY1331

JY1331                          STATION RECOVERY (1970)

JY1331

JY1331'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1970 (LFS)

JY1331'THE STATION MARK, REFERENCE MARK NO. 4 AND REFERENCE MARK NO. 5 WERE

JY1331'RECOVERED AND FOUND IN GOOD CONDITION.  A THOROUGH AND EXTENDED

JY1331'SEARCH WAS MADE FOR THE AZIMUTH MARK SET IN 1945, BUT IT COULD NOT

JY1331'BE FOUND, AND IS THOUGHT TO HAVE BEEN DESTROYED BY GRADING OF OIL

JY1331'FIELD ROADS NOW IN THE AREA.  IN CHECKING THE HORIZONTAL MEASURED

JY1331'DISTANCE BETWEEN THE STATION AND REFERENCE MARK NO. 4 A

JY1331'DISCREPANCY OF - 0.07 OF A FOOT WAS NOTED AND BETWEEN THE STATION

JY1331'AND REFERENCE MARK NO. 5 A DISCREPANCY OF - 0.34 OF A FOOT WAS

JY1331'NOTED.  THE STATION WAS OCCUPIED AND THE HORIZONTAL ANGLE OBSERVED

JY1331'BETWEEN REFERENCE MARK NO. 4 AND REFERENCE MARK NO. 5.  WHEN

JY1331'THE ANGLE OBSERVED THIS DATE WAS CHECKED AGAINST THE 1945

JY1331'OBSERVED VALUE, A DISCREPANCY OF + 01 FOOT 20.8 MIN WAS NOTED.

JY1331'

JY1331'FOLLOWING IS A NEW AND COMPLETE DESCRIPTION.

JY1331'

JY1331'THE STATION IS LOCATED ON THE SUMMIT OF A HEAVILY WOODED HILL ABOUT

JY1331'3.5 MILES EAST OF LOGAN, IN THE MIDST OF AN OIL FIELD, AND ON THE

JY1331'PROPERTY OF THE COOK FAMILY HEIRS.

JY1331'

JY1331'TO REACH THE STATION FROM THE NORTHWEST CORNER OF THE HOCKING

JY1331'COUNTY COURTHOUSE IN LOGAN, GO EAST ON MAIN STREET FOR 0.1 OF A MILE

JY1331'TO THE JUNCTION OF OLD U.S. HIGHWAY 33 AND STATE ROUTE 93.  HERE

JY1331'CONTINUE EAST ON OLD U.S. HIGHWAY 33 FOR 0.2 OF A MILE TO WHERE

JY1331'U.S. HIGHWAY 33 TURNS RIGHT.  HERE TURN RIGHT AND GO SOUTH ON

JY1331'U.S.  HIGHWAY 33 FOR 0.1 OF A MILE TO WHERE OLD U.S. HIGHWAY 33

JY1331'TURNS LEFT AND U.S. HIGHWAY 33B CONTINUES STRAIGHT AHEAD TO

JY1331'NEW U.S.  HIGHWAY 33.  HERE TURN LEFT ON OLD U.S. HIGHWAY 33

JY1331'AND GO SOUTHEASTERLY FOR 2.0 MILES TO STATE ROUTE 328 ON

JY1331'THE RIGHT, CONTINUE SOUTHEASTERLY ON OLD U.S. HIGHWAY 33

JY1331'FOR ABOUT 0.05 TO HOCKING COUNTY ROAD 17 (MARYVILLE-WILLIAMS

JY1331'ROAD) ON THE LEFT.  HERE TURN LEFT ON HOCKING COUNTY ROAD 17 AND

JY1331'GO NORTHEASTERLY FOR 0.1 OF A MILE TO GREEN TOWNSHIP ROAD 362

JY1331'RIGHT.  CONTINUE NORTHEAST, NORTH, AND NORTHEASTERLY ON HOCKING

JY1331'COUNTY ROAD 17 FOR 0.85 OF A MILE TO GREEN TOWNSHIP ROAD 350

JY1331'(DAVIS-GREENHILL ROAD) ON THE RIGHT.  HERE TURN RIGHT ON GREEN

JY1331'TOWNSHIP ROAD 350 AND GO EASTERLY FOR 1.0 MILE TO MILE TO A CINDER

JY1331'ROAD (GREEN TOWNSHIP ROAD 357 - ROBY ROAD) ON THE LEFT AND A

JY1331'HOUSE IN THE NORTHWEST ANGLE OF THE T INTERSECTION.  HERE TURN LEFT

JY1331'AND GO NORTHERLY ON THIS CINDER ROAD FOR 0.6 OF A MILE TO THE

JY1331'CREST OF A STEEP HILL AND A FORK.  KEEP LEFT FORK ON A CINDER ROAD

JY1331'KEEPING ALL LEFT FORKS, AND GO WEST FOR 0.2 OF A MILE TO AN OILWELL

JY1331'PUMPING STATION AT END OF ROAD AND END OF TRUCK TRAVEL.  HERE PACK

JY1331'NORTHERLY UP STEEP WOODED HILL FOR ABOUT 400 FEET TO THE STATION IN

JY1331'WOODS AS DESCRIBED.

JY1331'

JY1331'THE STATION IS LOCATED IN DENSE WOODS, 20 FEET NORTH-NORTHEAST OF A

JY1331'9-INCH, TWIN TRUNKED HAWTHORN TREE, 17 FEET SOUTH-SOUTHWEST OF A

JY1331'9-INCH HAWTHORN TREE, 12 FEET EAST-SOUTHEAST OF AN 8-INCH HICKORY

JY1331'TREE, AND 1.0 FOOT EAST OF A METAL WITNESS POST AND SIGN.  IT IS A

JY1331'STANDARD DISK SET IN THE TOP OF A 12-INCH, SQUARE, FLUSH,

JY1331'CONCRETE MONUMENT AND THE DISK IS STAMPED COOK 1881 1945.

JY1331'

JY1331'REFERENCE MARK NO. 4 IS LOCATED ON THE NORTH SLOPE OF THE WOODED

JY1331'HILL ABOUT 2 FEET BELOW THE STATION ELEVATION, 74 FEET NORTH OF THE

JY1331'METAL WITNESS POST AND SIGN, 26 FEET EAST OF AN 8-INCH DOGWOOD

JY1331'TREE, 13 FEET WEST OF AN 8-INCH TULIP POPLAR TREE, AND 11 FEET

JY1331'NORTHEAST OF A 12-INCH, LEANING, TULIP POPLAR TREE.  IT IS A

JY1331'STANDARD DISK SET IN THE TOP OF A 12-INCH, SQUARE, CONCRETE

JY1331'MONUMENT THAT PROJECTS 3 INCHES AND THE DISK IS STAMPED COOK NO 4

JY1331'1945.

JY1331'

JY1331'REFERENCE MARK NO. 5 IS LOCATED ON THE EAST SLOPE OF THE WOODED HILL

JY1331'ABOUT 4 FEET BELOW THE STATION ELEVATION, 66 FEET EAST OF THE METAL

JY1331'WITNESS POST AND SIGN, 19 FEET WEST-SOUTHWEST OF A 14-INCH TULIP

JY1331'POPLAR TREE, 15 FEET SOUTHEAST OF A 12-INCH TULIP POPLAR TREE, AND

JY1331'2 FEET SOUTH OF A 5-INCH DOGWOOD TREE.  IT IS A STANDARD DISK SET

JY1331'IN THE TOP OF A 12-INCH, SQUARE, CONCRETE MONUMENT THAT PROJECTS 5

JY1331'INCHES AND THE DISK IS STAMPED COOK NO 5 1945.

JY1331'

JY1331'THE HORIZONTAL MEASURED DISTANCE BETWEEN REFERENCE MARK NO. 4 AND

JY1331'REFERENCE MARK NO. 5 IS 93.50 FEET - 28.500 METERS.

JY1331'

JY1331'AIRLINE DISTANCE AND DIRECTION FROM NEAREST TOWN--ABOUT 3.5 MILES

JY1331'EAST OF LOGAN.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0093 ***********************************************************************

JY0093  DESIGNATION -  GAGING STATION

JY0093  PID         -  JY0093

JY0093  STATE/COUNTY-  OH/HOCKING

JY0093  USGS QUAD   -  LOGAN (1992)

JY0093

JY0093                         *CURRENT SURVEY CONTROL

JY0093  ___________________________________________________________________

JY0093* NAD 83(1986)-  39 33 54.     (N)    277 31 30      (E)     SCALED

JY0093* NAVD 88     -       227.006  (meters)     744.77   (feet)  ADJUSTED

JY0093  ___________________________________________________________________

JY0093  GEOID HEIGHT-         -34.15  (meters)                     GEOID03

JY0093  DYNAMIC HT  -         226.871 (meters)     744.33  (feet)  COMP

JY0093  MODELED GRAV-     980,026.9   (mgal)                       NAVD 88

JY0093

JY0093  VERT ORDER  -  SECOND    CLASS 0

JY0093

JY0093.The horizontal coordinates were scaled from a topographic map and have

JY0093.an estimated accuracy of +/- 6 seconds.

JY0093

JY0093.The orthometric height was determined by differential leveling

JY0093.and adjusted by the National Geodetic Survey in June 1991..

JY0093

JY0093.The geoid height was determined by GEOID03.

JY0093

JY0093.The dynamic height is computed by dividing the NAVD 88

JY0093.geopotential number by the normal gravity value computed on the

JY0093.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0093.degrees latitude (g = 980.6199 gals.).

JY0093

JY0093.The modeled gravity was interpolated from observed gravity values.

JY0093

JY0093;                    North         East    Units  Estimated Accuracy

JY0093;SPC OH S     -   173,740.      602,150.      MT  (+/- 180 meters Scaled)

JY0093

JY0093                          SUPERSEDED SURVEY CONTROL

JY0093

JY0093  NGVD 29 (??/??/92)  227.130  (m)          745.18   (f) ADJ UNCH    2 0

JY0093

JY0093.Superseded values are not recommended for survey control.

JY0093.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0093.See file dsdata.txt to determine how the superseded data were derived.

JY0093

JY0093_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD732805(NAD 83)

JY0093_MARKER: DD = SURVEY DISK

JY0093_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0093_SP_SET: BRIDGE

JY0093_STAMPING: GAGING STATION

JY0093_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0093

JY0093  HISTORY     - Date     Condition        Report By

JY0093  HISTORY     - UNK      MONUMENTED       USGS

JY0093  HISTORY     - 1936     GOOD             NGS

JY0093  HISTORY     - 20050617 MARK NOT FOUND   GEOCAC

JY0093

JY0093                          STATION DESCRIPTION

JY0093

JY0093'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0093'AT ENTERPRISE.

JY0093'AT ENTERPRISE, HOCKING COUNTY, ON OLD STATE HIGHWAY 31, 190 YARDS

JY0093'EAST OF THE NORTHEAST CORNER OF THE CHESAPEAKE AND OHIO RAILWAY

JY0093'STATION, 21 FEET EAST OF A HACKBERRY TREE, AT THE WEST END OF THE

JY0093'BRIDGE OVER HOCKING RIVER, IN THE TOP OF THE NORTHWEST WING WALL,

JY0093'12 FEET EAST OF THE WEST END, 10 FEET SOUTH OF THE GAGING-STATION

JY0093'HOUSE, 8 FEET NORTH OF THE CENTERLINE OF THE HIGHWAY, AND LEVEL

JY0093'WITH THE HIGHWAY.  A UNITED STATES GEOLOGICAL SURVEY STANDARD

JY0093'DISK, STAMPED GAGING STATION.

JY0093

JY0093                          STATION RECOVERY (2005)

JY0093

JY0093'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0093'THE BRIDGE OVER THE HOCKING RIVER HAS BEEN REPLACED.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0092 ***********************************************************************

JY0092  DESIGNATION -  H 69

JY0092  PID         -  JY0092

JY0092  STATE/COUNTY-  OH/HOCKING

JY0092  USGS QUAD   -  LOGAN (1992)

JY0092

JY0092                         *CURRENT SURVEY CONTROL

JY0092  ___________________________________________________________________

JY0092* NAD 83(1986)-  39 33 32.     (N)    277 31 51      (E)     SCALED

JY0092* NAVD 88     -       227.982  (meters)     747.97   (feet)  ADJUSTED

JY0092  ___________________________________________________________________

JY0092  GEOID HEIGHT-         -34.14  (meters)                     GEOID03

JY0092  DYNAMIC HT  -         227.847 (meters)     747.53  (feet)  COMP

JY0092  MODELED GRAV-     980,026.9   (mgal)                       NAVD 88

JY0092

JY0092  VERT ORDER  -  SECOND    CLASS 0

JY0092

JY0092.The horizontal coordinates were scaled from a topographic map and have

JY0092.an estimated accuracy of +/- 6 seconds.

JY0092

JY0092.The orthometric height was determined by differential leveling

JY0092.and adjusted by the National Geodetic Survey in June 1991..

JY0092.WARNING-Repeat measurements at this control monument indicate possible

JY0092.vertical movement.

JY0092

JY0092.The geoid height was determined by GEOID03.

JY0092

JY0092.The dynamic height is computed by dividing the NAVD 88

JY0092.geopotential number by the normal gravity value computed on the

JY0092.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0092.degrees latitude (g = 980.6199 gals.).

JY0092

JY0092.The modeled gravity was interpolated from observed gravity values.

JY0092

JY0092;                    North         East    Units  Estimated Accuracy

JY0092;SPC OH S     -   173,060.      602,650.      MT  (+/- 180 meters Scaled)

JY0092

JY0092                          SUPERSEDED SURVEY CONTROL

JY0092

JY0092  NGVD 29 (??/??/92)  228.106  (m)          748.38   (f) ADJ UNCH    2 0

JY0092

JY0092.Superseded values are not recommended for survey control.

JY0092.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0092.See file dsdata.txt to determine how the superseded data were derived.

JY0092

JY0092_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD737798(NAD 83)

JY0092_MARKER: DB = BENCH MARK DISK

JY0092_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0092_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0092_STAMPING: H 69 1934

JY0092_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0092+STABILITY: SURFACE MOTION

JY0092

JY0092  HISTORY     - Date     Condition        Report By

JY0092  HISTORY     - 1934     MONUMENTED       CGS

JY0092  HISTORY     - 1936     GOOD             NGS

JY0092  HISTORY     - 19881220 MARK NOT FOUND   USPSQD

JY0092

JY0092                          STATION DESCRIPTION

JY0092

JY0092'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0092'0.5 MI E FROM ENTERPRISE.

JY0092'0.5 MILE EAST ALONG OLD STATE HIGHWAY 31 FROM THE CHESAPEAKE AND

JY0092'OHIO RAILWAY STATION AT ENTERPRISE, HOCKING COUNTY, 56 YARDS WEST

JY0092'OF A BRIDGE, 46 YARDS SOUTH OF THE SOUTHEAST CORNER OF A WHITE

JY0092'FARMHOUSE, 18 YARDS WEST OF THE SOUTHWEST CORNER OF A PRIVATE

JY0092'GARAGE, 35.5 FEET NORTH OF THE CENTERLINE OF THE HIGHWAY, 3 FEET

JY0092'EAST OF A POLE, AND ABOUT 6 INCHES ABOVE THE LEVEL OF THE HIGHWAY.

JY0092'A STANDARD DISK, STAMPED H 69 1934 AND SET IN THE TOP OF A CONCRETE

JY0092'POST PROJECTING 5 INCHES ABOVE GROUND.

JY0092

JY0092                          STATION RECOVERY (1988)

JY0092

JY0092'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0092'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

AB6092 ***********************************************************************

AB6092  CBN         -  This is a Cooperative Base Network Control Station.

AB6092  DESIGNATION -  HOC GAR

AB6092  PID         -  AB6092

AB6092  STATE/COUNTY-  OH/HOCKING

AB6092  USGS QUAD   -  LOGAN (1992)

AB6092

AB6092                         *CURRENT SURVEY CONTROL

AB6092  ___________________________________________________________________

AB6092* NAD 83(1995)-  39 32 10.87857(N)    277 33 21.79385(E)     ADJUSTED

AB6092* NAVD 88     -       235.1    (meters)     771.     (feet)  GPS OBS

AB6092  ___________________________________________________________________

AB6092  X           -     647,722.425 (meters)                     COMP

AB6092  Y           -  -4,883,020.321 (meters)                     COMP

AB6092  Z           -   4,038,545.211 (meters)                     COMP

AB6092  LAPLACE CORR-           0.16  (seconds)                    DEFLEC99

AB6092  ELLIP HEIGHT-         201.02  (meters)          (03/08/05) GPS OBS

AB6092  GEOID HEIGHT-         -34.13  (meters)                     GEOID03

AB6092

AB6092  HORZ ORDER  -  B

AB6092  ELLP ORDER  -  FOURTH    CLASS II

AB6092

AB6092.The horizontal coordinates were established by GPS observations

AB6092.and adjusted by the National Geodetic Survey in August 1996..

AB6092

AB6092.The orthometric height was determined by GPS observations and a

AB6092.high-resolution geoid model.

AB6092

AB6092.The X, Y, and Z were computed from the position and the ellipsoidal ht.

AB6092

AB6092.The Laplace correction was computed from DEFLEC99 derived deflections.

AB6092

AB6092.The ellipsoidal height was determined by GPS observations

AB6092.and is referenced to NAD 83.

AB6092

AB6092.The geoid height was determined by GEOID03.

AB6092

AB6092;                    North         East     Units Scale Factor Converg.

AB6092;SPC OH S     -   170,557.843   604,818.578   MT  0.99993941   +0 02 08.0

AB6092;SPC OH S     -   559,571.86  1,984,308.95   sFT  0.99993941   +0 02 08.0

AB6092;UTM  17      - 4,377,294.689   375,913.116   MT  0.99978957   -0 55 09.4

AB6092

AB6092!             -  Elev Factor  x  Scale Factor =   Combined Factor

AB6092!SPC OH S     -   0.99996846  x   0.99993941  =   0.99990788

AB6092!UTM  17      -   0.99996846  x   0.99978957  =   0.99975804

AB6092

AB6092                          SUPERSEDED SURVEY CONTROL

AB6092

AB6092  ELLIP H (08/20/96)  201.03   (m)                       GP(       ) 4 2

AB6092

AB6092.Superseded values are not recommended for survey control.

AB6092.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

AB6092.See file dsdata.txt to determine how the superseded data were derived.

AB6092

AB6092_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7591377295(NAD 83)

AB6092_MARKER: I = METAL ROD

AB6092_SETTING: 59 = STAINLESS STEEL ROD IN SLEEVE (10 FT.+)

AB6092_STAMPING: HOC GAR 1995

AB6092_MARK LOGO: OHDT

AB6092_PROJECTION: FLUSH

AB6092_MAGNETIC: N = NO MAGNETIC MATERIAL

AB6092_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

AB6092_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

AB6092+SATELLITE: SATELLITE OBSERVATIONS - June 18, 2005

AB6092_ROD/PIPE-DEPTH: 8.5  meters

AB6092_SLEEVE-DEPTH  : 0.91 meters

AB6092

AB6092  HISTORY     - Date     Condition        Report By

AB6092  HISTORY     - 1995     MONUMENTED       OHDT

AB6092  HISTORY     - 20050618 GOOD             GEOCAC

AB6092

AB6092                          STATION DESCRIPTION

AB6092

AB6092'DESCRIBED BY OHIO DEPARTMENT OF TRANSPORTATION 1995 (CWP)

AB6092'THE MARK IS LOCATED AT THE HOCKING COUNTY, OHIO DEPARTMENT OF

AB6092'TRANSPORTATION GARAGE, APPROXIMATELY 0.23 KM (0.15 MI) SOUTH OF

AB6092'OVERPASS OF US HIGHWAY 33 ON STATE ROUTE 664.  THE MARK IS TO THE LEFT

AB6092'OF ENTRANCE DRIVE TO GARAGE AND DIRECTLY ACROSS FROM SECOND DRIVE TO

AB6092'PARKING LOT FOR HOCKING COUNTY VISITORS CENTER.  TO REACH THE STATION

AB6092'FROM US HIGHWAY 33 TAKE EXIT STATE ROUTE 664 SOUTH, FOR 0.23 KM (0.15

AB6092'MI) TO THE ENTRANCE ROAD TO GARAGE SITE AND THE HOCKING COUNTY

AB6092'VISITORS CENTER.  CONTINUE ON ENTRANCE ROAD TO SECOND ENTRANCE TO

AB6092'VISITORS CENTER PARKING LOT.  ROD MARK IS DIRECTLY NORTH OF DRIVE.

AB6092'THE MARK IS 11.50 M (37.73 FT) NORTH OF NORTH SIDE OF PAVED DRIVE,

AB6092'31.70 M (104.00 FT) EAST OF LIGHT POLE AND 26.60 M (87.27 FT) WEST OF

AB6092'LIGHT POLE.  THE MARK IS A STAINLESS STEEL ROD CLASS B MONUMENT FLUSH

AB6092'WITH THE GROUND AND DRIVEN TO A DEPTH OF 8.5 M (27.9 FT) , ROD MARK IS

AB6092'STAMPED --HOC GAR 1995--.  FIBERGLASS WITNESS POST IS 1.0 M (3.3 FT)

AB6092'WEST OF MARK.  OWNERSHIP--STATE OF OHIO, DEPARTMENT OF TRANSPORTATION,

AB6092'DISTRICT 10 DEPUTY DIRECTOR, MUSKINGUM DRIVE, MARIETTA, OHIO 45750.

AB6092'THE MARK IS ACESSIBLE AT ALL TIMES. NOTE--SLEEVE DEPTH DOES NOT MEET

AB6092'CLASS A REQUIREMENTS.

AB6092

AB6092                          STATION RECOVERY (2005)

AB6092

AB6092'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

AB6092'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

DI1856 ***********************************************************************

DI1856  CORS        -  This is a GPS Continuously Operating Reference Station.

DI1856  DESIGNATION -  HOCKING COUNTY CORS ARP

DI1856  CORS_ID     -  OHHO

DI1856  PID         -  DI1856

DI1856  STATE/COUNTY-  OH/HOCKING

DI1856  USGS QUAD   -  LOGAN (1992)

DI1856

DI1856                         *CURRENT SURVEY CONTROL

DI1856  ___________________________________________________________________

DI1856* NAD 83(CORS)-  39 32 07.27648(N)    277 33 22.12341(E)     ADJUSTED

DI1856* NAVD 88     -

DI1856  ___________________________________________________________________

DI1856  EPOCH DATE  -        2002.00

DI1856  X           -     647,739.947 (meters)                     COMP

DI1856  Y           -  -4,883,092.563 (meters)                     COMP

DI1856  Z           -   4,038,462.174 (meters)                     COMP

DI1856  ELLIP HEIGHT-         205.17  (meters)          (10/??/06) GPS OBS

DI1856  GEOID HEIGHT-         -34.13  (meters)                     GEOID03

DI1856

DI1856  HORZ ORDER  -  SPECIAL (CORS)

DI1856  ELLP ORDER  -  SPECIAL (CORS)

DI1856

DI1856.ITRF positions are available for this station.

DI1856.The coordinates were established by GPS observations

DI1856.and adjusted by the National Geodetic Survey in October 2006..

DI1856.The coordinates are valid at the epoch date displayed above.

DI1856.The epoch date for horizontal control is a decimal equivalence

DI1856.of Year/Month/Day.

DI1856

DI1856

DI1856.The PID for the CORS L1 Phase Center is DI1857.

DI1856

DI1856.The XYZ, and position/ellipsoidal ht. are equivalent.

DI1856

DI1856.The ellipsoidal height was determined by GPS observations

DI1856.and is referenced to NAD 83.

DI1856

DI1856.The geoid height was determined by GEOID03.

DI1856

DI1856;                    North         East     Units Scale Factor Converg.

DI1856;SPC OH S     -   170,446.764   604,826.517   MT  0.99993937   +0 02 08.3

DI1856;SPC OH S     -   559,207.42  1,984,335.00   sFT  0.99993937   +0 02 08.3

DI1856

DI1856!             -  Elev Factor  x  Scale Factor =   Combined Factor

DI1856!SPC OH S     -   0.99996781  x   0.99993937  =   0.99990718

DI1856

DI1856                          SUPERSEDED SURVEY CONTROL

DI1856

DI1856.No superseded survey control is available for this station.

DI1856

DI1856_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7591977184(NAD 83)

DI1856_MARKER: STATION IS THE ANTENNA REFERENCE POINT OF THE GPS ANTENNA

DI1856

DI1856                          STATION DESCRIPTION

DI1856

DI1856'DESCRIBED BY NATIONAL GEODETIC SURVEY 2006

DI1856'STATION IS A GPS CORS.  LATEST INFORMATION INCLUDING POSITIONS AND

DI1856'VELOCITIES ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

DI1856'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

DI1856'   FTP CORS.NGS.NOAA.GOV: CORS/COORD  AND  CORS/STATION_LOG

DI1856'   HTTP://WWW.NGS.NOAA.GOV  UNDER PRODUCTS AND SERVICES.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

DI1857 ***********************************************************************

DI1857  CORS        -  This is a GPS Continuously Operating Reference Station.

DI1857  DESIGNATION -  HOCKING COUNTY CORS L1 PHASE CENTER

DI1857  CORS_ID     -  OHHO

DI1857  PID         -  DI1857

DI1857  STATE/COUNTY-  OH/HOCKING

DI1857  USGS QUAD   -  LOGAN (1992)

DI1857

DI1857                         *CURRENT SURVEY CONTROL

DI1857  ___________________________________________________________________

DI1857* NAD 83(CORS)-  39 32 07.27649(N)    277 33 22.12343(E)     ADJUSTED

DI1857* NAVD 88     -

DI1857  ___________________________________________________________________

DI1857  EPOCH DATE  -        2002.00

DI1857  X           -     647,739.956 (meters)                     COMP

DI1857  Y           -  -4,883,092.624 (meters)                     COMP

DI1857  Z           -   4,038,462.225 (meters)                     COMP

DI1857  ELLIP HEIGHT-         205.25  (meters)          (10/??/06) GPS OBS

DI1857  GEOID HEIGHT-         -34.13  (meters)                     GEOID03

DI1857

DI1857  HORZ ORDER  -  SPECIAL (CORS)

DI1857  ELLP ORDER  -  SPECIAL (CORS)

DI1857

DI1857.ITRF positions are available for this station.

DI1857.The coordinates were established by GPS observations

DI1857.and adjusted by the National Geodetic Survey in October 2006..

DI1857.The coordinates are valid at the epoch date displayed above.

DI1857.The epoch date for horizontal control is a decimal equivalence

DI1857.of Year/Month/Day.

DI1857

DI1857

DI1857.The PID for the CORS ARP is DI1856.

DI1857

DI1857.The XYZ, and position/ellipsoidal ht. are equivalent.

DI1857

DI1857.The ellipsoidal height was determined by GPS observations

DI1857.and is referenced to NAD 83.

DI1857

DI1857.The geoid height was determined by GEOID03.

DI1857

DI1857;                    North         East     Units Scale Factor Converg.

DI1857;SPC OH S     -   170,446.765   604,826.517   MT  0.99993937   +0 02 08.3

DI1857;SPC OH S     -   559,207.43  1,984,335.00   sFT  0.99993937   +0 02 08.3

DI1857

DI1857!             -  Elev Factor  x  Scale Factor =   Combined Factor

DI1857!SPC OH S     -   0.99996780  x   0.99993937  =   0.99990717

DI1857

DI1857                          SUPERSEDED SURVEY CONTROL

DI1857

DI1857.No superseded survey control is available for this station.

DI1857

DI1857_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7591977184(NAD 83)

DI1857_MARKER: STATION IS THE L1 PHASE CENTER OF THE GPS ANTENNA

DI1857

DI1857                          STATION DESCRIPTION

DI1857

DI1857'DESCRIBED BY NATIONAL GEODETIC SURVEY

DI1857'STATION IS A GPS CORS.  LATEST INFORMATION INCLUDING POSITIONS AND

DI1857'VELOCITIES ARE AVAILABLE IN THE COORDINATE AND LOG FILES ACCESSIBLE

DI1857'BY ANONYMOUS FTP OR THE WORLDWIDE WEB.

DI1857'   FTP CORS.NGS.NOAA.GOV: CORS/COORD  AND  CORS/STATION_LOG

DI1857'   HTTP://WWW.NGS.NOAA.GOV  UNDER PRODUCTS AND SERVICES.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0095 ***********************************************************************

JY0095  DESIGNATION -  J 69

JY0095  PID         -  JY0095

JY0095  STATE/COUNTY-  OH/HOCKING

JY0095  USGS QUAD   -  LOGAN (1992)

JY0095

JY0095                         *CURRENT SURVEY CONTROL

JY0095  ___________________________________________________________________

JY0095* NAD 83(1986)-  39 33 34.     (N)    277 30 06      (E)     SCALED

JY0095* NAVD 88     -       242.276  (meters)     794.87   (feet)  ADJUSTED

JY0095  ___________________________________________________________________

JY0095  GEOID HEIGHT-         -34.14  (meters)                     GEOID03

JY0095  DYNAMIC HT  -         242.131 (meters)     794.39  (feet)  COMP

JY0095  MODELED GRAV-     980,023.5   (mgal)                       NAVD 88

JY0095

JY0095  VERT ORDER  -  SECOND    CLASS 0

JY0095

JY0095.The horizontal coordinates were scaled from a topographic map and have

JY0095.an estimated accuracy of +/- 6 seconds.

JY0095

JY0095.The orthometric height was determined by differential leveling

JY0095.and adjusted by the National Geodetic Survey in June 1991..

JY0095

JY0095.The geoid height was determined by GEOID03.

JY0095

JY0095.The dynamic height is computed by dividing the NAVD 88

JY0095.geopotential number by the normal gravity value computed on the

JY0095.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0095.degrees latitude (g = 980.6199 gals.).

JY0095

JY0095.The modeled gravity was interpolated from observed gravity values.

JY0095

JY0095;                    North         East    Units  Estimated Accuracy

JY0095;SPC OH S     -   173,120.      600,140.      MT  (+/- 180 meters Scaled)

JY0095

JY0095                          SUPERSEDED SURVEY CONTROL

JY0095

JY0095  NGVD 29 (??/??/92)  242.400  (m)          795.27   (f) ADJ UNCH    2 0

JY0095

JY0095.Superseded values are not recommended for survey control.

JY0095.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0095.See file dsdata.txt to determine how the superseded data were derived.

JY0095

JY0095_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD712799(NAD 83)

JY0095_MARKER: DB = BENCH MARK DISK

JY0095_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0095_SP_SET: BRIDGE

JY0095_STAMPING: J 69 1934

JY0095_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0095

JY0095  HISTORY     - Date     Condition        Report By

JY0095  HISTORY     - 1934     MONUMENTED       CGS

JY0095  HISTORY     - 1951     GOOD             NGS

JY0095  HISTORY     - 1960     MARK NOT FOUND   USGS

JY0095

JY0095                          STATION DESCRIPTION

JY0095

JY0095'DESCRIBED BY NATIONAL GEODETIC SURVEY 1951

JY0095'0.8 MI W FROM ENTERPRISE.

JY0095'0.8 MILE WEST ALONG OLD STATE HIGHWAY 31 FROM THE CHESAPEAKE AND

JY0095'OHIO RAILWAY STATION AT ENTERPRISE, HOCKING COUNTY, 265 YARDS

JY0095'WEST OF THE CENTER OF THE JUNCTION OF STATE HIGHWAY 180, 220

JY0095'YARDS WEST OF THE NORTHWEST CORNER OF A GULF SERVICE STATION, 130

JY0095'YARDS EAST OF THE JUNCTION OF A GRAVEL ROAD LEADING SOUTH, IN

JY0095'THE TOP OF THE SOUTH WALL OF A CONCRETE BRIDGE, 3.5 FEET WEST OF

JY0095'THE NORTHEAST CORNER OF THE WALL, 4 YARDS SOUTH OF THE CETERLINE

JY0095'OF THE HIGHWAY, AND ABOUT ONE FOOT ABOVE THE LEVEL OF THE HIGHWAY.

JY0095'A STANDARD DISK, STAMPED J 69 1934.  NOTE-- THE CONCRETE AROUND IT

JY0095'IS BEGINNING TO DISINTEGRATE.

JY0095

JY0095                          STATION RECOVERY (1960)

JY0095

JY0095'RECOVERY NOTE BY US GEOLOGICAL SURVEY 1960

JY0095'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0841 ***********************************************************************

JY0841  DESIGNATION -  K 69 RESET

JY0841  PID         -  JY0841

JY0841  STATE/COUNTY-  OH/HOCKING

JY0841  USGS QUAD   -  LOGAN (1992)

JY0841

JY0841                         *CURRENT SURVEY CONTROL

JY0841  ___________________________________________________________________

JY0841* NAD 83(1995)-  39 33 30.76673(N)    277 30 11.54849(E)     ADJUSTED

JY0841* NAVD 88     -       238.42   (+/-2cm)     782.2    (feet)  VERTCON

JY0841  ___________________________________________________________________

JY0841  X           -     643,013.838 (meters)                     COMP

JY0841  Y           -  -4,882,062.781 (meters)                     COMP

JY0841  Z           -   4,040,447.160 (meters)                     COMP

JY0841  LAPLACE CORR-          -0.38  (seconds)                    DEFLEC99

JY0841  ELLIP HEIGHT-         204.26  (meters)          (10/07/05) GPS OBS

JY0841  GEOID HEIGHT-         -34.14  (meters)                     GEOID03

JY0841

JY0841  HORZ ORDER  -  FIRST

JY0841  VERT ORDER  -  THIRD (See Below)

JY0841  ELLP ORDER  -  FOURTH    CLASS I

JY0841

JY0841.The horizontal coordinates were established by GPS observations

JY0841.and adjusted by the National Geodetic Survey in April 1998..

JY0841

JY0841.The NAVD 88 height was computed by applying the VERTCON shift value to

JY0841.the NGVD 29 height (displayed under SUPERSEDED SURVEY CONTROL.)

JY0841.The vertical order pertains to the NGVD 29 superseded value.

JY0841

JY0841.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JY0841

JY0841.The Laplace correction was computed from DEFLEC99 derived deflections.

JY0841

JY0841.The ellipsoidal height was determined by GPS observations

JY0841.and is referenced to NAD 83.

JY0841

JY0841.The geoid height was determined by GEOID03.

JY0841

JY0841;                    North         East     Units Scale Factor Converg.

JY0841;SPC OH S     -   173,019.998   600,275.676   MT  0.99994051   +0 00 07.3

JY0841;SPC OH S     -   567,649.78  1,969,404.45   sFT  0.99994051   +0 00 07.3

JY0841;UTM  17      - 4,379,831.866   371,412.533   MT  0.99980357   -0 57 12.2

JY0841

JY0841!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY0841!SPC OH S     -   0.99996796  x   0.99994051  =   0.99990847

JY0841!UTM  17      -   0.99996796  x   0.99980357  =   0.99977153

JY0841

JY0841                          SUPERSEDED SURVEY CONTROL

JY0841

JY0841  ELLIP H (04/01/98)  204.31   (m)                       GP(       ) 4 1

JY0841  NAD 83(1986)-  39 33 30.77220(N)    277 30 11.54427(E) AD(       ) 1

JY0841  NGVD 29 (??/??/??)  238.54   (m)          782.6    (f) RESET       3

JY0841

JY0841.Superseded values are not recommended for survey control.

JY0841.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0841.See file dsdata.txt to determine how the superseded data were derived.

JY0841

JY0841_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7141379832(NAD 83)

JY0841_MARKER: DB = BENCH MARK DISK

JY0841_SETTING: 30 = SET IN A LIGHT STRUCTURE

JY0841_SP_SET: CULVERT ENDWALL

JY0841_STAMPING: K 69 RESET 1960

JY0841_MARK LOGO: OHHD

JY0841_MAGNETIC: N = NO MAGNETIC MATERIAL

JY0841_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY0841_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JY0841+SATELLITE: SATELLITE OBSERVATIONS - September 05, 1991

JY0841

JY0841  HISTORY     - Date     Condition        Report By

JY0841  HISTORY     - 1960     MONUMENTED       OHHD

JY0841  HISTORY     - 1962     GOOD             NGS

JY0841  HISTORY     - 1974     GOOD             BAKER

JY0841  HISTORY     - 19910905 GOOD             WOOLPT

JY0841

JY0841                          STATION DESCRIPTION

JY0841

JY0841'DESCRIBED BY NATIONAL GEODETIC SURVEY 1962

JY0841'1.1 MI SW FROM ENTERPRISE.

JY0841'1.1 MILES SOUTHWEST FROM THE CHESAPEAKE AND OHIO RAILWAY STATION AT

JY0841'ENTERPRISE, HOCKING COUNTY, ALONG STATE ROUTE 180, 160 YARDS SOUTHWEST

JY0841'OF THE JUNCTION OF U.S. ROUTE 33, 12 YARDS NORTH OF THE CENTERLINE

JY0841'OF THE HIGHWAY, ABOUT 7.5 FEET LOWER THAN THE HIGHWAY, AND SET

JY0841'IN THE TOP OF A CONCRETE ENDWALL OF 10 FT - 8 IN SPAN BY 6 FT - 11 IN

JY0841'RISE CORRUGATED METAL CULVERT.

JY0841

JY0841                          STATION RECOVERY (1974)

JY0841

JY0841'RECOVERY NOTE BY M BAKER JR INCORPORATED 1974

JY0841'RECOVERED IN GOOD CONDITION.

JY0841

JY0841                          STATION RECOVERY (1991)

JY0841

JY0841'RECOVERY NOTE BY WOOLPERT CONSULTANTS 1991

JY0841'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0842 ***********************************************************************

JY0842  DESIGNATION -  L 69

JY0842  PID         -  JY0842

JY0842  STATE/COUNTY-  OH/HOCKING

JY0842  USGS QUAD   -  ROCKBRIDGE (1992)

JY0842

JY0842                         *CURRENT SURVEY CONTROL

JY0842  ___________________________________________________________________

JY0842* NAD 83(1986)-  39 32 01.     (N)    277 27 29      (E)     SCALED

JY0842* NAVD 88     -       318.867  (meters)    1046.15   (feet)  ADJUSTED

JY0842  ___________________________________________________________________

JY0842  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0842  DYNAMIC HT  -         318.674 (meters)    1045.52  (feet)  COMP

JY0842  MODELED GRAV-     980,013.3   (mgal)                       NAVD 88

JY0842

JY0842  VERT ORDER  -  SECOND    CLASS 0

JY0842

JY0842.The horizontal coordinates were scaled from a topographic map and have

JY0842.an estimated accuracy of +/- 6 seconds.

JY0842

JY0842.The orthometric height was determined by differential leveling

JY0842.and adjusted by the National Geodetic Survey in June 1991..

JY0842

JY0842.The geoid height was determined by GEOID03.

JY0842

JY0842.The dynamic height is computed by dividing the NAVD 88

JY0842.geopotential number by the normal gravity value computed on the

JY0842.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0842.degrees latitude (g = 980.6199 gals.).

JY0842

JY0842.The modeled gravity was interpolated from observed gravity values.

JY0842

JY0842;                    North         East    Units  Estimated Accuracy

JY0842;SPC OH S     -   170,250.      596,390.      MT  (+/- 180 meters Scaled)

JY0842

JY0842                          SUPERSEDED SURVEY CONTROL

JY0842

JY0842  NGVD 29 (??/??/92)  318.993  (m)         1046.56   (f) ADJ UNCH    2 0

JY0842

JY0842.Superseded values are not recommended for survey control.

JY0842.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0842.See file dsdata.txt to determine how the superseded data were derived.

JY0842

JY0842_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD674771(NAD 83)

JY0842_MARKER: DB = BENCH MARK DISK

JY0842_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0842_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0842_STAMPING: L 69 1934

JY0842_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0842+STABILITY: SURFACE MOTION

JY0842

JY0842  HISTORY     - Date     Condition        Report By

JY0842  HISTORY     - 1934     MONUMENTED       CGS

JY0842  HISTORY     - 1936     GOOD             NGS

JY0842  HISTORY     - 1974     GOOD             BAKER

JY0842

JY0842                          STATION DESCRIPTION

JY0842

JY0842'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0842'4.2 MI W FROM ENTERPRISE.

JY0842'4.2 MILES WEST ALONG STATE HIGHWAY 180 FROM THE CHESAPEAKE AND OHIO

JY0842'RAILWAY STATION AT ENTERPRISE, HOCKING COUNTY, 3.0 MILES WEST OF

JY0842'THE JUNCTION OF A ROAD LEADING EAST TO GIBSONVILLE, 95 YARDS

JY0842'SOUTHWEST OF THE SOUTHWEST CORNER OF A SHED, 70 YARDS SOUTHWEST

JY0842'OF THE CENTER OF THE JUNCTION OF A ROAD LEADING NORTH, 52 YARDS

JY0842'WEST OF THE NORTHEAST CORNER OF A PROPERTY-LINE FENCE, 22 FEET SOUTH

JY0842'OF THE CENTERLINE OF THE HIGHWAY, 3.5 FEET SOUTHEAST OF A POLE, 2

JY0842'FEET NORTH OF THE FENCE, AND ABOUT 3 FEET ABOVE THE LEVEL OF THE

JY0842'HIGHWAY.  A STANDARD DISK, STAMPED L 69 1934 AND SET IN THE TOP OF

JY0842'A CONCRETE POST.

JY0842

JY0842                          STATION RECOVERY (1974)

JY0842

JY0842'RECOVERY NOTE BY M BAKER JR INCORPORATED 1974

JY0842'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0103 ***********************************************************************

JY0103  DESIGNATION -  LOGAN

JY0103  PID         -  JY0103

JY0103  STATE/COUNTY-  OH/HOCKING

JY0103  USGS QUAD   -  LOGAN (1992)

JY0103

JY0103                         *CURRENT SURVEY CONTROL

JY0103  ___________________________________________________________________

JY0103* NAD 83(1986)-  39 32 23.     (N)    277 35 35      (E)     SCALED

JY0103* NAVD 88     -       225.816  (meters)     740.86   (feet)  POSTED

JY0103  ___________________________________________________________________

JY0103  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0103  DYNAMIC HT  -         225.680 (meters)     740.42  (feet)  COMP

JY0103  MODELED GRAV-     980,033.5   (mgal)                       NAVD 88

JY0103

JY0103  VERT ORDER  -  * POSTED, Code F , SEE BELOW

JY0103

JY0103.The horizontal coordinates were scaled from a topographic map and have

JY0103.an estimated accuracy of +/- 6 seconds.

JY0103

JY0103.The orthometric height was determined by differential leveling

JY0103.and adjusted by the National Geodetic Survey in 1992..

JY0103.* This is a POSTED BENCH MARK height.  Code F indicates a distribution

JY0103.rate greater than 8.0 mm/km.

JY0103

JY0103.The geoid height was determined by GEOID03.

JY0103

JY0103.The dynamic height is computed by dividing the NAVD 88

JY0103.geopotential number by the normal gravity value computed on the

JY0103.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0103.degrees latitude (g = 980.6199 gals.).

JY0103

JY0103.The modeled gravity was interpolated from observed gravity values.

JY0103

JY0103;                    North         East    Units  Estimated Accuracy

JY0103;SPC OH S     -   170,930.      608,000.      MT  (+/- 180 meters Scaled)

JY0103

JY0103                          SUPERSEDED SURVEY CONTROL

JY0103

JY0103  NGVD 29 (??/??/92)  225.943  (m)          741.28   (f) ADJ UNCH    2 0

JY0103

JY0103.Superseded values are not recommended for survey control.

JY0103.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0103.See file dsdata.txt to determine how the superseded data were derived.

JY0103

JY0103_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD790776(NAD 83)

JY0103_MARKER: DB = BENCH MARK DISK

JY0103_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0103_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0103_STAMPING: LOGAN 1934

JY0103_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0103+STABILITY: SURFACE MOTION

JY0103

JY0103  HISTORY     - Date     Condition        Report By

JY0103  HISTORY     - 1934     MONUMENTED       CGS

JY0103  HISTORY     - 1934     GOOD             NGS

JY0103

JY0103                          STATION DESCRIPTION

JY0103

JY0103'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0103'AT LOGAN.

JY0103'AT LOGAN, HOCKING COUNTY, 245 FEET SOUTHEAST OF THE NORTHEAST

JY0103'CORNER OF THE COUNTY COURTHOUSE, 140 FEET SOUTHEAST OF THE CENTER

JY0103'OF THE CROSSING OF STATE HIGHWAY 31 AND 75, 33 FEET EAST OF THE

JY0103'CENTERLINE OF STATE HIGHWAY 75, 19 FEET NORTH OF THE NORTHWEST

JY0103'CORNER OF A DIRT STATION, 2 FEET SOUTH OF THE SOUTHWEST CORNER OF

JY0103'THE PORCH OF A LADIES REST ROOM, AND LEVEL WITH THE PAVEMENT.

JY0103'A STANDARD DISK, STAMPED LOGAN 1934 AND SET IN THE TOP OF A CONCRETE

JY0103'POST.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0843 ***********************************************************************

JY0843  DESIGNATION -  M 69

JY0843  PID         -  JY0843

JY0843  STATE/COUNTY-  OH/HOCKING

JY0843  USGS QUAD   -  ROCKBRIDGE (1992)

JY0843

JY0843                         *CURRENT SURVEY CONTROL

JY0843  ___________________________________________________________________

JY0843* NAD 83(1986)-  39 31 36.     (N)    277 25 34      (E)     SCALED

JY0843* NAVD 88     -       266.030  (meters)     872.80   (feet)  ADJUSTED

JY0843  ___________________________________________________________________

JY0843  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY0843  DYNAMIC HT  -         265.868 (meters)     872.27  (feet)  COMP

JY0843  MODELED GRAV-     980,009.6   (mgal)                       NAVD 88

JY0843

JY0843  VERT ORDER  -  SECOND    CLASS 0

JY0843

JY0843.The horizontal coordinates were scaled from a topographic map and have

JY0843.an estimated accuracy of +/- 6 seconds.

JY0843

JY0843.The orthometric height was determined by differential leveling

JY0843.and adjusted by the National Geodetic Survey in June 1991..

JY0843

JY0843.The geoid height was determined by GEOID03.

JY0843

JY0843.The dynamic height is computed by dividing the NAVD 88

JY0843.geopotential number by the normal gravity value computed on the

JY0843.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0843.degrees latitude (g = 980.6199 gals.).

JY0843

JY0843.The modeled gravity was interpolated from observed gravity values.

JY0843

JY0843;                    North         East    Units  Estimated Accuracy

JY0843;SPC OH S     -   169,480.      593,650.      MT  (+/- 180 meters Scaled)

JY0843

JY0843                          SUPERSEDED SURVEY CONTROL

JY0843

JY0843  NGVD 29 (??/??/92)  266.156  (m)          873.21   (f) ADJ UNCH    2 0

JY0843

JY0843.Superseded values are not recommended for survey control.

JY0843.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0843.See file dsdata.txt to determine how the superseded data were derived.

JY0843

JY0843_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD647764(NAD 83)

JY0843_MARKER: DB = BENCH MARK DISK

JY0843_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0843_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0843_STAMPING: M 69 1934

JY0843_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0843+STABILITY: SURFACE MOTION

JY0843

JY0843  HISTORY     - Date     Condition        Report By

JY0843  HISTORY     - 1934     MONUMENTED       CGS

JY0843  HISTORY     - 1936     GOOD             NGS

JY0843  HISTORY     - 1974     GOOD             BAKER

JY0843

JY0843                          STATION DESCRIPTION

JY0843

JY0843'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0843'6.4 MI SW FROM ENTERPRISE.

JY0843'6.4 MILES SOUTHWEST ALONG STATE HIGHWAY 180 FROM THE CHESAPEAKE AND

JY0843'OHIO RAILWAY STATION AT ENTERPRISE, HOCKING COUNTY, 1.0 MILE WEST

JY0843'OF THE JUNCTION OF A ROAD LEADING NORTH TO CANTWELL CLIFFS, 50 YARDS

JY0843'WEST OF A CONCRETE BRIDGE OVER A CREEK, 23 YARDS SOUTHWEST OF A

JY0843'48-INCH BEECH TREE, 30 FEET SOUTH OF THE CENTERLINE OF THE HIGHWAY,

JY0843'3 FEET EAST OF A GATEPOST, AND ABOUT 3 FEET LOWER THAN THE

JY0843'HIGHWAY.  A STANDARD DISK, STAMPED M 69 1934 AND SET IN THE TOP OF

JY0843'A CONCRETE POST.

JY0843

JY0843                          STATION RECOVERY (1974)

JY0843

JY0843'RECOVERY NOTE BY M BAKER JR INCORPORATED 1974

JY0843'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1393 ***********************************************************************

JY1393  DESIGNATION -  MCDANIEL RESET

JY1393  PID         -  JY1393

JY1393  STATE/COUNTY-  OH/HOCKING

JY1393  USGS QUAD   -  UNION FURNACE (1995)

JY1393

JY1393                         *CURRENT SURVEY CONTROL

JY1393  ___________________________________________________________________

JY1393* NAD 83(1995)-  39 23 13.58583(N)    277 40 47.65528(E)     ADJUSTED

JY1393* NAVD 88     -       336.     (meters)    1102.     (feet)  SCALED

JY1393  ___________________________________________________________________

JY1393  LAPLACE CORR-          -0.31  (seconds)                    DEFLEC99

JY1393  GEOID HEIGHT-         -34.07  (meters)                     GEOID03

JY1393

JY1393  HORZ ORDER  -  FIRST

JY1393

JY1393.The horizontal coordinates were established by classical geodetic methods

JY1393.and adjusted by the National Geodetic Survey in April 1998..

JY1393

JY1393.The orthometric height was scaled from a topographic map.

JY1393

JY1393.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1393

JY1393.The geoid height was determined by GEOID03.

JY1393

JY1393;                    North         East     Units Scale Factor Converg.

JY1393;SPC OH S     -   154,002.674   615,498.179   MT  0.99993591   +0 06 50.9

JY1393;SPC OH S     -   505,257.11  2,019,346.94   sFT  0.99993591   +0 06 50.9

JY1393;UTM  17      - 4,360,566.849   386,314.094   MT  0.99975913   -0 50 16.0

JY1393

JY1393!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1393!SPC OH S     -   0.99995266  x   0.99993591  =   0.99988857

JY1393!UTM  17      -   0.99995266  x   0.99975913  =   0.99971180

JY1393

JY1393:                Primary Azimuth Mark                     Grid Az

JY1393:SPC OH S     -  MCDANIEL AZ MK                           110 11 48.7

JY1393:UTM  17      -  MCDANIEL AZ MK                           111 08 55.6

JY1393

JY1393|---------------------------------------------------------------------|

JY1393| PID    Reference Object                     Distance      Geod. Az  |

JY1393|                                                           dddmmss.s |

JY1393| CE6877 MCDANIEL AZ MK                                     1101839.6 |

JY1393| JY1394 ATHENS POWER CO MAST                APPROX. 0.7 KM 1111255.2 |

JY1393| CE6878 MCDANIEL RM 1                        15.432 METERS 25407     |

JY1393| CE6879 MCDANIEL RM 2                        14.218 METERS 35631     |

JY1393|---------------------------------------------------------------------|

JY1393

JY1393                          SUPERSEDED SURVEY CONTROL

JY1393

JY1393  NAD 83(1986)-  39 23 13.58814(N)    277 40 47.65050(E) AD(       ) 1

JY1393  NAD 27      -  39 23 13.33800(N)    277 40 47.19600(E) AD(       ) 1

JY1393

JY1393.Superseded values are not recommended for survey control.

JY1393.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1393.See file dsdata.txt to determine how the superseded data were derived.

JY1393

JY1393_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD8631460567(NAD 83)

JY1393_MARKER: V = STONE MONUMENT

JY1393_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY1393

JY1393  HISTORY     - Date     Condition        Report By

JY1393  HISTORY     - 1944     MONUMENTED       CGS

JY1393  HISTORY     - 1944     SEE DESCRIPTION  CGS

JY1393  HISTORY     - 1945     GOOD             CGS

JY1393  HISTORY     - 1976     GOOD             NGS

JY1393

JY1393                          STATION DESCRIPTION

JY1393

JY1393'DESCRIBED BY COAST AND GEODETIC SURVEY 1944 (FN)

JY1393'STATION WAS RECOVERED.  THE ORIGINAL 6- BY 6-INCH MARBLE POST WAS

JY1393'REPLACED BY A BRONZE STATION DISK.

JY1393'

JY1393'STATION IS LOCATED ABOUT 2.5 MILES ESE OF THE VILLAGE OF STARR, ON

JY1393'PROPERTY OF MR. MCDANIEL WHO LIVES ABOUT 0.25 MILE SSE OF THE

JY1393'STATION.  THE MARK IS SITUATED ON THE CREST OF RIDGE, AT THE W EDGE

JY1393'OF CLEARED FIELD, AND 6 FEET E OF A WITNESS POST.  THE DISK IS

JY1393'STAMPED MCDANIEL, 1882 1944.

JY1393'

JY1393'SURFACE, UNDERGROUND, REFERENCE, AND AZIMUTH MARKS ARE BRONZE DISKS

JY1393'SET IN CONCRETE.

JY1393'

JY1393'TWO REFERENCE MARKS AND AN AZIMUTH MARK WERE ESTABLISHED.

JY1393'

JY1393'REFERENCE MARK 1 IS ON THE CREST OF THE HILL.  IT IS STAMPED MCDANIEL

JY1393'NO 1 1944.

JY1393'

JY1393'REFERENCE MARK 2 IS AT THE W EDGE OF THE CLEARING.  IT IS STAMPED

JY1393'MCDANIEL NO 2 1944.

JY1393'

JY1393'AZIMUTH MARK IS 18 FEET S OF THE APPROXIMATE CENTER LINE OF ROAD, 39

JY1393'FEET N OF THE NE CORNER OF A WHITE FRAME HOUSE, AND 3 FEET

JY1393'SW OF A WITNESS POST.  IT IS STAMPED MCDANIEL 1944.

JY1393'

JY1393'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAYS 328 AND 56

JY1393'IN THE VILLAGE OF STARR, GO E ON HIGHWAY 56 FOR 4.1 MILES TO THE

JY1393'JUNCTION OF STATE HIGHWAY 278.  TURN RIGHT AND FOLLOW HIGHWAY 278 FOR

JY1393'0.3 MILE TO A CROSS ROAD, TURN RIGHT ON DIRT ROAD AND GO 0.2 MILE

JY1393'TO THE AZIMUTH MARK ON THE LEFT.  CONTINUE FOR 0.45 MILE TO A BOARD

JY1393'GATE ON THE RIGHT, TURN RIGHT INTO A FIELD AND GO ABOUT 0.3

JY1393'MILE TO THE CREST OF THE RIDGE.  TURN LEFT AND FOLLOW ALONG THE

JY1393'RIDGE FOR ABOUT 0.2 MILE TO THE HIGHEST POINT AND THE STATION.

JY1393'

JY1393'HEIGHT OF LIGHT ABOVE STATION MARK - 30 METERS.

JY1393

JY1393                          STATION RECOVERY (1945)

JY1393

JY1393'RECOVERY NOTE BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1393'STATION WAS RECOVERED AS DESCRIBED BY FRED NATELLA IN 1944.  AT THIS

JY1393'TIME THE STATION WAS RE-MARKED, REFERENCE MARKS AND AN AZIMUTH

JY1393'MARK WERE ESTABLISHED.  THE 1944 DESCRIPTION IS COMPLETE AND

JY1393'ADEQUATE, AND SINCE THE STATION WAS OCCUPIED BY THIS PARTY

JY1393'LATE LAST FALL, A COMPLETE DESCRIPTION IS UNNECESSARY AT THIS TIME.

JY1393

JY1393                          STATION RECOVERY (1976)

JY1393

JY1393'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1976 (JRS)

JY1393'THE STATION MARK, REFERENCE MARK 1 AND 2 WERE RECOVERED AND

JY1393'FOUND IN GOOD CONDITION.  HOWEVER A SHORTER DISTANCE OF 0.026

JY1393'METER TO REFERENCE MARK 1 AND A SHORTER DISTANCE OF 0.058 METER

JY1393'TO REFERENCE MARK 2 WAS NOTED.  THE DIRECTIONS TO THE REFERENCE

JY1393'MARKS CHECKED WITH THE 1944 DATA.  THE AZIMUTH MARK WAS SEARCHED

JY1393'FOR AND WAS NOT FOUND.  A COMPLETE NEW DESCRIPTION FOLLOWS.

JY1393'

JY1393'THE STATION IS LOCATED ABOUT 13 MILES NORTH-NORTHWEST OF ATHENS,

JY1393'8 MILES NORTH-NORTHEAST OF ZALESKI AND 7 MILES NORTHWEST OF

JY1393'NEW MARSHFIELD.  THE STATION IS ON PROPERTY OWNED BY MR. JAMES

JY1393'R. MCDANIEL, 7444 ST. CHARLES AVENUE, NEW ORLEANS, LOUISIANA.

JY1393'PHONE 504 866-0064.

JY1393'

JY1393'TO REACH THE STATION FROM THE JUNCTION OF STATE HIGHWAYS 278

JY1393'AND 56, 14 MILES WEST OF ATHENS, GO SOUTH ON STATE HIGHWAY 278

JY1393'FOR 0.3 MILE TO A CROSSROAD.  TURN RIGHT AND GO WEST FOR 0.7

JY1393'MILE TO A FIELD ROAD RIGHT AT A SHARP TURN.  TURN RIGHT AND GO

JY1393'NORTH ACROSS A MEADOW ON A TRACK ROAD FOR 0.1 MILE TO THE EDGE

JY1393'OF WOODS, CONTINUE NORTHERLY ON A WOODS ROAD FOR 0.15 MILE TO A

JY1393'DIM TRACK ROAD UPHILL ON THE RIGHT.  TURN RIGHT AND GO UPHILL

JY1393'FOR 0.05 MILE TO THE TOP AND STATION SITE.

JY1393'

JY1393'THE STATION MARK IS A STANDARD DISK STAMPED---MCDANIEL 1882

JY1393'1944---, SET IN THE TOP OF A 12-INCH SQUARE CONCRETE MONUMENT THAT

JY1393'PROJECTS 3 INCHES ABOVE THE GROUND.

JY1393'

JY1393'REFERENCE MARK 1 IS A STANDARD DISK STAMPED---MCDANIEL NO 1 1944---,

JY1393'SET IN THE TOP OF A 12-INCH SQUARE CONCRETE MONUMENT THAT

JY1393'PROJECTS 4 INCHES ABOVE THE GROUND.

JY1393'

JY1393'REFERENCE MARK 2 IS A STANDARD DISK STAMPED---MCDANIEL NO 2 1944---,

JY1393'SET IN THE TOP OF A 12-INCH SQUARE CONCRETE MONUMENT THAT

JY1393'PROJECTS 3 INCHES ABOVE THE GROUND.

JY1393'

JY1393'NO SUITABLE LOCATION FOR A NEW AZIMUTH MARK.

JY1393'HEIGHT OF LIGHT ABOVE MARK WAS 22.5 METERS.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0109 ***********************************************************************

JY0109  DESIGNATION -  N 68

JY0109  PID         -  JY0109

JY0109  STATE/COUNTY-  OH/HOCKING

JY0109  USGS QUAD   -  GORE (1992)

JY0109

JY0109                         *CURRENT SURVEY CONTROL

JY0109  ___________________________________________________________________

JY0109* NAD 83(1986)-  39 35 24.     (N)    277 39 03      (E)     SCALED

JY0109* NAVD 88     -       232.175  (meters)     761.73   (feet)  ADJUSTED

JY0109  ___________________________________________________________________

JY0109  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY0109  DYNAMIC HT  -         232.039 (meters)     761.28  (feet)  COMP

JY0109  MODELED GRAV-     980,035.7   (mgal)                       NAVD 88

JY0109

JY0109  VERT ORDER  -  SECOND    CLASS 0

JY0109

JY0109.The horizontal coordinates were scaled from a topographic map and have

JY0109.an estimated accuracy of +/- 6 seconds.

JY0109

JY0109.The orthometric height was determined by differential leveling

JY0109.and adjusted by the National Geodetic Survey in June 1991..

JY0109

JY0109.The geoid height was determined by GEOID03.

JY0109

JY0109.The dynamic height is computed by dividing the NAVD 88

JY0109.geopotential number by the normal gravity value computed on the

JY0109.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0109.degrees latitude (g = 980.6199 gals.).

JY0109

JY0109.The modeled gravity was interpolated from observed gravity values.

JY0109

JY0109;                    North         East    Units  Estimated Accuracy

JY0109;SPC OH S     -   176,520.      612,960.      MT  (+/- 180 meters Scaled)

JY0109

JY0109                          SUPERSEDED SURVEY CONTROL

JY0109

JY0109  NGVD 29 (??/??/92)  232.426  (m)          762.55   (f) ADJ UNCH    2 0

JY0109

JY0109.Superseded values are not recommended for survey control.

JY0109.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0109.See file dsdata.txt to determine how the superseded data were derived.

JY0109

JY0109_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD841831(NAD 83)

JY0109_MARKER: DB = BENCH MARK DISK

JY0109_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0109_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0109_STAMPING: N 68 1934

JY0109_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0109+STABILITY: SURFACE MOTION

JY0109

JY0109  HISTORY     - Date     Condition        Report By

JY0109  HISTORY     - 1934     MONUMENTED       CGS

JY0109  HISTORY     - 1934     GOOD             NGS

JY0109  HISTORY     - 19890104 MARK NOT FOUND   USPSQD

JY0109

JY0109                          STATION DESCRIPTION

JY0109

JY0109'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0109'5.2 MI NE FROM LOGAN.

JY0109'5.2 MILES NORTHEAST ALONG STATE HIGHWAY 75 FROM THE POST OFFICE

JY0109'AT LOGAN, HOCKING COUNTY, 63 FEET SOUTHWEST OF THE CENTER OF THE

JY0109'JUNCTION OF STATE HIGHWAY 312, 72 FEET WEST OF THE CENTERLINE OF

JY0109'STATE HIGHWAY 75, 27 FEET SOUTH OF THE CENTERLINE OF STATE HIGHWAY

JY0109'312, 45 FEET WEST OF THE NORTHWEST CORNER OF A NEW CONCRETE BRIDGE,

JY0109'AND ABOUT 3 FEET LOWER THAN THE ROADWAY.  A STANDARD DISK, STAMPED

JY0109'N 68 1934 AND SET IN THE TOP OF A CONCRETE POST.

JY0109

JY0109                          STATION RECOVERY (1989)

JY0109

JY0109'RECOVERY NOTE BY US POWER SQUADRON 1989 (RCC)

JY0109'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0844 ***********************************************************************

JY0844  DESIGNATION -  N 69

JY0844  PID         -  JY0844

JY0844  STATE/COUNTY-  OH/HOCKING

JY0844  USGS QUAD   -  ROCKBRIDGE (1992)

JY0844

JY0844                         *CURRENT SURVEY CONTROL

JY0844  ___________________________________________________________________

JY0844* NAD 83(1986)-  39 31 03.     (N)    277 23 31      (E)     SCALED

JY0844* NAVD 88     -       256.694  (meters)     842.17   (feet)  ADJUSTED

JY0844  ___________________________________________________________________

JY0844  GEOID HEIGHT-         -34.09  (meters)                     GEOID03

JY0844  DYNAMIC HT  -         256.538 (meters)     841.66  (feet)  COMP

JY0844  MODELED GRAV-     980,012.4   (mgal)                       NAVD 88

JY0844

JY0844  VERT ORDER  -  SECOND    CLASS 0

JY0844

JY0844.The horizontal coordinates were scaled from a topographic map and have

JY0844.an estimated accuracy of +/- 6 seconds.

JY0844

JY0844.The orthometric height was determined by differential leveling

JY0844.and adjusted by the National Geodetic Survey in June 1991..

JY0844

JY0844.The geoid height was determined by GEOID03.

JY0844

JY0844.The dynamic height is computed by dividing the NAVD 88

JY0844.geopotential number by the normal gravity value computed on the

JY0844.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0844.degrees latitude (g = 980.6199 gals.).

JY0844

JY0844.The modeled gravity was interpolated from observed gravity values.

JY0844

JY0844;                    North         East    Units  Estimated Accuracy

JY0844;SPC OH S     -   168,470.      590,710.      MT  (+/- 180 meters Scaled)

JY0844

JY0844                          SUPERSEDED SURVEY CONTROL

JY0844

JY0844  NGVD 29 (??/??/92)  256.827  (m)          842.61   (f) ADJ UNCH    2 0

JY0844

JY0844.Superseded values are not recommended for survey control.

JY0844.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0844.See file dsdata.txt to determine how the superseded data were derived.

JY0844

JY0844_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD617754(NAD 83)

JY0844_MARKER: DB = BENCH MARK DISK

JY0844_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0844_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0844_STAMPING: N 69 1934

JY0844_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0844+STABILITY: SURFACE MOTION

JY0844

JY0844  HISTORY     - Date     Condition        Report By

JY0844  HISTORY     - 1934     MONUMENTED       CGS

JY0844  HISTORY     - 1936     GOOD             NGS

JY0844  HISTORY     - 1974     GOOD             BAKER

JY0844

JY0844                          STATION DESCRIPTION

JY0844

JY0844'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0844'8.4 MI SW FROM ENTERPRISE.

JY0844'8.4 MILES SOUTHWEST ALONG STATE HIGHWAY 180 FROM THE CHESAPEAKE

JY0844'AND OHIO RAILWAY STATION AT ENTERPRISE, HOCKING COUNTY, 120 YARDS

JY0844'WEST OF THE WEST CORNER OF AN OHIO FUEL AND GAS CO. PUMPING STATION,

JY0844'80 YARDS NORTHEAST OF THE NORTHEAST CORNER OF A FRAME FARMHOUSE ON

JY0844'THE NORTHWEST SIDE OF THE HIGHWAY, 21 YARDS NORTHEAST OF THE

JY0844'NORTHEAST CORNER OF A CONCRETE CULVERT, 37 FEET NORTHWEST OF THE

JY0844'CENTERLINE OF THE HIGHWAY, AND LEVEL WITH THE HIGHWAY.  A STANDARD

JY0844'DISK, STAMPED N 69 1934 AND SET IN THE TOP OF A CONCRETE POST.

JY0844

JY0844                          STATION RECOVERY (1974)

JY0844

JY0844'RECOVERY NOTE BY M BAKER JR INCORPORATED 1974

JY0844'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0107 ***********************************************************************

JY0107  DESIGNATION -  P 68

JY0107  PID         -  JY0107

JY0107  STATE/COUNTY-  OH/HOCKING

JY0107  USGS QUAD   -  GORE (1992)

JY0107

JY0107                         *CURRENT SURVEY CONTROL

JY0107  ___________________________________________________________________

JY0107* NAD 83(1986)-  39 35 06.     (N)    277 37 52      (E)     SCALED

JY0107* NAVD 88     -       233.334  (meters)     765.53   (feet)  POSTED

JY0107  ___________________________________________________________________

JY0107  GEOID HEIGHT-         -34.11  (meters)                     GEOID03

JY0107  DYNAMIC HT  -         233.200 (meters)     765.09  (feet)  COMP

JY0107  MODELED GRAV-     980,035.4   (mgal)                       NAVD 88

JY0107

JY0107  VERT ORDER  -  * POSTED, Code F , SEE BELOW

JY0107

JY0107.The horizontal coordinates were scaled from a topographic map and have

JY0107.an estimated accuracy of +/- 6 seconds.

JY0107

JY0107.The orthometric height was determined by differential leveling

JY0107.and adjusted by the National Geodetic Survey in 1992..

JY0107.* This is a POSTED BENCH MARK height.  Code F indicates a distribution

JY0107.rate greater than 8.0 mm/km.

JY0107

JY0107.The geoid height was determined by GEOID03.

JY0107

JY0107.The dynamic height is computed by dividing the NAVD 88

JY0107.geopotential number by the normal gravity value computed on the

JY0107.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0107.degrees latitude (g = 980.6199 gals.).

JY0107

JY0107.The modeled gravity was interpolated from observed gravity values.

JY0107

JY0107;                    North         East    Units  Estimated Accuracy

JY0107;SPC OH S     -   175,970.      611,260.      MT  (+/- 180 meters Scaled)

JY0107

JY0107                          SUPERSEDED SURVEY CONTROL

JY0107

JY0107  NGVD 29 (??/??/92)  233.558  (m)          766.26   (f) ADJ UNCH    2 0

JY0107

JY0107.Superseded values are not recommended for survey control.

JY0107.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0107.See file dsdata.txt to determine how the superseded data were derived.

JY0107

JY0107_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD824825(NAD 83)

JY0107_MARKER: DB = BENCH MARK DISK

JY0107_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0107_SP_SET: CONCRETE POST

JY0107_STAMPING: P 68 1934

JY0107_MARK LOGO: CGS

JY0107_MAGNETIC: N = NO MAGNETIC MATERIAL

JY0107_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0107+STABILITY: SURFACE MOTION

JY0107

JY0107  HISTORY     - Date     Condition        Report By

JY0107  HISTORY     - 1934     MONUMENTED       CGS

JY0107  HISTORY     - 1934     GOOD             NGS

JY0107  HISTORY     - 19820707 MARK NOT FOUND   OHDT

JY0107  HISTORY     - 19890104 MARK NOT FOUND   USPSQD

JY0107

JY0107                          STATION DESCRIPTION

JY0107

JY0107'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0107'4 MI NE FROM LOGAN.

JY0107'4.0 MILES NORTHEAST ALONG STATE HIGHWAY 75 FROM THE POST OFFICE

JY0107'AT LOGAN, HOCKING COUNTY, 170 YARDS NORTH OF A CONCRETE BRIDGE

JY0107'OVER A CREEK, 160 YARDS SOUTH OF THE SOUTHEAST CORNER OF THE

JY0107'HAMILTON LEMON FARMHOUSE, 24 FEET WEST OF THE CENTERLINE OF THE

JY0107'HIGHWAY, 15 FEET NORTH OF A POLE, 2 FEET EAST OF THE FENCE, AND ABOUT

JY0107'ONE FOOT ABOVE THE LEVEL OF THE ROADWAY.  A STANDARD DISK, STAMPED

JY0107'P 68 1934 AND SET IN THE TOP OF A CONCRETE POST.

JY0107

JY0107                          STATION RECOVERY (1982)

JY0107

JY0107'RECOVERY NOTE BY OHIO DEPARTMENT OF TRANSPORTATION 1982

JY0107'MARK NOT FOUND.

JY0107

JY0107                          STATION RECOVERY (1989)

JY0107

JY0107'RECOVERY NOTE BY US POWER SQUADRON 1989 (RCC)

JY0107'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1754 ***********************************************************************

JY1754  DESIGNATION -  P 68 RESET

JY1754  PID         -  JY1754

JY1754  STATE/COUNTY-  OH/HOCKING

JY1754  USGS QUAD   -  GORE (1992)

JY1754

JY1754                         *CURRENT SURVEY CONTROL

JY1754  ___________________________________________________________________

JY1754* NAD 83(1986)-  39 35 06.     (N)    277 37 52      (E)     SCALED

JY1754* NAVD 88     -       232.50   (+/-2cm)     762.8    (feet)  VERTCON

JY1754  ___________________________________________________________________

JY1754  GEOID HEIGHT-         -34.11  (meters)                     GEOID03

JY1754

JY1754  VERT ORDER  -  THIRD (See Below)

JY1754

JY1754.The horizontal coordinates were scaled from a topographic map and have

JY1754.an estimated accuracy of +/- 6 seconds.

JY1754

JY1754.The NAVD 88 height was computed by applying the VERTCON shift value to

JY1754.the NGVD 29 height (displayed under SUPERSEDED SURVEY CONTROL.)

JY1754.The vertical order pertains to the NGVD 29 superseded value.

JY1754

JY1754.The geoid height was determined by GEOID03.

JY1754

JY1754;                    North         East    Units  Estimated Accuracy

JY1754;SPC OH S     -   175,970.      611,260.      MT  (+/- 180 meters Scaled)

JY1754

JY1754                          SUPERSEDED SURVEY CONTROL

JY1754

JY1754  NGVD 29 (10/13/05)  232.71   (m)          763.5    (f) RESET       3

JY1754

JY1754.Superseded values are not recommended for survey control.

JY1754.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1754.See file dsdata.txt to determine how the superseded data were derived.

JY1754

JY1754_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD824825(NAD 83)

JY1754_MARKER: DV = VERTICAL CONTROL DISK

JY1754_SETTING: 37 = SET IN A MASSIVE RETAINING WALL

JY1754_SP_SET: WINGWALL

JY1754_STAMPING: P 68 RESET 1973

JY1754_MARK LOGO: NGS

JY1754_MAGNETIC: N = NO MAGNETIC MATERIAL

JY1754_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY1754

JY1754  HISTORY     - Date     Condition        Report By

JY1754  HISTORY     - 1973     MONUMENTED       OHDT

JY1754  HISTORY     - 19820512 GOOD             OHDT

JY1754

JY1754                          STATION DESCRIPTION

JY1754

JY1754'DESCRIBED BY OHIO DEPARTMENT OF TRANSPORTATION 1982

JY1754'GO APPROXIMATELY 4.0 MILES (6.4 KM) NORTHEAST ALONG STATE ROUTE 93

JY1754'FROM THE POST OFFICE IN LOGAN, TO THE INTERSECTION OF STATE ROUTE 93

JY1754'AND HOCKING COUNTY ROAD 76 (BEAR RUN ROAD), IN THE NORTHWEST PART OF

JY1754'A 12 FOOT (3.7 M) CULVERT (HOC-93-16.61), 160 FEET (48.8 M) NORTHEAST

JY1754'OF THE INTERSECTION OF STATE ROUTE 93 AND HOCKING COUNTY ROAD 76

JY1754'(BEAR RUN ROAD), 50 FEET (15.2 M) SOUTHEAST OF A POWER AND TELEPHONE

JY1754'POLE, 50 FEET (15.2 M) EAST OF THE EAST EDGE OF PAVEMENT OF HOCKING

JY1754'COUNTY ROAD 76 AND 100 FEET (30.5 M) NORTH OF THE NORTH EDGE OF

JY1754'PAVEMENT OF STATE ROUTE 93.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0846 ***********************************************************************

JY0846  DESIGNATION -  P 69

JY0846  PID         -  JY0846

JY0846  STATE/COUNTY-  OH/HOCKING

JY0846  USGS QUAD   -  CLEARPORT (1994)

JY0846

JY0846                         *CURRENT SURVEY CONTROL

JY0846  ___________________________________________________________________

JY0846* NAD 83(1986)-  39 30 01.     (N)    277 21 50      (E)     SCALED

JY0846* NAVD 88     -       250.674  (meters)     822.42   (feet)  ADJUSTED

JY0846  ___________________________________________________________________

JY0846  GEOID HEIGHT-         -34.07  (meters)                     GEOID03

JY0846  DYNAMIC HT  -         250.522 (meters)     821.92  (feet)  COMP

JY0846  MODELED GRAV-     980,015.3   (mgal)                       NAVD 88

JY0846

JY0846  VERT ORDER  -  SECOND    CLASS 0

JY0846

JY0846.The horizontal coordinates were scaled from a topographic map and have

JY0846.an estimated accuracy of +/- 6 seconds.

JY0846

JY0846.The orthometric height was determined by differential leveling

JY0846.and adjusted by the National Geodetic Survey in June 1991..

JY0846

JY0846.The geoid height was determined by GEOID03.

JY0846

JY0846.The dynamic height is computed by dividing the NAVD 88

JY0846.geopotential number by the normal gravity value computed on the

JY0846.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0846.degrees latitude (g = 980.6199 gals.).

JY0846

JY0846.The modeled gravity was interpolated from observed gravity values.

JY0846

JY0846;                    North         East    Units  Estimated Accuracy

JY0846;SPC OH S     -   166,560.      588,290.      MT  (+/- 180 meters Scaled)

JY0846

JY0846                          SUPERSEDED SURVEY CONTROL

JY0846

JY0846  NGVD 29 (??/??/92)  250.809  (m)          822.86   (f) ADJ UNCH    2 0

JY0846

JY0846.Superseded values are not recommended for survey control.

JY0846.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0846.See file dsdata.txt to determine how the superseded data were derived.

JY0846

JY0846_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD593735(NAD 83)

JY0846_MARKER: DB = BENCH MARK DISK

JY0846_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0846_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0846_STAMPING: P 69 1934

JY0846_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0846+STABILITY: SURFACE MOTION

JY0846

JY0846  HISTORY     - Date     Condition        Report By

JY0846  HISTORY     - 1934     MONUMENTED       CGS

JY0846  HISTORY     - 1936     GOOD             NGS

JY0846  HISTORY     - 1961     MARK NOT FOUND   NGS

JY0846

JY0846                          STATION DESCRIPTION

JY0846

JY0846'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0846'0.8 MI SW FROM MOUND CROSSING.

JY0846'0.8 MILE SOUTHWEST ALONG STATE HIGHWAY 180 FROM THE SCHOOL AT

JY0846'MOUND CROSSING, HOCKING COUNTY, 0.5 MILE WEST OF THE JUNCTION OF

JY0846'STATE HIGHWAY 374, 85 YARDS WEST OF THE SOUTHWEST CORNER OF THE LOUIS

JY0846'FOX FARMHOUSE, 25 YARDS NORTHWEST OF THE CENTER OF THE JUNCTION OF

JY0846'A ROAD LEADING NORTH TO THE RAYMOND STEVENS FARMHOUSE, 12 YARDS

JY0846'NORTHWEST OF THE NORTHWEST CORNER OF A CONCRETE CULVERT, 12 YARDS

JY0846'NORTH OF THE CENTERLINE OF THE HIGHWAY, 4 FEET WEST OF THE SOUTHWEST

JY0846'CORNER OF A FENCE, AND ABOUT ONE FOOT LOWER THAN THE HIGHWAY.  A

JY0846'STANDARD DISK, STAMPED P 69 1934 AND SET IN THE TOP OF A CONCRETE

JY0846'POST.

JY0846

JY0846                          STATION RECOVERY (1961)

JY0846

JY0846'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1961

JY0846'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0845 ***********************************************************************

JY0845  DESIGNATION -  PTS 8

JY0845  PID         -  JY0845

JY0845  STATE/COUNTY-  OH/HOCKING

JY0845  USGS QUAD   -  ROCKBRIDGE (1992)

JY0845

JY0845                         *CURRENT SURVEY CONTROL

JY0845  ___________________________________________________________________

JY0845* NAD 83(1986)-  39 30 22.     (N)    277 22 34      (E)     SCALED

JY0845* NAVD 88     -       258.474  (meters)     848.01   (feet)  ADJUSTED

JY0845  ___________________________________________________________________

JY0845  GEOID HEIGHT-         -34.08  (meters)                     GEOID03

JY0845  DYNAMIC HT  -         258.317 (meters)     847.50  (feet)  COMP

JY0845  MODELED GRAV-     980,014.1   (mgal)                       NAVD 88

JY0845

JY0845  VERT ORDER  -  SECOND    CLASS 0

JY0845

JY0845.The horizontal coordinates were scaled from a topographic map and have

JY0845.an estimated accuracy of +/- 6 seconds.

JY0845

JY0845.The orthometric height was determined by differential leveling

JY0845.and adjusted by the National Geodetic Survey in June 1991..

JY0845

JY0845.The geoid height was determined by GEOID03.

JY0845

JY0845.The dynamic height is computed by dividing the NAVD 88

JY0845.geopotential number by the normal gravity value computed on the

JY0845.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0845.degrees latitude (g = 980.6199 gals.).

JY0845

JY0845.The modeled gravity was interpolated from observed gravity values.

JY0845

JY0845;                    North         East    Units  Estimated Accuracy

JY0845;SPC OH S     -   167,210.      589,350.      MT  (+/- 180 meters Scaled)

JY0845

JY0845                          SUPERSEDED SURVEY CONTROL

JY0845

JY0845  NGVD 29 (??/??/92)  258.605  (m)          848.44   (f) ADJ UNCH    2 0

JY0845

JY0845.Superseded values are not recommended for survey control.

JY0845.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0845.See file dsdata.txt to determine how the superseded data were derived.

JY0845

JY0845_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD603742(NAD 83)

JY0845_MARKER: P = PIPE CAP

JY0845_SETTING: 17 = SET INTO TOP OF METAL PIPE DRIVEN INTO GROUND

JY0845_SP_SET: METAL PIPE DRIVEN INTO GROUND

JY0845_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY0845

JY0845  HISTORY     - Date     Condition        Report By

JY0845  HISTORY     - UNK      MONUMENTED       CGS

JY0845  HISTORY     - 1936     GOOD             NGS

JY0845

JY0845                          STATION DESCRIPTION

JY0845

JY0845'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0845'AT MOUND CROSSING.

JY0845'PTS NO 8 848

JY0845'AT MOUND CROSSING, HOCKING COUNTY, ON STATE HIGHWAY 180, 27 YARDS

JY0845'NORTHEAST OF THE CENTER OF THE JUNCTION OF A GRAVEL ROAD LEADING

JY0845'NORTH, 29 YARDS NORTHEAST OF THE NORTHWEST CORNER OF THE MOUND

JY0845'CROSSING STORE, 40 FEET NORTH OF THE CENTERLINE OF THE HIGHWAY,

JY0845'11 FEET SOUTH OF THE SOUTHWEST CORNER OF A SCHOOLHOUSE, 7 FEET

JY0845'WEST OF A MAPLE TREE, AND ABOUT 3 FEET ABOVE THE LEVEL OF THE HIGHWAY.

JY0845'A UNITED STATES GEOLOGICAL SURVEY STANDARD CAP, STAMPED PTS NO 8 848

JY0845'AND RIVETED ON THE TOP OF A 3-1/2-INCH IRON PIPE.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0106 ***********************************************************************

JY0106  DESIGNATION -  Q 68

JY0106  PID         -  JY0106

JY0106  STATE/COUNTY-  OH/HOCKING

JY0106  USGS QUAD   -  LOGAN (1992)

JY0106

JY0106                         *CURRENT SURVEY CONTROL

JY0106  ___________________________________________________________________

JY0106* NAD 83(1986)-  39 33 56.     (N)    277 36 51      (E)     SCALED

JY0106* NAVD 88     -       226.954  (meters)     744.60   (feet)  POSTED

JY0106  ___________________________________________________________________

JY0106  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0106  DYNAMIC HT  -         226.820 (meters)     744.16  (feet)  COMP

JY0106  MODELED GRAV-     980,034.8   (mgal)                       NAVD 88

JY0106

JY0106  VERT ORDER  -  * POSTED, Code F , SEE BELOW

JY0106

JY0106.The horizontal coordinates were scaled from a topographic map and have

JY0106.an estimated accuracy of +/- 6 seconds.

JY0106

JY0106.The orthometric height was determined by differential leveling

JY0106.and adjusted by the National Geodetic Survey in 1992..

JY0106.* This is a POSTED BENCH MARK height.  Code F indicates a distribution

JY0106.rate greater than 8.0 mm/km.

JY0106

JY0106.The geoid height was determined by GEOID03.

JY0106

JY0106.The dynamic height is computed by dividing the NAVD 88

JY0106.geopotential number by the normal gravity value computed on the

JY0106.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0106.degrees latitude (g = 980.6199 gals.).

JY0106

JY0106.The modeled gravity was interpolated from observed gravity values.

JY0106

JY0106;                    North         East    Units  Estimated Accuracy

JY0106;SPC OH S     -   173,800.      609,810.      MT  (+/- 180 meters Scaled)

JY0106

JY0106                          SUPERSEDED SURVEY CONTROL

JY0106

JY0106  NGVD 29 (??/??/92)  227.137  (m)          745.20   (f) ADJ UNCH    2 0

JY0106

JY0106.Superseded values are not recommended for survey control.

JY0106.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0106.See file dsdata.txt to determine how the superseded data were derived.

JY0106

JY0106_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD809804(NAD 83)

JY0106_MARKER: DB = BENCH MARK DISK

JY0106_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0106_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0106_STAMPING: Q 68 1934

JY0106_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0106+STABILITY: SURFACE MOTION

JY0106

JY0106  HISTORY     - Date     Condition        Report By

JY0106  HISTORY     - 1934     MONUMENTED       CGS

JY0106  HISTORY     - 1960     MARK NOT FOUND   USGS

JY0106  HISTORY     - 19881227 MARK NOT FOUND   USPSQD

JY0106

JY0106                          STATION DESCRIPTION

JY0106

JY0106'DESCRIBED BY COAST AND GEODETIC SURVEY 1934

JY0106'2.4 MI NE FROM LOGAN.

JY0106'2.4 MILES NORTHEAST ALONG STATE HIGHWAY 75 FROM THE POST OFFICE

JY0106'AT LOGAN, HOCKING COUNTY, 75 YARDS NORTHWEST OF THE CENTER OF THE

JY0106'CROSSING OF COUNTY ROAD 15, 24 FEET NORTH OF THE CENTERLINE OF THE

JY0106'HIGHWAY, 4 FEET WEST OF A POLE, 3 FEET SOUTH OF THE RIGHT-OF-WAY

JY0106'FENCE, AND ABOUT 2 FEET LOWER THAN THE ROADWAY.  A STANDARD DISK,

JY0106'STAMPED Q 68 1934 AND SET IN THE TOP OF A CONCRETE POST.

JY0106

JY0106                          STATION RECOVERY (1960)

JY0106

JY0106'RECOVERY NOTE BY US GEOLOGICAL SURVEY 1960

JY0106'MARK NOT FOUND.

JY0106

JY0106                          STATION RECOVERY (1988)

JY0106

JY0106'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0106'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0544 ***********************************************************************

JY0544  DESIGNATION -  Q 69

JY0544  PID         -  JY0544

JY0544  STATE/COUNTY-  OH/HOCKING

JY0544  USGS QUAD   -  LAURELVILLE (1992)

JY0544

JY0544                         *CURRENT SURVEY CONTROL

JY0544  ___________________________________________________________________

JY0544* NAD 83(1986)-  39 29 49.     (N)    277 17 32      (E)     SCALED

JY0544* NAVD 88     -       236.707  (meters)     776.60   (feet)  ADJUSTED

JY0544  ___________________________________________________________________

JY0544  GEOID HEIGHT-         -34.05  (meters)                     GEOID03

JY0544  DYNAMIC HT  -         236.565 (meters)     776.13  (feet)  COMP

JY0544  MODELED GRAV-     980,020.7   (mgal)                       NAVD 88

JY0544

JY0544  VERT ORDER  -  SECOND    CLASS 0

JY0544

JY0544.The horizontal coordinates were scaled from a topographic map and have

JY0544.an estimated accuracy of +/- 6 seconds.

JY0544

JY0544.The orthometric height was determined by differential leveling

JY0544.and adjusted by the National Geodetic Survey in June 1991..

JY0544

JY0544.The geoid height was determined by GEOID03.

JY0544

JY0544.The dynamic height is computed by dividing the NAVD 88

JY0544.geopotential number by the normal gravity value computed on the

JY0544.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0544.degrees latitude (g = 980.6199 gals.).

JY0544

JY0544.The modeled gravity was interpolated from observed gravity values.

JY0544

JY0544;                    North         East    Units  Estimated Accuracy

JY0544;SPC OH S     -   166,200.      582,130.      MT  (+/- 180 meters Scaled)

JY0544

JY0544                          SUPERSEDED SURVEY CONTROL

JY0544

JY0544  NGVD 29 (??/??/92)  236.847  (m)          777.06   (f) ADJ UNCH    2 0

JY0544

JY0544.Superseded values are not recommended for survey control.

JY0544.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0544.See file dsdata.txt to determine how the superseded data were derived.

JY0544

JY0544_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD531733(NAD 83)

JY0544_MARKER: DB = BENCH MARK DISK

JY0544_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0544_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0544_STAMPING: Q 69 1934

JY0544_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0544+STABILITY: SURFACE MOTION

JY0544

JY0544  HISTORY     - Date     Condition        Report By

JY0544  HISTORY     - 1934     MONUMENTED       CGS

JY0544  HISTORY     - 1936     GOOD             NGS

JY0544  HISTORY     - 1960     MARK NOT FOUND   NGS

JY0544

JY0544                          STATION DESCRIPTION

JY0544

JY0544'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0544'2.1 MI W FROM SOUTH PERRY.

JY0544'2.1 MILES WEST ALONG STATE HIGHWAY 180 FROM THE POST OFFICE AT

JY0544'SOUTH PERRY, HOCKING COUNTY, 0.4 MILE WEST OF THE JUNCTION OF A

JY0544'GRAVEL ROAD LEADING NORTH, 480 YARDS WEST OF A CONCRETE BRIDGE

JY0544'OVER MIDDLE FORK CREEK, 30 YARDS EAST OF A CONCRETE CULVERT,

JY0544'15 YARDS EAST OF A 30-INCH WILLOW TREE, 7 YARDS NORTH OF THE

JY0544'CENTERLINE OF THE HIGHWAY AND 2 YARDS EAST OF THE EAST POINT OF A

JY0544'FIELD GATE.  A STANDARD DISK, STAMPED Q 69 1934 AND SET IN THE

JY0544'TOP OF A CONCRETE POST.

JY0544

JY0544                          STATION RECOVERY (1960)

JY0544

JY0544'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1960

JY0544'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0100 ***********************************************************************

JY0100  DESIGNATION -  R 68

JY0100  PID         -  JY0100

JY0100  STATE/COUNTY-  OH/HOCKING

JY0100  USGS QUAD   -  LOGAN (1992)

JY0100

JY0100                         *CURRENT SURVEY CONTROL

JY0100  ___________________________________________________________________

JY0100* NAD 83(1986)-  39 32 23.     (N)    277 35 29      (E)     SCALED

JY0100* NAVD 88     -       226.320  (meters)     742.52   (feet)  ADJUSTED

JY0100  ___________________________________________________________________

JY0100  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0100  DYNAMIC HT  -         226.187 (meters)     742.08  (feet)  COMP

JY0100  MODELED GRAV-     980,033.4   (mgal)                       NAVD 88

JY0100

JY0100  VERT ORDER  -  SECOND    CLASS 0

JY0100

JY0100.The horizontal coordinates were scaled from a topographic map and have

JY0100.an estimated accuracy of +/- 6 seconds.

JY0100

JY0100.The orthometric height was determined by differential leveling

JY0100.and adjusted by the National Geodetic Survey in June 1991..

JY0100

JY0100.The geoid height was determined by GEOID03.

JY0100

JY0100.The dynamic height is computed by dividing the NAVD 88

JY0100.geopotential number by the normal gravity value computed on the

JY0100.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0100.degrees latitude (g = 980.6199 gals.).

JY0100

JY0100.The modeled gravity was interpolated from observed gravity values.

JY0100

JY0100;                    North         East    Units  Estimated Accuracy

JY0100;SPC OH S     -   170,930.      607,860.      MT  (+/- 180 meters Scaled)

JY0100

JY0100                          SUPERSEDED SURVEY CONTROL

JY0100

JY0100  NGVD 29 (??/??/92)  226.443  (m)          742.92   (f) ADJ UNCH    2 0

JY0100

JY0100.Superseded values are not recommended for survey control.

JY0100.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0100.See file dsdata.txt to determine how the superseded data were derived.

JY0100

JY0100_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD789776(NAD 83)

JY0100_MARKER: DB = BENCH MARK DISK

JY0100_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0100_SP_SET: WALL

JY0100_STAMPING: R 68 1934

JY0100_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0100_SATELLITE: THE SITE LOCATION WAS REPORTED AS NOT SUITABLE FOR

JY0100+SATELLITE: SATELLITE OBSERVATIONS - December 27, 2005

JY0100

JY0100  HISTORY     - Date     Condition        Report By

JY0100  HISTORY     - 1934     MONUMENTED       CGS

JY0100  HISTORY     - 1936     GOOD             NGS

JY0100  HISTORY     - 20051227 GOOD             GEOCAC

JY0100

JY0100                          STATION DESCRIPTION

JY0100

JY0100'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0100'AT LOGAN.

JY0100'AT LOGAN, HOCKING COUNTY, 81 FEET SOUTHEAST OF THE CENTER OF THE

JY0100'CROSSING OF MAIN AND MARKET STREET, IN THE NORTH WALL OF THE

JY0100'COUNTY COURTHOUSE, 33 FEET WEST OF THE CENTERLINE OF THE NORTH

JY0100'DOOR, 2 FEET EAST OF THE NORTHWEST CORNER, 63 FEET SOUTH OF THE

JY0100'CENTERLINE OF MAIN STREET, 30 FEET SOUTH OF A FIRE HYDRANT, AND

JY0100'ABOUT 3 FEET ABOVE THE LEVEL OF THE SIDEWALK.  A STANDARD DISK,

JY0100'STAMPED R 68 1934 AND SET VERTICALLY.

JY0100

JY0100                          STATION RECOVERY (2005)

JY0100

JY0100'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0100'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0545 ***********************************************************************

JY0545  DESIGNATION -  R 69

JY0545  PID         -  JY0545

JY0545  STATE/COUNTY-  OH/HOCKING

JY0545  USGS QUAD   -  LAURELVILLE (1992)

JY0545

JY0545                         *CURRENT SURVEY CONTROL

JY0545  ___________________________________________________________________

JY0545* NAD 83(1986)-  39 29 05.     (N)    277 15 57      (E)     SCALED

JY0545* NAVD 88     -       232.269  (meters)     762.04   (feet)  ADJUSTED

JY0545  ___________________________________________________________________

JY0545  GEOID HEIGHT-         -34.04  (meters)                     GEOID03

JY0545  DYNAMIC HT  -         232.129 (meters)     761.58  (feet)  COMP

JY0545  MODELED GRAV-     980,019.3   (mgal)                       NAVD 88

JY0545

JY0545  VERT ORDER  -  SECOND    CLASS 0

JY0545

JY0545.The horizontal coordinates were scaled from a topographic map and have

JY0545.an estimated accuracy of +/- 6 seconds.

JY0545

JY0545.The orthometric height was determined by differential leveling

JY0545.and adjusted by the National Geodetic Survey in June 1991..

JY0545

JY0545.The geoid height was determined by GEOID03.

JY0545

JY0545.The dynamic height is computed by dividing the NAVD 88

JY0545.geopotential number by the normal gravity value computed on the

JY0545.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0545.degrees latitude (g = 980.6199 gals.).

JY0545

JY0545.The modeled gravity was interpolated from observed gravity values.

JY0545

JY0545;                    North         East    Units  Estimated Accuracy

JY0545;SPC OH S     -   164,850.      579,860.      MT  (+/- 180 meters Scaled)

JY0545

JY0545                          SUPERSEDED SURVEY CONTROL

JY0545

JY0545  NGVD 29 (??/??/92)  232.411  (m)          762.50   (f) ADJ UNCH    2 0

JY0545

JY0545.Superseded values are not recommended for survey control.

JY0545.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0545.See file dsdata.txt to determine how the superseded data were derived.

JY0545

JY0545_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD508720(NAD 83)

JY0545_MARKER: DB = BENCH MARK DISK

JY0545_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0545_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0545_STAMPING: R 69 1934

JY0545_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0545+STABILITY: SURFACE MOTION

JY0545

JY0545  HISTORY     - Date     Condition        Report By

JY0545  HISTORY     - 1934     MONUMENTED       CGS

JY0545  HISTORY     - 1936     GOOD             NGS

JY0545

JY0545                          STATION DESCRIPTION

JY0545

JY0545'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0545'1.3 MI NE FROM LAURELVILLE.

JY0545'1.3 MILES NORTHEAST ALONG STATE HIGHWAY 180 FROM THE POST

JY0545'OFFICE AT LAURELVILLE, HOCKING COUNTY, 114 YARDS SOUTH OF THE

JY0545'SOUTHEAST CORNER OF A BRICK HOUSE, 35 YARDS NORTHWEST OF THE

JY0545'NORTH POST OF A GUARDRAIL AT A SHARP CURVE, 12 YARDS WEST OF THE

JY0545'CENTERLINE OF THE HIGHWAY, 4 YARDS SOUTH OF THE WEST GATEPOST

JY0545'OF A PASTURE GATE, 2 FEET EAST OF THE FENCE, AND ABOUT 2 FEET

JY0545'ABOVE THE LEVEL OF THE HIGHWAY.  A STANDARD DISK, STAMPED R 69

JY0545'1934 AND SET IN THE TOP OF A CONCRETE POST.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1399 ***********************************************************************

JY1399  DESIGNATION -  ROCK HOUSE LOOKOUT TOWER

JY1399  PID         -  JY1399

JY1399  STATE/COUNTY-  OH/HOCKING

JY1399  USGS QUAD   -  SOUTH BLOOMINGVILLE (1990)

JY1399

JY1399                         *CURRENT SURVEY CONTROL

JY1399  ___________________________________________________________________

JY1399* NAD 83(1995)-  39 29 43.75609(N)    277 23 15.15260(E)     ADJUSTED

JY1399* NAVD 88     -

JY1399  ___________________________________________________________________

JY1399  LAPLACE CORR-          -0.43  (seconds)                    DEFLEC99

JY1399  GEOID HEIGHT-         -34.08  (meters)                     GEOID03

JY1399

JY1399  HORZ ORDER  -  THIRD

JY1399

JY1399.The horizontal coordinates were established by classical geodetic methods

JY1399.and adjusted by the National Geodetic Survey in April 1998..

JY1399

JY1399

JY1399.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1399

JY1399.The geoid height was determined by GEOID03.

JY1399

JY1399;                    North         East     Units Scale Factor Converg.

JY1399;SPC OH S     -   166,025.337   590,327.115   MT  0.99993778   -0 04 16.9

JY1399;SPC OH S     -   544,701.46  1,936,764.88   sFT  0.99993778   -0 04 16.9

JY1399;UTM  17      - 4,373,004.788   361,349.984   MT  0.99983669   -1 01 32.6

JY1399

JY1399                          SUPERSEDED SURVEY CONTROL

JY1399

JY1399  NAD 83(1986)-  39 29 43.76077(N)    277 23 15.14785(E) AD(       ) 4

JY1399  NAD 27      -  39 29 43.53200(N)    277 23 14.73100(E) AD(       ) 3

JY1399

JY1399.Superseded values are not recommended for survey control.

JY1399.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1399.See file dsdata.txt to determine how the superseded data were derived.

JY1399

JY1399_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD6135073005(NAD 83)

JY1399_MARKER: 57 = LOOKOUT TOWER

JY1399

JY1399  HISTORY     - Date     Condition        Report By

JY1399  HISTORY     - 1945     FIRST OBSERVED   CGS

JY1399

JY1399                          STATION DESCRIPTION

JY1399

JY1399'DESCRIBED BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1399'STATION IS LOCATED ABOUT 10 MILES SW OF ENTERPRISE IN ROCK HOUSE

JY1399'STATE PARK AND IN THE STATE FOREST RESERVE.  IT IS A STANDARD

JY1399'STATE LOOKOUT TOWER ABOUT 80 FEET HIGH SUPPORTED BY FOUR STEEL LEGS.

JY1399'IT IS PAINTED GREY AND HAS AN ENCLOSED CAB ON TOP.

JY1399'

JY1399'THE POINT INTERSECTED WAS THE APEX OF THE ROOF.

JY1399'

JY1399'TO REACH FROM THE JUNCTION OF STATE HIGHWAYS 180 AND 374 ABOUT 10

JY1399'MILES SW OF ENTERPRISE, GO E ON HIGHWAY 374 FOR 1.1 MILES TO SIGN

JY1399'ROCK HOUSE, TURN LEFT AND FOLLOW MAIN ROAD 0.25 MILE TO THE LOOKOUT

JY1399'TOWER.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0098 ***********************************************************************

JY0098  DESIGNATION -  RV 174

JY0098  PID         -  JY0098

JY0098  STATE/COUNTY-  OH/HOCKING

JY0098  USGS QUAD   -  LOGAN (1992)

JY0098

JY0098                         *CURRENT SURVEY CONTROL

JY0098  ___________________________________________________________________

JY0098* NAD 83(1986)-  39 31 46.     (N)    277 36 41      (E)     SCALED

JY0098* NAVD 88     -       219.501  (meters)     720.15   (feet)  ADJUSTED

JY0098  ___________________________________________________________________

JY0098  GEOID HEIGHT-         -34.11  (meters)                     GEOID03

JY0098  DYNAMIC HT  -         219.372 (meters)     719.72  (feet)  COMP

JY0098  MODELED GRAV-     980,032.6   (mgal)                       NAVD 88

JY0098

JY0098  VERT ORDER  -  SECOND    CLASS 0

JY0098

JY0098.The horizontal coordinates were scaled from a topographic map and have

JY0098.an estimated accuracy of +/- 6 seconds.

JY0098

JY0098.The orthometric height was determined by differential leveling

JY0098.and adjusted by the National Geodetic Survey in June 1991..

JY0098

JY0098.The geoid height was determined by GEOID03.

JY0098

JY0098.The dynamic height is computed by dividing the NAVD 88

JY0098.geopotential number by the normal gravity value computed on the

JY0098.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0098.degrees latitude (g = 980.6199 gals.).

JY0098

JY0098.The modeled gravity was interpolated from observed gravity values.

JY0098

JY0098;                    North         East    Units  Estimated Accuracy

JY0098;SPC OH S     -   169,800.      609,580.      MT  (+/- 180 meters Scaled)

JY0098

JY0098                          SUPERSEDED SURVEY CONTROL

JY0098

JY0098  NGVD 29 (??/??/92)  219.628  (m)          720.56   (f) ADJ UNCH    2 0

JY0098

JY0098.Superseded values are not recommended for survey control.

JY0098.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0098.See file dsdata.txt to determine how the superseded data were derived.

JY0098

JY0098_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD806764(NAD 83)

JY0098_MARKER: R = RIVET

JY0098_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0098_SP_SET: ABUTMENT

JY0098_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0098

JY0098  HISTORY     - Date     Condition        Report By

JY0098  HISTORY     - UNK      MONUMENTED       CORR

JY0098  HISTORY     - 1934     GOOD             NGS

JY0098  HISTORY     - 19881227 GOOD             USPSQD

JY0098

JY0098                          STATION DESCRIPTION

JY0098

JY0098'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0098'1.5 MI S FROM LOGAN.

JY0098'1.5 MILES SOUTH ALONG THE CHESAPEAKE AND OHIO RAILWAY FROM THE

JY0098'STATION AT LOGAN, HOCKING COUNTY, 0.2 MILE SOUTH OF MILEPOST

JY0098'C 51, 65 YARDS NORTH OF THE CENTER OF A ROAD CROSSING, AT THE

JY0098'WEST SIDE OF THE SOUTH ABUTMENT OF BRIDGE 51.2, IN THE TOP OF THE

JY0098'TOP COURSE OF MASONRY, ONE FOOT SOUTHEAST OF THE NORTHWEST CORNER,

JY0098'7.8 FEET WEST OF THE WEST RAIL, AND ABOUT ONE FOOT LOWER THAN THE

JY0098'TRACK.  A STANDARD MONEL-METAL RIVET.

JY0098

JY0098                          STATION RECOVERY (1988)

JY0098

JY0098'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0098'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0097 ***********************************************************************

JY0097  DESIGNATION -  RV 175

JY0097  PID         -  JY0097

JY0097  STATE/COUNTY-  OH/HOCKING

JY0097  USGS QUAD   -  LOGAN (1992)

JY0097

JY0097                         *CURRENT SURVEY CONTROL

JY0097  ___________________________________________________________________

JY0097* NAD 83(1986)-  39 31 20.     (N)    277 37 02      (E)     SCALED

JY0097* NAVD 88     -       221.523  (meters)     726.78   (feet)  ADJUSTED

JY0097  ___________________________________________________________________

JY0097  GEOID HEIGHT-         -34.11  (meters)                     GEOID03

JY0097  DYNAMIC HT  -         221.392 (meters)     726.35  (feet)  COMP

JY0097  MODELED GRAV-     980,031.9   (mgal)                       NAVD 88

JY0097

JY0097  VERT ORDER  -  SECOND    CLASS 0

JY0097

JY0097.The horizontal coordinates were scaled from a topographic map and have

JY0097.an estimated accuracy of +/- 6 seconds.

JY0097

JY0097.The orthometric height was determined by differential leveling

JY0097.and adjusted by the National Geodetic Survey in June 1991..

JY0097

JY0097.The geoid height was determined by GEOID03.

JY0097

JY0097.The dynamic height is computed by dividing the NAVD 88

JY0097.geopotential number by the normal gravity value computed on the

JY0097.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0097.degrees latitude (g = 980.6199 gals.).

JY0097

JY0097.The modeled gravity was interpolated from observed gravity values.

JY0097

JY0097;                    North         East    Units  Estimated Accuracy

JY0097;SPC OH S     -   168,990.      610,080.      MT  (+/- 180 meters Scaled)

JY0097

JY0097                          SUPERSEDED SURVEY CONTROL

JY0097

JY0097  NGVD 29 (??/??/92)  221.650  (m)          727.20   (f) ADJ UNCH    2 0

JY0097

JY0097.Superseded values are not recommended for survey control.

JY0097.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0097.See file dsdata.txt to determine how the superseded data were derived.

JY0097

JY0097_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD811756(NAD 83)

JY0097_MARKER: R = RIVET

JY0097_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0097_SP_SET: ABUTMENT

JY0097_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0097

JY0097  HISTORY     - Date     Condition        Report By

JY0097  HISTORY     - UNK      MONUMENTED       CORR

JY0097  HISTORY     - 1934     GOOD             NGS

JY0097  HISTORY     - 19881227 MARK NOT FOUND   USPSQD

JY0097

JY0097                          STATION DESCRIPTION

JY0097

JY0097'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0097'2.1 MI S FROM LOGAN.

JY0097'2.1 MILES SOUTH ALONG THE CHESAPEAKE AND OHIO RAILWAY FROM THE

JY0097'STATION AT LOGAN, HOCKING COUNTY, 0.2 MILE NORTH OF MILEPOST

JY0097'C 52, AT BRIDGE 51.81, IN THE TOP OF THE NORTHWEST ABUTMENT,

JY0097'1.5 FEET NORTHWEST OF THE NORTHWEST CORNER, ONE FOOT EAST OF THE

JY0097'WEST END, 11.5 FEET WEST OF THE WEST RAIL, AND ABOUT ONE FOOT

JY0097'LOWER THAN THE TRACK.  A STANDARD MONEL-METAL RIVET.

JY0097

JY0097                          STATION RECOVERY (1988)

JY0097

JY0097'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0097'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0096 ***********************************************************************

JY0096  DESIGNATION -  RV 176

JY0096  PID         -  JY0096

JY0096  STATE/COUNTY-  OH/HOCKING

JY0096  USGS QUAD   -  GORE (1992)

JY0096

JY0096                         *CURRENT SURVEY CONTROL

JY0096  ___________________________________________________________________

JY0096* NAD 83(1986)-  39 30 18.     (N)    277 38 00      (E)     SCALED

JY0096* NAVD 88     -       216.960  (meters)     711.81   (feet)  ADJUSTED

JY0096  ___________________________________________________________________

JY0096  GEOID HEIGHT-         -34.10  (meters)                     GEOID03

JY0096  DYNAMIC HT  -         216.832 (meters)     711.39  (feet)  COMP

JY0096  MODELED GRAV-     980,030.5   (mgal)                       NAVD 88

JY0096

JY0096  VERT ORDER  -  SECOND    CLASS 0

JY0096

JY0096.The horizontal coordinates were scaled from a topographic map and have

JY0096.an estimated accuracy of +/- 6 seconds.

JY0096

JY0096.The orthometric height was determined by differential leveling

JY0096.and adjusted by the National Geodetic Survey in June 1991..

JY0096

JY0096.The geoid height was determined by GEOID03.

JY0096

JY0096.The dynamic height is computed by dividing the NAVD 88

JY0096.geopotential number by the normal gravity value computed on the

JY0096.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0096.degrees latitude (g = 980.6199 gals.).

JY0096

JY0096.The modeled gravity was interpolated from observed gravity values.

JY0096

JY0096;                    North         East    Units  Estimated Accuracy

JY0096;SPC OH S     -   167,080.      611,470.      MT  (+/- 180 meters Scaled)

JY0096

JY0096                          SUPERSEDED SURVEY CONTROL

JY0096

JY0096  NGVD 29 (??/??/92)  217.090  (m)          712.24   (f) ADJ UNCH    2 0

JY0096

JY0096.Superseded values are not recommended for survey control.

JY0096.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0096.See file dsdata.txt to determine how the superseded data were derived.

JY0096

JY0096_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD825737(NAD 83)

JY0096_MARKER: R = RIVET

JY0096_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0096_SP_SET: BRIDGE MASONRY

JY0096_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0096

JY0096  HISTORY     - Date     Condition        Report By

JY0096  HISTORY     - UNK      MONUMENTED       CORR

JY0096  HISTORY     - 1934     GOOD             NGS

JY0096  HISTORY     - 19890105 MARK NOT FOUND   USPSQD

JY0096

JY0096                          STATION DESCRIPTION

JY0096

JY0096'DESCRIBED BY NATIONAL GEODETIC SURVEY 1934

JY0096'3.4 MI N FROM UNION FURNACE.

JY0096'3.4 MILES NORTH ALONG THE CHESAPEAKE AND OHIO RAILWAY FROM THE

JY0096'STATION AT UNION FURNACE, HOCKING COUNTY, 0.3 MILE SOUTH OF

JY0096'MILEPOST C 53, AT THE EAST SIDE OF BRIDGE 53.3, 15 FEET NORTHWEST

JY0096'OF THE SOUTHWEST CORNER OF THE SOUTH ABUTMENT, IN THE TOP OF THE

JY0096'TOP COURSE OF MASONRY, 9 FEET EAST OF THE EAST RAIL, AND ABOUT ONE

JY0096'FOOT LOWER THAN THE TRACK.  A STANDARD MONEL-METAL RIVET.

JY0096

JY0096                          STATION RECOVERY (1989)

JY0096

JY0096'RECOVERY NOTE BY US POWER SQUADRON 1989 (RCC)

JY0096'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0224 ***********************************************************************

JY0224  DESIGNATION -  RV 177

JY0224  PID         -  JY0224

JY0224  STATE/COUNTY-  OH/HOCKING

JY0224  USGS QUAD   -  UNION FURNACE (1995)

JY0224

JY0224                         *CURRENT SURVEY CONTROL

JY0224  ___________________________________________________________________

JY0224* NAD 83(1986)-  39 27 28.     (N)    277 38 32      (E)     SCALED

JY0224* NAVD 88     -       230.127  (meters)     755.01   (feet)  ADJUSTED

JY0224  ___________________________________________________________________

JY0224  GEOID HEIGHT-         -34.09  (meters)                     GEOID03

JY0224  DYNAMIC HT  -         229.989 (meters)     754.56  (feet)  COMP

JY0224  MODELED GRAV-     980,022.8   (mgal)                       NAVD 88

JY0224

JY0224  VERT ORDER  -  SECOND    CLASS 0

JY0224

JY0224.The horizontal coordinates were scaled from a topographic map and have

JY0224.an estimated accuracy of +/- 6 seconds.

JY0224

JY0224.The orthometric height was determined by differential leveling

JY0224.and adjusted by the National Geodetic Survey in June 1991..

JY0224

JY0224.The geoid height was determined by GEOID03.

JY0224

JY0224.The dynamic height is computed by dividing the NAVD 88

JY0224.geopotential number by the normal gravity value computed on the

JY0224.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0224.degrees latitude (g = 980.6199 gals.).

JY0224

JY0224.The modeled gravity was interpolated from observed gravity values.

JY0224

JY0224;                    North         East    Units  Estimated Accuracy

JY0224;SPC OH S     -   161,840.      612,240.      MT  (+/- 180 meters Scaled)

JY0224

JY0224                          SUPERSEDED SURVEY CONTROL

JY0224

JY0224  NGVD 29 (??/??/92)  230.260  (m)          755.44   (f) ADJ UNCH    2 0

JY0224

JY0224.Superseded values are not recommended for survey control.

JY0224.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0224.See file dsdata.txt to determine how the superseded data were derived.

JY0224

JY0224_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD831684(NAD 83)

JY0224_MARKER: R = RIVET

JY0224_SETTING: 30 = SET IN A LIGHT STRUCTURE

JY0224_SP_SET: CURB

JY0224_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY0224

JY0224  HISTORY     - Date     Condition        Report By

JY0224  HISTORY     - UNK      MONUMENTED       CORR

JY0224  HISTORY     - 1943     GOOD             NGS

JY0224  HISTORY     - 1976     GOOD             NGS

JY0224

JY0224                          STATION DESCRIPTION

JY0224

JY0224'DESCRIBED BY NATIONAL GEODETIC SURVEY 1943

JY0224'AT UNION FURNACE.

JY0224'AT UNION FURNACE, ON THE CHESAPEAKE AND OHIO RAILROAD, 16.2

JY0224'FEET WEST OF THE SOUTHWEST CORNER OF THE STATION, 75 FEET

JY0224'NORTH OF THE SOUTHWEST CORNER OF THE BRICK PLATFORM, 21 FEET

JY0224'SOUTHWEST OF THE CENTER OF THE WAITING-ROOM DOOR, IN THE TOP

JY0224'OF THE CURB, 3 FEET EAST OF THE EAST RAIL, AND ABOUT ONE FOOT

JY0224'ABOVE THE LEVEL OF THE TRACK.  A STANDARD MONEL-METAL RIVET.

JY0224

JY0224                          STATION RECOVERY (1976)

JY0224

JY0224'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1976

JY0224'NO RAILROAD STATION ABOUT 100 FT N OF N END OF BRIDGE NUMBER 56-7.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0222 ***********************************************************************

JY0222  DESIGNATION -  RV 178

JY0222  PID         -  JY0222

JY0222  STATE/COUNTY-  OH/HOCKING

JY0222  USGS QUAD   -  UNION FURNACE (1995)

JY0222

JY0222                         *CURRENT SURVEY CONTROL

JY0222  ___________________________________________________________________

JY0222* NAD 83(1986)-  39 25 49.     (N)    277 38 27      (E)     SCALED

JY0222* NAVD 88     -       271.450  (meters)     890.58   (feet)  ADJUSTED

JY0222  ___________________________________________________________________

JY0222  GEOID HEIGHT-         -34.08  (meters)                     GEOID03

JY0222  DYNAMIC HT  -         271.287 (meters)     890.05  (feet)  COMP

JY0222  MODELED GRAV-     980,019.6   (mgal)                       NAVD 88

JY0222

JY0222  VERT ORDER  -  SECOND    CLASS 0

JY0222

JY0222.The horizontal coordinates were scaled from a topographic map and have

JY0222.an estimated accuracy of +/- 6 seconds.

JY0222

JY0222.The orthometric height was determined by differential leveling

JY0222.and adjusted by the National Geodetic Survey in June 1991..

JY0222

JY0222.The geoid height was determined by GEOID03.

JY0222

JY0222.The dynamic height is computed by dividing the NAVD 88

JY0222.geopotential number by the normal gravity value computed on the

JY0222.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0222.degrees latitude (g = 980.6199 gals.).

JY0222

JY0222.The modeled gravity was interpolated from observed gravity values.

JY0222

JY0222;                    North         East    Units  Estimated Accuracy

JY0222;SPC OH S     -   158,790.      612,130.      MT  (+/- 180 meters Scaled)

JY0222

JY0222                          SUPERSEDED SURVEY CONTROL

JY0222

JY0222  NGVD 29 (??/??/92)  271.589  (m)          891.04   (f) ADJ UNCH    2 0

JY0222

JY0222.Superseded values are not recommended for survey control.

JY0222.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0222.See file dsdata.txt to determine how the superseded data were derived.

JY0222

JY0222_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD830654(NAD 83)

JY0222_MARKER: R = RIVET

JY0222_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0222_SP_SET: ABUTMENT

JY0222_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0222

JY0222  HISTORY     - Date     Condition        Report By

JY0222  HISTORY     - UNK      MONUMENTED       CORR

JY0222  HISTORY     - 1943     GOOD             NGS

JY0222  HISTORY     - 1976     GOOD             NGS

JY0222

JY0222                          STATION DESCRIPTION

JY0222

JY0222'DESCRIBED BY NATIONAL GEODETIC SURVEY 1943

JY0222'2.6 MI N FROM STARR.

JY0222'2.6 MILES NORTH ALONG THE CHESAPEAKE AND OHIO RAILWAY FROM THE

JY0222'STATION AT STARR, HOCKING COUNTY, AT A BRIDGE, IN THE TOP OF

JY0222'THE NORTHWEST ABUTMENT, 4 INCHES SOUTHEAST OF THE NORTHWEST

JY0222'CORNER, ONE FOOT WEST OF AN IRON BRACE, 33 FEET NORTHWEST OF A

JY0222'POLE, 7 FEET EAST OF THE EAST RAIL, AND ONE FOOT LOWER THAN THE

JY0222'TRACK.  A STANDARD MONEL-METAL RIVET.

JY0222

JY0222                          STATION RECOVERY (1976)

JY0222

JY0222'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1976

JY0222'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0221 ***********************************************************************

JY0221  DESIGNATION -  RV 179

JY0221  PID         -  JY0221

JY0221  STATE/COUNTY-  OH/HOCKING

JY0221  USGS QUAD   -  UNION FURNACE (1995)

JY0221

JY0221                         *CURRENT SURVEY CONTROL

JY0221  ___________________________________________________________________

JY0221* NAD 83(1986)-  39 24 17.     (N)    277 38 29      (E)     SCALED

JY0221* NAVD 88     -       241.721  (meters)     793.05   (feet)  ADJUSTED

JY0221  ___________________________________________________________________

JY0221  GEOID HEIGHT-         -34.07  (meters)                     GEOID03

JY0221  DYNAMIC HT  -         241.575 (meters)     792.57  (feet)  COMP

JY0221  MODELED GRAV-     980,016.7   (mgal)                       NAVD 88

JY0221

JY0221  VERT ORDER  -  SECOND    CLASS 0

JY0221

JY0221.The horizontal coordinates were scaled from a topographic map and have

JY0221.an estimated accuracy of +/- 6 seconds.

JY0221

JY0221.The orthometric height was determined by differential leveling

JY0221.and adjusted by the National Geodetic Survey in June 1991..

JY0221

JY0221.The geoid height was determined by GEOID03.

JY0221

JY0221.The dynamic height is computed by dividing the NAVD 88

JY0221.geopotential number by the normal gravity value computed on the

JY0221.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0221.degrees latitude (g = 980.6199 gals.).

JY0221

JY0221.The modeled gravity was interpolated from observed gravity values.

JY0221

JY0221;                    North         East    Units  Estimated Accuracy

JY0221;SPC OH S     -   155,950.      612,180.      MT  (+/- 180 meters Scaled)

JY0221

JY0221                          SUPERSEDED SURVEY CONTROL

JY0221

JY0221  NGVD 29 (??/??/92)  241.862  (m)          793.51   (f) ADJ UNCH    2 0

JY0221

JY0221.Superseded values are not recommended for survey control.

JY0221.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0221.See file dsdata.txt to determine how the superseded data were derived.

JY0221

JY0221_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD830625(NAD 83)

JY0221_MARKER: R = RIVET

JY0221_SETTING: 36 = SET IN A MASSIVE STRUCTURE

JY0221_SP_SET: BRIDGE

JY0221_STABILITY: B = PROBABLY HOLD POSITION/ELEVATION WELL

JY0221

JY0221  HISTORY     - Date     Condition        Report By

JY0221  HISTORY     - UNK      MONUMENTED       CORR

JY0221  HISTORY     - 1943     GOOD             NGS

JY0221  HISTORY     - 1976     GOOD             NGS

JY0221

JY0221                          STATION DESCRIPTION

JY0221

JY0221'DESCRIBED BY NATIONAL GEODETIC SURVEY 1943

JY0221'0.6 MI N FROM STARR.

JY0221'0.6 MILE NORTH ALONG THE CHESAPEAKE AND OHIO RAILWAY FROM THE

JY0221'STATION AT STARR, HOCKING COUNTY, AT THE WEST SIDE OF THE EAST

JY0221'WING WALL OF BRIDGE 60.6, IN THE TOP OF THE THIRD COURSE OF

JY0221'MASONRY, ONE FOOT EAST OF THE NORTHWEST CORNER, 12 FEET WEST

JY0221'OF THE WEST RAIL, AND ABOUT 3 FEET LOWER THAN THE TRACK.  A

JY0221'STANDARD MONEL-METAL RIVET.

JY0221

JY0221                          STATION RECOVERY (1976)

JY0221

JY0221'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1976

JY0221'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0101 ***********************************************************************

JY0101  DESIGNATION -  S 68

JY0101  PID         -  JY0101

JY0101  STATE/COUNTY-  OH/HOCKING

JY0101  USGS QUAD   -  LOGAN (1992)

JY0101

JY0101                         *CURRENT SURVEY CONTROL

JY0101  ___________________________________________________________________

JY0101* NAD 83(1986)-  39 32 27.     (N)    277 35 29      (E)     SCALED

JY0101* NAVD 88     -       226.684  (meters)     743.71   (feet)  ADJUSTED

JY0101  ___________________________________________________________________

JY0101  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0101  DYNAMIC HT  -         226.550 (meters)     743.27  (feet)  COMP

JY0101  MODELED GRAV-     980,033.6   (mgal)                       NAVD 88

JY0101

JY0101  VERT ORDER  -  SECOND    CLASS 0

JY0101

JY0101.The horizontal coordinates were scaled from a topographic map and have

JY0101.an estimated accuracy of +/- 6 seconds.

JY0101

JY0101.The orthometric height was determined by differential leveling

JY0101.and adjusted by the National Geodetic Survey in June 1991..

JY0101

JY0101.The geoid height was determined by GEOID03.

JY0101

JY0101.The dynamic height is computed by dividing the NAVD 88

JY0101.geopotential number by the normal gravity value computed on the

JY0101.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0101.degrees latitude (g = 980.6199 gals.).

JY0101

JY0101.The modeled gravity was interpolated from observed gravity values.

JY0101

JY0101;                    North         East    Units  Estimated Accuracy

JY0101;SPC OH S     -   171,060.      607,860.      MT  (+/- 180 meters Scaled)

JY0101

JY0101                          SUPERSEDED SURVEY CONTROL

JY0101

JY0101  NGVD 29 (??/??/92)  226.806  (m)          744.11   (f) ADJ UNCH    2 0

JY0101

JY0101.Superseded values are not recommended for survey control.

JY0101.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0101.See file dsdata.txt to determine how the superseded data were derived.

JY0101

JY0101_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD789777(NAD 83)

JY0101_MARKER: DB = BENCH MARK DISK

JY0101_SETTING: 30 = SET IN A LIGHT STRUCTURE

JY0101_SP_SET: BALUSTRADE

JY0101_STAMPING: S 68 1934

JY0101_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY0101_SATELLITE: THE SITE LOCATION WAS REPORTED AS NOT SUITABLE FOR

JY0101+SATELLITE: SATELLITE OBSERVATIONS - December 27, 2005

JY0101

JY0101  HISTORY     - Date     Condition        Report By

JY0101  HISTORY     - 1934     MONUMENTED       CGS

JY0101  HISTORY     - 1936     GOOD             NGS

JY0101  HISTORY     - 19881227 GOOD             USPSQD

JY0101  HISTORY     - 20051227 GOOD             GEOCAC

JY0101

JY0101                          STATION DESCRIPTION

JY0101

JY0101'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0101'AT LOGAN.

JY0101'AT LOGAN, HOCKING COUNTY, 85 FEET EAST OF THE CENTERLINE ON MARKET

JY0101'STREET, AT THE POST OFFICE, IN THE TOP OF THE NORTH CONCRETE

JY0101'BALUSTRADE OF THE STEPS TO THE WEST ENTRANCE, 40 FEET SOUTH OF

JY0101'THE NORTHWEST CORNER OF THE BUILDING, 11.5 FEET NORTH OF THE CENTER

JY0101'OF THE WEST ENTRANCE, 3.8 FEET WEST OF THE WEST WALL, AND ABOUT

JY0101'4 FEET ABOVE GROUND.  A STANDARD DISK, STAMPED S 68 1934.

JY0101

JY0101                          STATION RECOVERY (1988)

JY0101

JY0101'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0101'RECOVERED IN GOOD CONDITION.

JY0101

JY0101                          STATION RECOVERY (2005)

JY0101

JY0101'RECOVERY NOTE BY GEOCACHING 2005 (RCF)

JY0101'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0546 ***********************************************************************

JY0546  DESIGNATION -  S 69

JY0546  PID         -  JY0546

JY0546  STATE/COUNTY-  OH/HOCKING

JY0546  USGS QUAD   -  LAURELVILLE (1992)

JY0546

JY0546                         *CURRENT SURVEY CONTROL

JY0546  ___________________________________________________________________

JY0546* NAD 83(1986)-  39 28 29.     (N)    277 15 39      (E)     SCALED

JY0546* NAVD 88     -       225.633  (meters)     740.26   (feet)  ADJUSTED

JY0546  ___________________________________________________________________

JY0546  GEOID HEIGHT-         -34.04  (meters)                     GEOID03

JY0546  DYNAMIC HT  -         225.497 (meters)     739.82  (feet)  COMP

JY0546  MODELED GRAV-     980,018.7   (mgal)                       NAVD 88

JY0546

JY0546  VERT ORDER  -  SECOND    CLASS 0

JY0546

JY0546.The horizontal coordinates were scaled from a topographic map and have

JY0546.an estimated accuracy of +/- 6 seconds.

JY0546

JY0546.The orthometric height was determined by differential leveling

JY0546.and adjusted by the National Geodetic Survey in June 1991..

JY0546

JY0546.The geoid height was determined by GEOID03.

JY0546

JY0546.The dynamic height is computed by dividing the NAVD 88

JY0546.geopotential number by the normal gravity value computed on the

JY0546.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0546.degrees latitude (g = 980.6199 gals.).

JY0546

JY0546.The modeled gravity was interpolated from observed gravity values.

JY0546

JY0546;                    North         East    Units  Estimated Accuracy

JY0546;SPC OH S     -   163,740.      579,420.      MT  (+/- 180 meters Scaled)

JY0546

JY0546                          SUPERSEDED SURVEY CONTROL

JY0546

JY0546  NGVD 29 (??/??/92)  225.776  (m)          740.73   (f) ADJ UNCH    2 0

JY0546

JY0546.Superseded values are not recommended for survey control.

JY0546.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0546.See file dsdata.txt to determine how the superseded data were derived.

JY0546

JY0546_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD504709(NAD 83)

JY0546_MARKER: DB = BENCH MARK DISK

JY0546_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0546_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0546_STAMPING: S 69 1934

JY0546_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0546+STABILITY: SURFACE MOTION

JY0546

JY0546  HISTORY     - Date     Condition        Report By

JY0546  HISTORY     - 1934     MONUMENTED       CGS

JY0546  HISTORY     - 1936     GOOD             NGS

JY0546  HISTORY     - 1960     MARK NOT FOUND   NGS

JY0546

JY0546                          STATION DESCRIPTION

JY0546

JY0546'DESCRIBED BY NATIONAL GEODETIC SURVEY 1936

JY0546'AT LAURELVILLE.

JY0546'AT LAURELVILLE, HOCKING COUNTY, ON STATE HIGHWAY 180, 170

JY0546'YARDS NORTH OF THE INTERSECTION OF MAIN STREET, 23 YARDS EAST

JY0546'OF THE SOUTHEAST CORNER OF A HOUSE WITH A STONE PORCH, 27 FEET

JY0546'WEST OF THE CENTERLINE OF THE HIGHWAY, ONE YARDS EAST OF A POLE,

JY0546'AND LEVEL WITH THE HIGHWAY.  A STANDARD DISK, STAMPED S 69 1934

JY0546'AND SET IN THE TOP OF A CONCRETE POST.

JY0546

JY0546                          STATION RECOVERY (1960)

JY0546

JY0546'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1960

JY0546'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1333 ***********************************************************************

JY1333  DESIGNATION -  SCHULTZ

JY1333  PID         -  JY1333

JY1333  STATE/COUNTY-  OH/HOCKING

JY1333  USGS QUAD   -  LOGAN (1992)

JY1333

JY1333                         *CURRENT SURVEY CONTROL

JY1333  ___________________________________________________________________

JY1333* NAD 83(1995)-  39 30 23.74896(N)    277 35 02.12611(E)     ADJUSTED

JY1333* NAVD 88     -       336.     (meters)    1102.     (feet)  SCALED

JY1333  ___________________________________________________________________

JY1333  LAPLACE CORR-           0.21  (seconds)                    DEFLEC99

JY1333  GEOID HEIGHT-         -34.11  (meters)                     GEOID03

JY1333

JY1333  HORZ ORDER  -  SECOND

JY1333

JY1333.The horizontal coordinates were established by classical geodetic methods

JY1333.and adjusted by the National Geodetic Survey in April 1998..

JY1333

JY1333.The orthometric height was scaled from a topographic map.

JY1333

JY1333.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1333

JY1333.The geoid height was determined by GEOID03.

JY1333

JY1333;                    North         East     Units Scale Factor Converg.

JY1333;SPC OH S     -   167,255.983   607,217.454   MT  0.99993818   +0 03 11.7

JY1333;SPC OH S     -   548,739.00  1,992,179.26   sFT  0.99993818   +0 03 11.7

JY1333;UTM  17      - 4,373,953.835   378,256.295   MT  0.99978248   -0 54 03.5

JY1333

JY1333!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1333!SPC OH S     -   0.99995266  x   0.99993818  =   0.99989084

JY1333!UTM  17      -   0.99995266  x   0.99978248  =   0.99973515

JY1333

JY1333:                Primary Azimuth Mark                     Grid Az

JY1333:SPC OH S     -  COOK RESET                               057 38 21.8

JY1333:UTM  17      -  COOK RESET                               058 35 37.0

JY1333

JY1333|---------------------------------------------------------------------|

JY1333| PID    Reference Object                     Distance      Geod. Az  |

JY1333|                                                           dddmmss.s |

JY1333| JY1331 COOK RESET                          APPROX. 7.2 KM 0574133.5 |

JY1333|---------------------------------------------------------------------|

JY1333

JY1333                          SUPERSEDED SURVEY CONTROL

JY1333

JY1333  NAD 83(1986)-  39 30 23.75207(N)    277 35 02.12190(E) AD(       ) 2

JY1333  NAD 27      -  39 30 23.51300(N)    277 35 01.67800(E) AD(       ) 2

JY1333

JY1333.Superseded values are not recommended for survey control.

JY1333.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1333.See file dsdata.txt to determine how the superseded data were derived.

JY1333

JY1333_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7825673954(NAD 83)

JY1333_MARKER: V = STONE MONUMENT

JY1333_SETTING: 0 = UNSPECIFIED SETTING

JY1333_MARK LOGO: USGS

JY1333_STABILITY: D = MARK OF QUESTIONABLE OR UNKNOWN STABILITY

JY1333

JY1333  HISTORY     - Date     Condition        Report By

JY1333  HISTORY     - 1885     MONUMENTED       CGS

JY1333

JY1333                          STATION DESCRIPTION

JY1333

JY1333'DESCRIBED BY COAST AND GEODETIC SURVEY 1885 (RSD)

JY1333'SCHULTZ IS ON THE ESTATE OF DANIEL SCHULTZ IN THE W 1/2 OF THE NE 1/4

JY1333'OF SEC. 26, FALLS TOWNSHIP, HOCKING COUNTY, OHIO.  OBSERVING SCAFFOLD

JY1333'AND TRIPOD WERE ERECTED IN 1881.  THE STATION WAS OCCUPIED WITH

JY1333'THEODOLITE IN 1885, BUT MEASUREMENTS OF THE ANGLES ARE NOT COMPLETED

JY1333'(AT CLOSE OF SEASON, 1885).  HEIGHT OF TOP OF CAP-BLOCK OF TRIPOD

JY1333'ABOVE TOP OF SURFACE OF STATION MARK 13 FEET, 10 INCHES.

JY1333'

JY1333'BEARINGS (WITH STATION TRANSIT)--

JY1333'

JY1333'FROM STATION TO WITNESS POST NO. 1 N 63 DEGREES 35 MINUTES E

JY1333'

JY1333'FROM STATION TO WITNESS POST NO. 2 (APPLE TREE INTERVENES)

JY1333'

JY1333'FROM STATION TO WITNESS POST NO. 3 N 66 DEGREES 25 MINUTES W

JY1333'

JY1333'FROM STATION TO WITNESS POST C N 67 DEGREES 55 MINUTES E

JY1333'

JY1333'DISTANCES (WITH STEEL TAPE)

JY1333'

JY1333'FROM STATION TO WITNESS POST NO. 1 62 FEET

JY1333'

JY1333'FROM STATION TO WITNESS POST NO. 2 72 FEET 11 INCHES

JY1333'

JY1333'FROM STATION TO WITNESS POST NO. 3 72 FEET

JY1333'

JY1333'FROM STATION TO C (WALNUT TREE 6 INCHES DIAMETER) 69 FEET 1 INCH

JY1333'

JY1333'FROM STATION TO A (FENCE N OF ORCHARD) 29 FEET

JY1333'

JY1333'FROM A TO B 25 FEET 3 INCHES

JY1333'

JY1333'FROM A TO WITNESS POST NO. 1 55 FEET 6 INCHES

JY1333'

JY1333'FROM A TO WITNESS POST NO. 3 66 FEET 2 INCHES

JY1333'

JY1333'FROM B TO WITNESS POST NO. 2 98 FEET

JY1333'

JY1333'MARKS--

JY1333'

JY1333'THE SURFACE STATION MARK IS A MARBLE POST 6 INCHES SQUARE AND ABOUT

JY1333'30 INCHES LONG.  THE WITNESS POSTS ARE MARBLE POSTS 4 INCHES

JY1333'SQUARE AND ABOUT 30 INCHES LONG.  THE UNDERGROUND STATION MARK IS AN

JY1333'EARTHEN PYRAMID, THE APEX BEING ABOUT 30 INCHES BELOW THE

JY1333'SURFACE OF THE GROUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1323 ***********************************************************************

JY1323  DESIGNATION -  SHAWNEE LOOKOUT TOWER

JY1323  PID         -  JY1323

JY1323  STATE/COUNTY-  OH/HOCKING

JY1323  USGS QUAD   -  NEW STRAITSVILLE (1995)

JY1323

JY1323                         *CURRENT SURVEY CONTROL

JY1323  ___________________________________________________________________

JY1323* NAD 83(1995)-  39 32 54.69747(N)    277 46 22.09764(E)     ADJUSTED

JY1323* NAVD 88     -

JY1323  ___________________________________________________________________

JY1323  LAPLACE CORR-           0.53  (seconds)                    DEFLEC99

JY1323  GEOID HEIGHT-         -34.05  (meters)                     GEOID03

JY1323

JY1323  HORZ ORDER  -  THIRD

JY1323

JY1323.The horizontal coordinates were established by classical geodetic methods

JY1323.and adjusted by the National Geodetic Survey in April 1998..

JY1323

JY1323

JY1323.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1323

JY1323.The geoid height was determined by GEOID03.

JY1323

JY1323;                    North         East     Units Scale Factor Converg.

JY1323;SPC OH S     -   171,943.082   623,447.114   MT  0.99994000   +0 10 23.2

JY1323;SPC OH S     -   564,116.59  2,045,426.07   sFT  0.99994000   +0 10 23.2

JY1323;UTM  17      - 4,378,369.154   394,558.842   MT  0.99973688   -0 46 53.3

JY1323

JY1323                          SUPERSEDED SURVEY CONTROL

JY1323

JY1323  NAD 83(1986)-  39 32 54.69930(N)    277 46 22.09017(E) AD(       ) 4

JY1323  NAD 27      -  39 32 54.44900(N)    277 46 21.63700(E) AD(       ) 3

JY1323

JY1323.Superseded values are not recommended for survey control.

JY1323.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1323.See file dsdata.txt to determine how the superseded data were derived.

JY1323

JY1323_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD9455978369(NAD 83)

JY1323_MARKER: 57 = LOOKOUT TOWER

JY1323

JY1323  HISTORY     - Date     Condition        Report By

JY1323  HISTORY     - 1959     FIRST OBSERVED   CGS

JY1323

JY1323                          STATION DESCRIPTION

JY1323

JY1323'DESCRIBED BY COAST AND GEODETIC SURVEY 1959 (WER)

JY1323'THE STATION IS THE TOP CENTER OF A LOOKOUT TOWER WHICH IS LOCATED

JY1323'ABOUT 2-1/2 MILES SOUTH OF NEW STRAITSVILLE, 5 MILES NORTHWEST OF

JY1323'MURRAY CITY, ON THE SUMMIT OF A TIMBERED RIDGE WHICH IS LOCALLY KNOWN

JY1323'AS SAND RUN HILL.

JY1323'

JY1323'THE TOWER IS A FOUR-LEGGED STEEL STRUCTURE WITH A METAL CABIN ATOP

JY1323'AND ABOUT 110 FEET IN HEIGHT.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0099 ***********************************************************************

JY0099  DESIGNATION -  T 68

JY0099  PID         -  JY0099

JY0099  STATE/COUNTY-  OH/HOCKING

JY0099  USGS QUAD   -  LOGAN (1992)

JY0099

JY0099                         *CURRENT SURVEY CONTROL

JY0099  ___________________________________________________________________

JY0099* NAD 83(1986)-  39 32 14.     (N)    277 34 59      (E)     SCALED

JY0099* NAVD 88     -       222.225  (meters)     729.08   (feet)  ADJUSTED

JY0099  ___________________________________________________________________

JY0099  GEOID HEIGHT-         -34.12  (meters)                     GEOID03

JY0099  DYNAMIC HT  -         222.094 (meters)     728.65  (feet)  COMP

JY0099  MODELED GRAV-     980,031.9   (mgal)                       NAVD 88

JY0099

JY0099  VERT ORDER  -  SECOND    CLASS 0

JY0099

JY0099.The horizontal coordinates were scaled from a topographic map and have

JY0099.an estimated accuracy of +/- 6 seconds.

JY0099

JY0099.The orthometric height was determined by differential leveling

JY0099.and adjusted by the National Geodetic Survey in June 1991..

JY0099

JY0099.The geoid height was determined by GEOID03.

JY0099

JY0099.The dynamic height is computed by dividing the NAVD 88

JY0099.geopotential number by the normal gravity value computed on the

JY0099.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0099.degrees latitude (g = 980.6199 gals.).

JY0099

JY0099.The modeled gravity was interpolated from observed gravity values.

JY0099

JY0099;                    North         East    Units  Estimated Accuracy

JY0099;SPC OH S     -   170,660.      607,140.      MT  (+/- 180 meters Scaled)

JY0099

JY0099                          SUPERSEDED SURVEY CONTROL

JY0099

JY0099  NGVD 29 (??/??/92)  222.348  (m)          729.49   (f) ADJ UNCH    2 0

JY0099

JY0099.Superseded values are not recommended for survey control.

JY0099.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0099.See file dsdata.txt to determine how the superseded data were derived.

JY0099

JY0099_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD782773(NAD 83)

JY0099_MARKER: DB = BENCH MARK DISK

JY0099_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0099_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0099_STAMPING: T 68 1934

JY0099_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0099+STABILITY: SURFACE MOTION

JY0099

JY0099  HISTORY     - Date     Condition        Report By

JY0099  HISTORY     - 1934     MONUMENTED       CGS

JY0099  HISTORY     - 1960     MARK NOT FOUND   USGS

JY0099  HISTORY     - 19881227 MARK NOT FOUND   USPSQD

JY0099

JY0099                          STATION DESCRIPTION

JY0099

JY0099'DESCRIBED BY COAST AND GEODETIC SURVEY 1934

JY0099'AT LOGAN.

JY0099'AT LOGAN, HOCKING COUNTY, 108 FEET EAST OF THE NORTHEAST CORNER

JY0099'OF THE CHESAPEAKE AND OHIO RAILWAY STATION, 78 FEET EAST OF THE

JY0099'PLATFORM CURB, 69 FEET NORTHEAST OF THE NORTHEAST CORNER OF THE

JY0099'FREIGHT HOUSE, 102 FEET EAST OF THE EAST RAIL, AND ABOUT ONE

JY0099'FOOT ABOVE THE LEVEL OF THE TRACK.  A STANDARD DISK, STAMPED

JY0099'T 68 1934 AND SET IN THE TOP OF A CONCRETE POST.

JY0099

JY0099                          STATION RECOVERY (1960)

JY0099

JY0099'RECOVERY NOTE BY US GEOLOGICAL SURVEY 1960

JY0099'MARK NOT FOUND.

JY0099

JY0099                          STATION RECOVERY (1988)

JY0099

JY0099'RECOVERY NOTE BY US POWER SQUADRON 1988 (RCC)

JY0099'MARK NOT FOUND.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY1332 ***********************************************************************

JY1332  DESIGNATION -  TABOR

JY1332  PID         -  JY1332

JY1332  STATE/COUNTY-  OH/HOCKING

JY1332  USGS QUAD   -  LOGAN (1992)

JY1332

JY1332                         *CURRENT SURVEY CONTROL

JY1332  ___________________________________________________________________

JY1332* NAD 83(1995)-  39 35 56.32457(N)    277 34 21.52193(E)     ADJUSTED

JY1332* NAVD 88     -       331.54   (+/-2cm)    1087.7    (feet)  VERTCON

JY1332  ___________________________________________________________________

JY1332  X           -     648,562.488 (meters)                     COMP

JY1332  Y           -  -4,878,515.760 (meters)                     COMP

JY1332  Z           -   4,043,966.635 (meters)                     COMP

JY1332  LAPLACE CORR-           0.50  (seconds)                    DEFLEC99

JY1332  ELLIP HEIGHT-         297.43  (meters)          (04/01/98) GPS OBS

JY1332  GEOID HEIGHT-         -34.14  (meters)                     GEOID03

JY1332

JY1332  HORZ ORDER  -  FIRST

JY1332  VERT ORDER  -  THIRD ? (See Below)

JY1332  ELLP ORDER  -  FOURTH    CLASS I

JY1332

JY1332.The horizontal coordinates were established by classical geodetic methods

JY1332.and adjusted by the National Geodetic Survey in April 1998..

JY1332

JY1332.The NAVD 88 height was computed by applying the VERTCON shift value to

JY1332.the NGVD 29 height (displayed under SUPERSEDED SURVEY CONTROL.)

JY1332.The vertical order pertains to the NGVD 29 superseded value.

JY1332

JY1332.The X, Y, and Z were computed from the position and the ellipsoidal ht.

JY1332

JY1332.The Laplace correction was computed from DEFLEC99 derived deflections.

JY1332

JY1332.The ellipsoidal height was determined by GPS observations

JY1332.and is referenced to NAD 83.

JY1332

JY1332.The geoid height was determined by GEOID03.

JY1332

JY1332;                    North         East     Units Scale Factor Converg.

JY1332;SPC OH S     -   177,511.348   606,239.214   MT  0.99994290   +0 02 45.9

JY1332;SPC OH S     -   582,385.15  1,988,969.82   sFT  0.99994290   +0 02 45.9

JY1332;UTM  17      - 4,384,222.473   377,449.267   MT  0.99978491   -0 54 35.7

JY1332

JY1332!             -  Elev Factor  x  Scale Factor =   Combined Factor

JY1332!SPC OH S     -   0.99995334  x   0.99994290  =   0.99989624

JY1332!UTM  17      -   0.99995334  x   0.99978491  =   0.99973826

JY1332

JY1332|---------------------------------------------------------------------|

JY1332| PID    Reference Object                     Distance      Geod. Az  |

JY1332|                                                           dddmmss.s |

JY1332| CE7435 TABOR RM 1                           14.339 METERS 10249     |

JY1332| CE7436 TABOR RM 2                           23.999 METERS 18722     |

JY1332| CE7434 TABOR AZ MK                                        2043348.5 |

JY1332| JY1426 SUGAR GROVE LOOKOUT TOWER           APPROX. 6.9 KM 2752025.0 |

JY1332|---------------------------------------------------------------------|

JY1332

JY1332                          SUPERSEDED SURVEY CONTROL

JY1332

JY1332  NAD 83(1986)-  39 35 56.32800(N)    277 34 21.51795(E) AD(       ) 1

JY1332  NAD 27      -  39 35 56.09600(N)    277 34 21.07500(E) AD(       ) 1

JY1332  NGVD 29 (07/19/86)  331.73   (m)         1088.4    (f) LEVELING    3

JY1332

JY1332.Superseded values are not recommended for survey control.

JY1332.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY1332.See file dsdata.txt to determine how the superseded data were derived.

JY1332

JY1332_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD7744984222(NAD 83)

JY1332_MARKER: DD = SURVEY DISK

JY1332_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY1332_SP_SET: CONCRETE POST

JY1332_STAMPING: TABOR 1945

JY1332_MARK LOGO: CGS

JY1332_MAGNETIC: N = NO MAGNETIC MATERIAL

JY1332_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY1332+STABILITY: SURFACE MOTION

JY1332_SATELLITE: THE SITE LOCATION WAS REPORTED AS SUITABLE FOR

JY1332+SATELLITE: SATELLITE OBSERVATIONS - July 28, 1987

JY1332

JY1332  HISTORY     - Date     Condition        Report By

JY1332  HISTORY     - 1945     MONUMENTED       CGS

JY1332  HISTORY     - 1945     GOOD             CGS

JY1332  HISTORY     - 1960     MARK NOT FOUND   USGS

JY1332  HISTORY     - 1986     GOOD             NGS

JY1332  HISTORY     - 19870728 GOOD

JY1332

JY1332                          STATION DESCRIPTION

JY1332

JY1332'DESCRIBED BY COAST AND GEODETIC SURVEY 1945 (JCP)

JY1332'STATION IS LOCATED ABOUT 4-1/2 MILES NW OF LOGAN, 3 MILES NE OF

JY1332'ENTERPRISE, ON THE E SIDE OF STATE HIGHWAY 664, AND ON LAND OWNED

JY1332'BY RAYMOND PITTMAN.  IT IS 11.5 FEET NW OF POWERLINE GUY POLE, 30

JY1332'FEET E OF THE APPROXIMATE CENTER OF STATE HIGHWAY 664, AND 68

JY1332'FEET W OF A TWIN CHERRY TREE.  IT IS WITNESSED BY A WHITE POST.

JY1332'

JY1332'STATION MARK IS A BRONZE STATION DISK SET IN TOP OF A CONCRETE POST.

JY1332'IT IS STAMPED TABOR 1945 AND IS 8 INCHES BELOW THE SURFACE OF THE

JY1332'GROUND.  UNDERGROUND MARK IS A BRONZE STATION DISK SET IN CONCRETE.

JY1332'

JY1332'REFERENCE MARK 1 IS 47.03 FEET E OF THE STATION AND 3 FEET N OF AN

JY1332'E-W FENCELINE.  IT IS A BRONZE REFERENCE DISK SET IN TOP OF A

JY1332'CONCRETE POST.  IT IS STAMPED TABOR NO 1 1945 AND PROJECTS 6 INCHES.

JY1332'

JY1332'REFERENCE MARK 2 IS 78.73 FEET N OF THE STATION AND 25 FEET E OF THE

JY1332'APPROXIMATE CENTER OF THE ROAD.  IT IS A BRONZE REFERENCE DISK

JY1332'SET IN TOP OF A CONCRETE POST.  IT IS STAMPED TABOR NO 2 1945 AND

JY1332'PROJECTS 6 INCHES.

JY1332'

JY1332'AZIMUTH MARK IS APPROXIMATELY 0.4 MILE SW OF THE STATION, 1 FOOT N OF

JY1332'THE NE CORNER OF A BARN, 15.8 FEET SW OF A TELEPHONE POLE,

JY1332'AND 30 FEET W OF THE APPROXIMATE CENTER OF THE ROAD.  IT IS A BRONZE

JY1332'AZIMUTH DISK SET IN TOP OF A CONCRETE POST.  IT IS STAMPED

JY1332'TABOR 1945 AND PROJECTS 6 INCHES.

JY1332'

JY1332'TO REACH THE STATION FROM THE JUNCTION OF U.S. HIGHWAY 33 AND STATE

JY1332'HIGHWAY 664 ABOUT 1-1/2 MILES W OF LOGAN, GO N ON STATE

JY1332'HIGHWAY 664 FOR 3.5 MILES TO THE MT. PLEASANT CHURCH ON THE RIGHT.

JY1332'CONTINUE STRAIGHT AHEAD FOR 0.3 MILE TO THE AZIMUTH MARK ON THE

JY1332'LEFT.  CONTINUE FROM THIS POINT FOR 0.4 MILE TO THE STATION ON THE

JY1332'RIGHT AS DESCRIBED.

JY1332'

JY1332'HEIGHT OF LIGHT ABOVE STATION MARK - 22 METERS.

JY1332

JY1332                          STATION RECOVERY (1945)

JY1332

JY1332'RECOVERY NOTE BY COAST AND GEODETIC SURVEY 1945

JY1332'RECOVERED IN GOOD CONDITION.

JY1332

JY1332                          STATION RECOVERY (1960)

JY1332

JY1332'RECOVERY NOTE BY US GEOLOGICAL SURVEY 1960

JY1332'REFERENCE MARK NO. 2 DISTURBED.  CONCRETE BASE WITH MONUMENT HAS

JY1332'BEEN UPROOTED AND IS LYING ON SIDE OF ROAD.

JY1332

JY1332                          STATION RECOVERY (1986)

JY1332

JY1332'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1986

JY1332'THE STATION IS LOCATED ABOUT 5 MILES NORTHWEST OF LOGAN, 3 MILES

JY1332'NORTHEAST OF ENTERPRISE ON TOP A 6.0 METER CUT BANK ALONG THE EAST

JY1332'SIDE OF STATE HIGHWAY 664. OWNERSHIP MR RAYMOND PITTMAN ROUTE 664 BOX

JY1332'8990, MARION OHIO PHONE 614-385-4961.

JY1332'TO REACH THE STATION FROM THE JUNCTION OF US HIGHWAY 33 AND STATE

JY1332'HIGHWAY 664, ABOUT 1.5 MILES WEST-SOUTHWEST OF LOGAN, GO NORTH ON

JY1332'HIGHWAY 664 FOR 0.85 MILES TO A STOP LIGHT JUST AFTER CROSSING A

JY1332'BRIDGE OVER THE HOCKING RIVER.  TURN RIGHT THEN LEFT STILL ON

JY1332'HIGHWAY 664 AND CONTINUE NORTH FOR 3.45 MILES TO THE PLEASANT

JY1332'HILL CHURCH ON RIGHT.  CONTINUE AHEAD OF 0.75 MILES TO THE STATION

JY1332'ON THE RIGHT ATOP CUT HILL.

JY1332'THE STATION IS A CGS DISK STAMPED --TABOR 1945-- SET IN THE TOP OF A

JY1332'ROUND CONCRETE POST RECESS 20 CM.  IT IS 3.5 METERS (11.5 FEET) NORTH

JY1332'NORTHWEST OF A TELEPHONE POLE, 3.5 METERS (11.5 FEET) NORTH-NORTHWEST

JY1332'OF FENCE CORNER, 2.1 METERS (7.0 FEET) EAST OF BANK EDGE AND 1.8

JY1332'METERS (6.0 FEET) SOUTHEAST OF FIBERGLASS WITNESS POST.

JY1332'DESCRIBED BY B L LAMBERT.  TYPED BY JAMES MALONEY 9/10/87.

JY1332

JY1332                          STATION RECOVERY (1987)

JY1332

JY1332'RECOVERED 1987

JY1332'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0226 ***********************************************************************

JY0226  DESIGNATION -  V 68

JY0226  PID         -  JY0226

JY0226  STATE/COUNTY-  OH/HOCKING

JY0226  USGS QUAD   -  UNION FURNACE (1995)

JY0226

JY0226                         *CURRENT SURVEY CONTROL

JY0226  ___________________________________________________________________

JY0226* NAD 83(1986)-  39 28 11.     (N)    277 38 25      (E)     SCALED

JY0226* NAVD 88     -       225.118  (meters)     738.57   (feet)  ADJUSTED

JY0226  ___________________________________________________________________

JY0226  GEOID HEIGHT-         -34.09  (meters)                     GEOID03

JY0226  DYNAMIC HT  -         224.983 (meters)     738.13  (feet)  COMP

JY0226  MODELED GRAV-     980,023.9   (mgal)                       NAVD 88

JY0226

JY0226  VERT ORDER  -  SECOND    CLASS 0

JY0226

JY0226.The horizontal coordinates were scaled from a topographic map and have

JY0226.an estimated accuracy of +/- 6 seconds.

JY0226

JY0226.The orthometric height was determined by differential leveling

JY0226.and adjusted by the National Geodetic Survey in June 1991..

JY0226

JY0226.The geoid height was determined by GEOID03.

JY0226

JY0226.The dynamic height is computed by dividing the NAVD 88

JY0226.geopotential number by the normal gravity value computed on the

JY0226.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0226.degrees latitude (g = 980.6199 gals.).

JY0226

JY0226.The modeled gravity was interpolated from observed gravity values.

JY0226

JY0226;                    North         East    Units  Estimated Accuracy

JY0226;SPC OH S     -   163,170.      612,070.      MT  (+/- 180 meters Scaled)

JY0226

JY0226                          SUPERSEDED SURVEY CONTROL

JY0226

JY0226  NGVD 29 (??/??/92)  225.249  (m)          739.00   (f) ADJ UNCH    2 0

JY0226

JY0226.Superseded values are not recommended for survey control.

JY0226.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0226.See file dsdata.txt to determine how the superseded data were derived.

JY0226

JY0226_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD830697(NAD 83)

JY0226_MARKER: DB = BENCH MARK DISK

JY0226_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0226_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0226_STAMPING: V 68 1934

JY0226_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0226+STABILITY: SURFACE MOTION

JY0226

JY0226  HISTORY     - Date     Condition        Report By

JY0226  HISTORY     - 1934     MONUMENTED       CGS

JY0226  HISTORY     - 1976     GOOD             NGS

JY0226

JY0226                          STATION DESCRIPTION

JY0226

JY0226'DESCRIBED BY COAST AND GEODETIC SURVEY 1934

JY0226'0.8 MI N FROM UNION FURNACE.

JY0226'0.8 MILE NORTH ALONG THE CHESAPEAKE AND OHIO RAILWAY FROM THE

JY0226'STATION AT UNION FURNACE, HOCKING COUNTY, 156 YARDS NORTH OF

JY0226'MILEPOST C 56, 33 FEET SOUTHWEST OF THE CENTER OF A ROAD CROSSING,

JY0226'27 FEET WEST OF THE WEST RAIL, 21 FEET SOUTH OF A CROSSING-WARNING

JY0226'SIGN, AND ABOUT 6 FEET ABOVE THE LEVEL OF THE TRACK.  A STANDARD

JY0226'DISK, STAMPED V 68 1934 AND SET IN THE TOP OF A CONCRETE POST.

JY0226

JY0226                          STATION RECOVERY (1976)

JY0226

JY0226'RECOVERY NOTE BY NATIONAL GEODETIC SURVEY 1976

JY0226'RECOVERED IN GOOD CONDITION.

1        National Geodetic Survey,   Retrieval Date = MAY  2, 2007

JY0031 ***********************************************************************

JY0031  DESIGNATION -  Z 67

JY0031  PID         -  JY0031

JY0031  STATE/COUNTY-  OH/HOCKING

JY0031  USGS QUAD   -  GORE (1992)

JY0031

JY0031                         *CURRENT SURVEY CONTROL

JY0031  ___________________________________________________________________

JY0031* NAD 83(1986)-  39 34 45.     (N)    277 40 50      (E)     SCALED

JY0031* NAVD 88     -       223.332  (meters)     732.72   (feet)  ADJUSTED

JY0031  ___________________________________________________________________

JY0031  GEOID HEIGHT-         -34.09  (meters)                     GEOID03

JY0031  DYNAMIC HT  -         223.201 (meters)     732.29  (feet)  COMP

JY0031  MODELED GRAV-     980,036.1   (mgal)                       NAVD 88

JY0031

JY0031  VERT ORDER  -  SECOND    CLASS 0

JY0031

JY0031.The horizontal coordinates were scaled from a topographic map and have

JY0031.an estimated accuracy of +/- 6 seconds.

JY0031

JY0031.The orthometric height was determined by differential leveling

JY0031.and adjusted by the National Geodetic Survey in June 1991..

JY0031

JY0031.The geoid height was determined by GEOID03.

JY0031

JY0031.The dynamic height is computed by dividing the NAVD 88

JY0031.geopotential number by the normal gravity value computed on the

JY0031.Geodetic Reference System of 1980 (GRS 80) ellipsoid at 45

JY0031.degrees latitude (g = 980.6199 gals.).

JY0031

JY0031.The modeled gravity was interpolated from observed gravity values.

JY0031

JY0031;                    North         East    Units  Estimated Accuracy

JY0031;SPC OH S     -   175,330.      615,510.      MT  (+/- 180 meters Scaled)

JY0031

JY0031                          SUPERSEDED SURVEY CONTROL

JY0031

JY0031  NGVD 29 (??/??/92)  223.581  (m)          733.53   (f) ADJ UNCH    2 0

JY0031

JY0031.Superseded values are not recommended for survey control.

JY0031.NGS no longer adjusts projects to the NAD 27 or NGVD 29 datums.

JY0031.See file dsdata.txt to determine how the superseded data were derived.

JY0031

JY0031_U.S. NATIONAL GRID SPATIAL ADDRESS: 17SLD866818(NAD 83)

JY0031_MARKER: DB = BENCH MARK DISK

JY0031_SETTING: 7 = SET IN TOP OF CONCRETE MONUMENT

JY0031_SP_SET: SET IN TOP OF CONCRETE MONUMENT

JY0031_STAMPING: Z 67 1934

JY0031_STABILITY: C = MAY HOLD, BUT OF TYPE COMMONLY SUBJECT TO

JY0031+STABILITY: SURFACE MOTION

JY0031

JY0031  HISTORY     - Date     Condition        Report By

JY0031  HISTORY     - 1934     MONUMENTED       CGS

JY0031  HISTORY     - 1960     MARK NOT FOUND   USGS

JY0031

JY0031                          STATION DESCRIPTION

JY0031

JY0031'DESCRIBED BY COAST AND GEODETIC SURVEY 1934

JY0031'4.8 MI SW FROM NEW STRAITSVILLE.

JY0031'IN HOCKING COUNTY, 4.8 MILES SOUTHWEST ALONG STATE HIGHWAY 75

JY0031'FROM THE POST OFFICE AT NEW STRAITSVILLE, PERRY COUNTY, 0.4

JY0031'MILE SOUTH OF THE JUNCTION OF A GRAVEL ROAD LEADING SOUTH OVER

JY0031'AN IRON BRIDGE, 125 YARDS NORTH OF THE NORTHWEST CORNER OF THE

JY0031'TOM MORRIS FARMHOUSE, 10 YARDS WEST OF THE CENTERLINE OF THE

JY0031'HIGHWAY, 2 FEET SOUTH OF THE SOUTHEAST CORNER OF A FENCE, AND

JY0031'ABOUT 3 FEET LOWER THAN THE HIGHWAY.  A STANDARD DISK, STAMPED

JY0031'Z 67 1934 AND SET IN THE TOP OF A CONCRETE POST.

JY0031

JY0031                          STATION RECOVERY (1960)

JY0031

JY0031'RECOVERY NOTE BY US GEOLOGICAL SURVEY 1960

JY0031'MARK NOT FOUND.

*** retrieval complete.

Elapsed Time = 00:00:05